Test bed configuration:

source gateway (cisco x.y.z.105) ->Asterisk (ooh323)->destination gateway (cisco x.y.z.103)

ooh323.conf:

[general]

bindaddr=0.0.0.0

h323id=ObjSysAsterisk

e164=100

callerid=asterisk

gatekeeper=DISABLE

context=GeneralProxy

disallow=all

allow=gsm

allow=ulaw

dtmfmode=rfc2833

extensions.conf:

[GeneralProxy]

exten => _XXXX.,1,NoOp(GeneralProxy)

exten => _XXXX.,n,Dial(OOH323/${EXTEN}@x.y.z.103,15,o)

exten => _XXXX.,n,Hangup

exten => h,1,NoOp(hangup cause is ${HANGUPCAUSE})

CLI:

---   onNewCallCreated ooh323c_1

+++   onNewCallCreated ooh323c_1

---   ooh323_onReceivedSetup ooh323c_1

---   find_user

+++   find_user

Adding capabilities to call(incoming, ooh323c_1)

Adding gsm capability to call(incoming, ooh323c_1)

Adding g711 ulaw capability to call(incoming, ooh323c_1)

---   configure_local_rtp

+++   configure_local_rtp

+++   ooh323_onReceivedSetup - Determined context GeneralProxy, extension 6781234567

---   setup_rtp_connection

---   find_call

+++   find_call

+++   setup_rtp_connection

--- onAlerting ooh323c_1

---   find_call

+++   find_call

+++ onAlerting ooh323c_1

-- Executing [6781234567@GeneralProxy:1] NoOp("OOH323/x.y.z.105-47c7", "GeneralProxy") in new stack

-- Executing [6781234567@GeneralProxy:2] Dial("OOH323/x.y.z.105-47c7", "OOH323/6781234567@x.y.z.103|15|o") in new stack

---   ooh323_request - data 6781234567@x.y.z.103 format 0x2 (gsm)

---   find_peer

+++   find_peer

+++   ooh323_request

---   ooh323_set_peer - OOH323/x.y.z.103-0410

---   ooh323_call- 6781234567@x.y.z.103

---   onNewCallCreated ooh323c_o_1

---   find_call

+++   find_call

setting callid number 4041234567

Outgoing call x.y.z.103(ooh323c_o_1) - Codec prefs - (gsm|ulaw)

Adding capabilities to call(outgoing, ooh323c_o_1)

Adding gsm capability to call(outgoing, ooh323c_o_1)

Adding g711 ulaw capability to call(outgoing, ooh323c_o_1)

---   configure_local_rtp

+++   configure_local_rtp

+++   onNewCallCreated ooh323c_o_1

+++   ooh323_call

-- Called 6781234567@x.y.z.103

--- onAlerting ooh323c_o_1

---   find_call

+++   find_call

+++ onAlerting ooh323c_o_1

-- OOH323/x.y.z.103-0410 is ringing

---   ooh323_set_peer - OOH323/x.y.z.105-47c7

----- ooh323_indicate 3 on call ooh323c_1

++++  ooh323_indicate 3 on ooh323c_1

[Sep 19 14:04:31] WARNING[7614]: channel.c:2279 ast_indicate_data: Unable to handle indication 3 for 'OOH323/x.y.z.105-47c7'

---   onCallEstablished ooh323c_o_1

---   find_call

+++   find_call

+++   onCallEstablished ooh323c_o_1

-- OOH323/x.y.z.103-0410 answered OOH323/x.y.z.105-47c7

----- ooh323_indicate -1 on call ooh323c_1

[Sep 19 14:04:34] WARNING[7614]: src/chan_h323.c:1016 ooh323_indicate: Don't know how to indicate condition -1 on ooh323c_1

++++  ooh323_indicate -1 on ooh323c_1

--- ooh323_answer

+++ ooh323_answer

-- Native bridging OOH323/x.y.z.105-47c7 and OOH323/x.y.z.103-0410

---   ooh323_set_peer - OOH323/x.y.z.105-47c7

---   ooh323_set_peer - OOH323/x.y.z.103-0410

---   onCallEstablished ooh323c_1

---   find_call

+++   find_call

+++   onCallEstablished ooh323c_1

---   setup_rtp_connection

---   find_call

+++   find_call

+++   setup_rtp_connection

---   ooh323_set_peer - OOH323/x.y.z.105-47c7

---   close_rtp_connection

---   find_call

+++   find_call

+++   close_rtp_connection

---   onCallCleared ooh323c_1

---   find_call

+++   find_call

---   ooh323_set_peer - OOH323/x.y.z.105-47c7

---   ooh323_set_peer - OOH323/x.y.z.103-0410

---   ooh323_hangup

hanging x.y.z.103

+++   ooh323_hangup

-- Executing [h@GeneralProxy:1] NoOp("OOH323/x.y.z.105-47c7", "hangup cause is 16") in new stack

---   ooh323_hangup

hanging x.y.z.105

+++   ooh323_hangup

---   close_rtp_connection

---   find_call

+++   find_call

+++   close_rtp_connection

---   onCallCleared ooh323c_o_1

---   find_call

+++   find_call

+++   onCallCleared

---   ooh323_destroy

Destroying x.y.z.103

+++   ooh323_destroy

---   ooh323_destroy

Destroying x.y.z.105

+++   ooh323_destroy

H323_log:

---------Date 09/19/06---------

14:04:21:526  Signalling IP address is set to 0.0.0.0

14:04:21:526  Listen port number is set to 1720

14:04:21:526  Enabled RFC2833 DTMF capability for end-point

14:04:21:527  H323 listener creation - successful

14:04:21:527  Creating CMD listener at 0.0.0.0:7575

14:04:21:528  CMD listener creation - successful

14:04:21:549  H.323 Endpoint Configuration is as follows:

14:04:21:549    Trace File: /var/log/asterisk/h323_log

14:04:21:550    FastStart - enabled

14:04:21:550    H245 Tunneling - enabled

14:04:21:550    MediaWaitForConnect - disabled

14:04:21:550    AutoAnswer - disabled

14:04:21:550    Terminal Type - 50

14:04:21:551    T35 CountryCode - 1

14:04:21:551    T35 Extension - 0

14:04:21:551    Manufacturer Code - 71

14:04:21:551    ProductID - objsys

14:04:21:551    VersionID - v0.8.2

14:04:21:551    Local signalling IP address - 0.0.0.0

14:04:21:552    H225 ListenPort - 1720

14:04:21:552    CallerID - asterisk

14:04:21:552    Call Establishment Timeout - 60 seconds

14:04:21:552    MasterSlaveDetermination Timeout - 30 seconds

14:04:21:552    TerminalCapabilityExchange Timeout - 30 seconds

14:04:21:552    LogicalChannel  Timeout - 30 seconds

14:04:21:553    Session Timeout - 15 seconds

14:04:31:128  Created a new call (incoming, ooh323c_1)

14:04:31:180  Received SETUP message (incoming, ooh323c_1)

14:04:31:180  Call has tunneling active (incoming,ooh323c_1)

14:04:31:181  FastStart enabled for call(incoming, ooh323c_1)

14:04:31:182  Enabled RFC2833 DTMF capability for (incoming, ooh323c_1)

14:04:31:183  Receive Channel data type supported

14:04:31:184  Created new logical channel entry (incoming, ooh323c_1)

14:04:31:184  Receive channel of type audio started (incoming, ooh323c_1)

14:04:31:185  Transmit Channel data type supported

14:04:31:185  Created new logical channel entry (incoming, ooh323c_1)

14:04:31:185  Transmit channel of type audio started (incoming, ooh323c_1)

14:04:31:187  Sent Message - CallProceeding (incoming, ooh323c_1)

14:04:31:187  Sent Message - Alerting (incoming, ooh323c_1)

14:04:31:193  Cmd connection accepted

14:04:31:194  Processing MakeCall command ooh323c_o_1

14:04:31:194  Created a new call (outgoing, ooh323c_o_1)

14:04:31:195  Enabled RFC2833 DTMF capability for (outgoing, ooh323c_o_1)

14:04:31:195  Parsing destination x.y.z.103

14:04:31:196  Trying to connect to remote endpoint(x.y.z.103:1720) to setup H2250 channel (outgoing, ooh323c_o_1)

14:04:31:200  H2250 transmiter channel creation - succesful (outgoing, ooh323c_o_1)

14:04:31:200  Created new logical channel entry (outgoing, ooh323c_o_1)

14:04:31:200  Receive channel of type audio started (outgoing, ooh323c_o_1)

14:04:31:201  Created new logical channel entry (outgoing, ooh323c_o_1)

14:04:31:201  Created new logical channel entry (outgoing, ooh323c_o_1)

14:04:31:201  Receive channel of type audio started (outgoing, ooh323c_o_1)

14:04:31:202  Created new logical channel entry (outgoing, ooh323c_o_1)

14:04:31:203  Sent Message - Setup (outgoing, ooh323c_o_1)

14:04:31:331  H.225 Call Proceeding message received (outgoing, ooh323c_o_1)

14:04:31:331  ERROR: Logical Channel 1004 not found, fast start. (outgoing, ooh323c_o_1)

14:04:31:365  H.225 Alerting message received (outgoing, ooh323c_o_1)

14:04:34:607  H.225 Connect message received (outgoing, ooh323c_o_1)

14:04:34:607  Remote endpoint has rejected fastStart. (outgoing, ooh323c_o_1)

14:04:34:607  Clearing all logical channels (outgoing, ooh323c_o_1)

14:04:34:608  Stopped Receive channel 1001 (outgoing, ooh323c_o_1)

14:04:34:608  Stopped Receive channel 1003 (outgoing, ooh323c_o_1)

14:04:34:610  Sent Message - Facility(OOTerminalCapabilitySetAck) (outgoing, ooh323c_o_1)

14:04:34:610  Tunneled Message - TerminalCapabilitySet (outgoing, ooh323c_o_1)

14:04:34:610  Sent Message - Facility(OOMasterSlaveAck) (outgoing, ooh323c_o_1)

14:04:34:610  Tunneled Message - MasterSlaveDetermination (outgoing, ooh323c_o_1)

14:04:34:612  Processing Answer Call command for ooh323c_1

14:04:34:613  Sent Message - Connect (incoming, ooh323c_1)

14:04:34:668  H.225 Facility message Received (outgoing, ooh323c_o_1)

14:04:34:669  Sent Message - Facility(OOTerminalCapabilitySetReject) (outgoing, ooh323c_o_1)

14:04:34:670  Tunneled Message - TerminalCapabilitySetAck (outgoing, ooh323c_o_1)

14:04:34:691  H.225 Facility message Received (outgoing, ooh323c_o_1)

14:04:34:691  Master Slave Determination received (outgoing, ooh323c_o_1)

14:04:34:692  MasterSlaveDetermination done - Slave(outgoing, ooh323c_o_1)

14:04:34:692  Sent Message - Facility(OOMasterSlaveReject) (outgoing, ooh323c_o_1)

14:04:34:692  Tunneled Message - MasterSlaveDeterminationAck (outgoing, ooh323c_o_1)

14:04:34:714  H.225 Facility message Received (outgoing, ooh323c_o_1)

14:04:34:715  Opening logical channels (outgoing, ooh323c_o_1)

14:04:34:715  Looking for matching capabilities. (outgoing, ooh323c_o_1)

14:04:34:715  Created new logical channel entry (outgoing, ooh323c_o_1)

14:04:34:715  Sent Message - Facility(OOOpenLogicalChannelAck) (outgoing, ooh323c_o_1)

14:04:34:716  Tunneled Message - OpenLogicalChannel(1005). (outgoing, ooh323c_o_1)

14:04:34:762  H.225 Facility message Received (outgoing, ooh323c_o_1)

14:04:34:848  H.225 Facility message Received (outgoing, ooh323c_o_1)

14:04:34:849  TransmitLogical Channel of type OO_G711ULAW64K started (outgoing, ooh323c_o_1)

14:04:42:702  H.225 Release Complete message received (incoming, ooh323c_1)

14:04:42:703  Cleaning Call (incoming, ooh323c_1)- reason:OO_REASON_REMOTE_CLEARED

14:04:42:703  Clearing all logical channels (incoming, ooh323c_1)

14:04:42:703  Stopped Receive channel 1 (incoming, ooh323c_1)

14:04:42:703  Stopped Transmit channel 1001 (incoming, ooh323c_1)

14:04:42:704  Closing H.245 connection (incoming, ooh323c_1)

14:04:42:704  Removed call (incoming, ooh323c_1) from list

14:04:42:706  Processing Hang call command ooh323c_o_1

14:04:42:706  Hanging up call (outgoing, ooh323c_o_1)

14:04:42:706  Clearing all logical channels. (outgoing, ooh323c_o_1)

14:04:42:707  Clearing all logical channels (outgoing, ooh323c_o_1)

14:04:42:707  Stopped Transmit channel 1005 (outgoing, ooh323c_o_1)

14:04:42:708  Sent Message - ReleaseComplete (outgoing, ooh323c_o_1)

14:04:42:744  H.225 Release Complete message received (outgoing, ooh323c_o_1)

14:04:42:744  Closing H.245 connection (outgoing, ooh323c_o_1)

14:04:42:745  Cleaning Call (outgoing, ooh323c_o_1)- reason:OO_REASON_UNKNOWN

14:04:42:745  Removed call (outgoing, ooh323c_o_1) from list