## Cisco log

1d06h: H225 NONSTD OUTGOING PDU ::=

value H323_UU_NonStdInfo ::=

{

version 18

protoParam qsigNonStdInfo :

{

iei 28

rawMesg '1C399E0100036774640000002E49414D2C0D0A47...'H

}

progIndParam progIndIEinfo :

{

progIndIE '00000003'H

}

}

1d06h: H225 NONSTD OUTGOING ENCODE BUFFER::= E0011200011C3B1C399E0100036774640000002E49414D2C0D0A4743492C36653934363462343135656531316363383062306233323935643932646636630D0A0D0A068006000400000003

1d06h:

1d06h: H225.0 OUTGOING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body setup :

{

protocolIdentifier { 0 0 8 2250 0 4 }

sourceInfo

{

vendor

{

vendor

{

t35CountryCode 181

t35Extension 0

manufacturerCode 18

}

}

gateway

{

protocol

{

voice :

{

supportedPrefixes

{

}

}

}

}

mc FALSE

undefinedNode FALSE

}

activeMC FALSE

conferenceID '6E9464B415EE11CC80B0B3295D92DF6C'H

conferenceGoal create : NULL

callType pointToPoint : NULL

sourceCallSignalAddress ipAddress :

{

ip 'AC10040D'H

port 19888

}

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

fastStart

{

'0000000C6013800B05000100AC10040D444B00'H,

'400000060401004C6013801215000100AC10040D...'H

}

mediaWaitForConnect FALSE

canOverlapSend FALSE

multipleCalls TRUE

maintainConnection TRUE

symmetricOperationRequired NULL

}

h245Tunneling TRUE

nonStandardControl

{

{

nonStandardIdentifier h221NonStandard :

{

t35CountryCode 181

t35Extension 0

manufacturerCode 18

}

data 'E0011200011C3B1C399E0100036774640000002E...'H

}

}

tunnelledSignallingMessage

{

tunnelledProtocolID

{

id tunnelledProtocolAlternateID :

{

protocolType "gtd"

}

}

messageContent

{

'49414D2C0D0A4743492C36653934363462343135...'H

}

}

}

}

1d06h: H225.0 OUTGOING ENCODE BUFFER::= 2080060008914A00042800B500001240013C050100006E9464B415EE11CC80B0B3295D92DF6C00CD1D82000700AC10040D4DB011006E9500DC15EE11CC80B2B3295D92DF6C3402130000000C6013800B05000100AC10040D444B001E400000060401004C6013801215000100AC10040D444A00AC10040D444B000100010001800180010010A80180520140B50000124BE0011200011C3B1C399E0100036774640000002E49414D2C0D0A4743492C36653934363462343135656531316363383062306233323935643932646636630D0A0D0A068006000400000003350204677464012E49414D2C0D0A4743492C36653934363462343135656531316363383062306233323935643932646636630D0A0D0A

1d06h:

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x002E

Message Type           : 0x05: SETUP

Bearer Capability: Length Of IE=3

Data 9090A3

Calling Party Number: Length Of IE=11

Data 8034303435323636303631

Called Party Number: Length Of IE=11

Data 8134303435323636303632

User-User: Length Of IE=274

Data 052080060008914A00042800B500001240013C050100006E9464B415EE11CC80B0B3295D92DF6C00CD1D82000700AC10040D4DB011006E9500DC15EE11CC80B2B3295D92DF6C3402130000000C6013800B05000100AC10040D444B001E400000060401004C6013801215000100AC10040D444A00AC10040D444B000100010001800180010010A80180520140B50000124BE0011200011C3B1C399E0100036774640000002E49414D2C0D0A4743492C36653934363462343135656531316363383062306233323935643932646636630D0A0D0A068006000400000003350204677464012E49414D2C0D0A4743492C36653934363462343135656531316363383062306233323935643932646636630D0A0D0A

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x802E

Message Type           : 0x02: CALL_PROC

Display: Length Of IE=11

Data 3430343532363630363200

User-User: Length Of IE=128

Data 052180060008914A000422C00900003D36546865204E75466F6E65204E6574776F726B7320482E333233204368616E6E656C2044726976657220666F7220417374657269736B000019312E302E3020284F70656E483332332076312E31382E3029000000D0C011006E9500DC15EE11CC80B2B3295D92DF6C0100010002800180

1d06h: H225.0 INCOMING ENCODE BUFFER::= 2180060008914A000422C00900003D36546865204E75466F6E65204E6574776F726B7320482E333233204368616E6E656C2044726976657220666F7220417374657269736B000019312E302E3020284F70656E483332332076312E31382E3029000000D0C011006E9500DC15EE11CC80B2B3295D92DF6C0100010002800180

1d06h:

1d06h: H225.0 INCOMING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body callProceeding :

{

protocolIdentifier { 0 0 8 2250 0 4 }

destinationInfo

{

vendor

{

vendor

{

t35CountryCode 9

t35Extension 0

manufacturerCode 61

}

productId '546865204E75466F6E65204E6574776F726B7320...'H

versionId '312E302E3020284F70656E483332332076312E31...'H

}

terminal

{

}

mc FALSE

undefinedNode FALSE

}

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

multipleCalls FALSE

maintainConnection FALSE

}

h245Tunneling TRUE

}

}

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x802E

Message Type           : 0x03: PROGRESS

Progress Ind: Length Of IE=2

Data 8088

Display: Length Of IE=11

Data 3430343532363630363200

User-User: Length Of IE=181

Data 05280080AC84060008914A000422C00900003D36546865204E75466F6E65204E6574776F726B7320482E333233204368616E6E656C2044726976657220666F7220417374657269736B000019312E302E3020284F70656E483332332076312E31382E30290000006E9500DC15EE11CC80B2B3295D92DF6C021A0000000C609580121D000100AC1004383E1800AC1004383E190017400064060401004C6013800B0D000100AC1004383E190003800100010002800180

1d06h: H225.0 INCOMING ENCODE BUFFER::= 280080AC84060008914A000422C00900003D36546865204E75466F6E65204E6574776F726B7320482E333233204368616E6E656C2044726976657220666F7220417374657269736B000019312E302E3020284F70656E483332332076312E31382E30290000006E9500DC15EE11CC80B2B3295D92DF6C021A0000000C609580121D000100AC1004383E1800AC1004383E190017400064060401004C6013800B0D000100AC1004383E190003800100010002800180

1d06h:

1d06h: H225.0 INCOMING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body progress :

{

protocolIdentifier { 0 0 8 2250 0 4 }

destinationInfo

{

vendor

{

vendor

{

t35CountryCode 9

t35Extension 0

manufacturerCode 61

}

productId '546865204E75466F6E65204E6574776F726B7320...'H

versionId '312E302E3020284F70656E483332332076312E31...'H

}

terminal

{

}

mc FALSE

undefinedNode FALSE

}

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

fastStart

{

'0000000C609580121D000100AC1004383E1800AC...'H,

'400064060401004C6013800B0D000100AC100438...'H

}

multipleCalls FALSE

maintainConnection FALSE

}

h245Tunneling TRUE

}

}

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x802E

Message Type           : 0x01: ALERTING

Display: Length Of IE=11

Data 3430343532363630363200

User-User: Length Of IE=128

Data 052380060008914A000422C00900003D36546865204E75466F6E65204E6574776F726B7320482E333233204368616E6E656C2044726976657220666F7220417374657269736B000019312E302E3020284F70656E483332332076312E31382E3029000000D0C011006E9500DC15EE11CC80B2B3295D92DF6C0100010002800180

1d06h: H225.0 INCOMING ENCODE BUFFER::= 2380060008914A000422C00900003D36546865204E75466F6E65204E6574776F726B7320482E333233204368616E6E656C2044726976657220666F7220417374657269736B000019312E302E3020284F70656E483332332076312E31382E3029000000D0C011006E9500DC15EE11CC80B2B3295D92DF6C0100010002800180

1d06h:

1d06h: H225.0 INCOMING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body alerting :

{

protocolIdentifier { 0 0 8 2250 0 4 }

destinationInfo

{

vendor

{

vendor

{

t35CountryCode 9

t35Extension 0

manufacturerCode 61

}

productId '546865204E75466F6E65204E6574776F726B7320...'H

versionId '312E302E3020284F70656E483332332076312E31...'H

}

terminal

{

}

mc FALSE

undefinedNode FALSE

}

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

multipleCalls FALSE

maintainConnection FALSE

}

h245Tunneling TRUE

}

}

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x802E

Message Type           : 0x01: ALERTING

Display: Length Of IE=11

Data 3430343532363630363200

User-User: Length Of IE=128

Data 052380060008914A000422C00900003D36546865204E75466F6E65204E6574776F726B7320482E333233204368616E6E656C2044726976657220666F7220417374657269736B000019312E302E3020284F70656E483332332076312E31382E3029000000D0C011006E9500DC15EE11CC80B2B3295D92DF6C0100010002800180

1d06h: H225.0 INCOMING ENCODE BUFFER::= 2380060008914A000422C00900003D36546865204E75466F6E65204E6574776F726B7320482E333233204368616E6E656C2044726976657220666F7220417374657269736B000019312E302E3020284F70656E483332332076312E31382E3029000000D0C011006E9500DC15EE11CC80B2B3295D92DF6C0100010002800180

1d06h:

1d06h: H225.0 INCOMING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body alerting :

{

protocolIdentifier { 0 0 8 2250 0 4 }

destinationInfo

{

vendor

{

vendor

{

t35CountryCode 9

t35Extension 0

manufacturerCode 61

}

productId '546865204E75466F6E65204E6574776F726B7320...'H

versionId '312E302E3020284F70656E483332332076312E31...'H

}

terminal

{

}

mc FALSE

undefinedNode FALSE

}

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

multipleCalls FALSE

maintainConnection FALSE

}

h245Tunneling TRUE

}

}

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x802E

Message Type           : 0x07: CONNECT

Bearer Capability: Length Of IE=3

Data 8090A5

Display: Length Of IE=11

Data 3430343532363630363200

User-User: Length Of IE=226

Data 052280060008914A000422C00900003D36546865204E75466F6E65204E6574776F726B7320482E333233204368616E6E656C2044726976657220666F7220417374657269736B000019312E302E3020284F70656E483332332076312E31382E30290000006E9464B415EE11CC80B0B3295D92DF6C170C2011006E9500DC15EE11CC80B2B3295D92DF6C01000100010004C001804E02440270010600088175000B80138000FA000100000100000100000CC0010001800280000020C0138000018301508000028A061404302D313600800102000000000001000002070100328056F3F4

1d06h: H225.0 INCOMING ENCODE BUFFER::= 2280060008914A000422C00900003D36546865204E75466F6E65204E6574776F726B7320482E333233204368616E6E656C2044726976657220666F7220417374657269736B000019312E302E3020284F70656E483332332076312E31382E30290000006E9464B415EE11CC80B0B3295D92DF6C170C2011006E9500DC15EE11CC80B2B3295D92DF6C01000100010004C001804E02440270010600088175000B80138000FA000100000100000100000CC0010001800280000020C0138000018301508000028A061404302D313600800102000000000001000002070100328056F3F4

1d06h:

1d06h: H225.0 INCOMING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body connect :

{

protocolIdentifier { 0 0 8 2250 0 4 }

destinationInfo

{

vendor

{

vendor

{

t35CountryCode 9

t35Extension 0

manufacturerCode 61

}

productId '546865204E75466F6E65204E6574776F726B7320...'H

versionId '312E302E3020284F70656E483332332076312E31...'H

}

terminal

{

}

mc FALSE

undefinedNode FALSE

}

conferenceID '6E9464B415EE11CC80B0B3295D92DF6C'H

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

multipleCalls FALSE

maintainConnection FALSE

fastConnectRefused NULL

}

h245Tunneling TRUE

h245Control

{

'0270010600088175000B80138000FA0001000001...'H,

'0100328056F3F4'H

}

}

}

1d06h: H225.0 OUTGOING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body facility :

{

protocolIdentifier { 0 0 8 2250 0 4 }

conferenceID '6E9464B415EE11CC80B0B3295D92DF6C'H

reason transportedInformation : NULL

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

multipleCalls FALSE

maintainConnection FALSE

}

h245Tunneling TRUE

h245Control

{

'0270010600088175000780138000140001000001...'H

}

}

}

1d06h: H225.0 OUTGOING ENCODE BUFFER::= 2690060008914A00046E9464B415EE11CC80B0B3295D92DF6C8601001F018011006E9500DC15EE11CC80B2B3295D92DF6C0100010010C0018070016E027001060008817500078013800014000100000100000100000CC0010001000580001D8A061404302D31368000134810B50000120C52747044746D6652656C6179000080001983015080001883014080001583011080000020C0130080010200000003001D001800150013000019

1d06h:

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x002E

Message Type           : 0x62: FACILITY

Facility: Length Of IE=0

Data

Display: Length Of IE=19

Data 636973636F2053797374656D732C20496E632E

User-User: Length Of IE=171

Data 052690060008914A00046E9464B415EE11CC80B0B3295D92DF6C8601001F018011006E9500DC15EE11CC80B2B3295D92DF6C0100010010C0018070016E027001060008817500078013800014000100000100000100000CC0010001000580001D8A061404302D31368000134810B50000120C52747044746D6652656C6179000080001983015080001883014080001583011080000020C0130080010200000003001D001800150013000019

1d06h: H225.0 OUTGOING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body facility :

{

protocolIdentifier { 0 0 8 2250 0 4 }

conferenceID '6E9464B415EE11CC80B0B3295D92DF6C'H

reason transportedInformation : NULL

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

multipleCalls FALSE

maintainConnection FALSE

}

h245Tunneling TRUE

h245Control

{

'01003C401754'H

}

}

}

1d06h: H225.0 OUTGOING ENCODE BUFFER::= 2690060008914A00046E9464B415EE11CC80B0B3295D92DF6C8601001F018011006E9500DC15EE11CC80B2B3295D92DF6C0100010010C0018008010601003C401754

1d06h:

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x002E

Message Type           : 0x62: FACILITY

Facility: Length Of IE=0

Data

Display: Length Of IE=19

Data 636973636F2053797374656D732C20496E632E

User-User: Length Of IE=67

Data 052690060008914A00046E9464B415EE11CC80B0B3295D92DF6C8601001F018011006E9500DC15EE11CC80B2B3295D92DF6C0100010010C0018008010601003C401754

1d06h: H225.0 OUTGOING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body facility :

{

protocolIdentifier { 0 0 8 2250 0 4 }

conferenceID '6E9464B415EE11CC80B0B3295D92DF6C'H

reason transportedInformation : NULL

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

multipleCalls FALSE

maintainConnection FALSE

}

h245Tunneling TRUE

h245Control

{

'218001'H

}

}

}

1d06h: H225.0 OUTGOING ENCODE BUFFER::= 2690060008914A00046E9464B415EE11CC80B0B3295D92DF6C8601001F018011006E9500DC15EE11CC80B2B3295D92DF6C0100010010C00180050103218001

1d06h:

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x002E

Message Type           : 0x62: FACILITY

Facility: Length Of IE=0

Data

Display: Length Of IE=19

Data 636973636F2053797374656D732C20496E632E

User-User: Length Of IE=64

Data 052690060008914A00046E9464B415EE11CC80B0B3295D92DF6C8601001F018011006E9500DC15EE11CC80B2B3295D92DF6C0100010010C00180050103218001

1d06h: H225.0 OUTGOING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body facility :

{

protocolIdentifier { 0 0 8 2250 0 4 }

conferenceID '6E9464B415EE11CC80B0B3295D92DF6C'H

reason transportedInformation : NULL

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

multipleCalls FALSE

maintainConnection FALSE

}

h245Tunneling TRUE

h245Control

{

'20A0'H

}

}

}

1d06h: H225.0 OUTGOING ENCODE BUFFER::= 2690060008914A00046E9464B415EE11CC80B0B3295D92DF6C8601001F018011006E9500DC15EE11CC80B2B3295D92DF6C0100010010C0018004010220A0

1d06h:

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x002E

Message Type           : 0x62: FACILITY

Facility: Length Of IE=0

Data

Display: Length Of IE=19

Data 636973636F2053797374656D732C20496E632E

User-User: Length Of IE=63

Data 052690060008914A00046E9464B415EE11CC80B0B3295D92DF6C8601001F018011006E9500DC15EE11CC80B2B3295D92DF6C0100010010C0018004010220A0

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x802E

Message Type           : 0x62: FACILITY

Facility: Length Of IE=0

Data

User-User: Length Of IE=15

Data 052810010004C00180050103218001

1d06h: H225.0 INCOMING ENCODE BUFFER::= 2810010004C00180050103218001

1d06h:

1d06h: H225.0 INCOMING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body empty : NULL

h245Tunneling TRUE

h245Control

{

'218001'H

}

}

}

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x802E

Message Type           : 0x62: FACILITY

Facility: Length Of IE=0

Data

User-User: Length Of IE=14

Data 052810010004C001800401022080

1d06h: H225.0 INCOMING ENCODE BUFFER::= 2810010004C001800401022080

1d06h:

1d06h: H225.0 INCOMING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body empty : NULL

h245Tunneling TRUE

h245Control

{

'2080'H

}

}

}

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x802E

Message Type           : 0x5A: RELEASE_COMP

Cause: Length Of IE=2

Data 8090

User-User: Length Of IE=38

Data 052580060008914A00040111006E9500DC15EE11CC80B2B3295D92DF6C04C001800401024A40

1d06h: H225.0 INCOMING ENCODE BUFFER::= 2580060008914A00040111006E9500DC15EE11CC80B2B3295D92DF6C04C001800401024A40

1d06h:

1d06h: H225.0 INCOMING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body releaseComplete :

{

protocolIdentifier { 0 0 8 2250 0 4 }

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

}

h245Tunneling TRUE

h245Control

{

'4A40'H

}

}

}

1d06h: H225.0 OUTGOING PDU ::=

value H323_UserInformation ::=

{

h323-uu-pdu

{

h323-message-body releaseComplete :

{

protocolIdentifier { 0 0 8 2250 0 4 }

callIdentifier

{

guid '6E9500DC15EE11CC80B2B3295D92DF6C'H

}

}

h245Tunneling TRUE

}

}

1d06h: H225.0 OUTGOING ENCODE BUFFER::= 2580060008914A0004110011006E9500DC15EE11CC80B2B3295D92DF6C10800180

1d06h:

Q931 Message IE Decodes

Protocol Discriminator : 0x08

CRV Length             : 2

CRV Value              : 0x002E

Message Type           : 0x5A: RELEASE_COMP

Cause: Length Of IE=2

Data 8090

User-User: Length Of IE=34

Data 052580060008914A0004110011006E9500DC15EE11CC80B2B3295D92DF6C10800180

## Asterisk log

*CLI>   2:40.265          H323 Listener:90cccb8   transports.cxx(1559)  H323TCP Started connection:  host=172.16.4.13:19888, if=172.16.4.56:1720, handle=22

2:40.266        H323 Listener:90cccb8   transports.cxx(1356)  TCP     Waiting on socket accept on ip$*:1720

2:40.267          H225 Answer:90f4718   transports.cxx(595)   H225    Started incoming call thread

2:40.268          H225 Answer:90f4718   transports.cxx(1161)  H225    Awaiting first PDU

2:40.344          H225 Answer:90f4718      h323pdu.cxx(553)   H225    Receiving PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = originator

messageType = Setup

IE: Bearer-Capability = {

90 90 a3                                           ...

}

IE: Calling-Party-Number = {

80 34 30 34 35 32 36 36  30 36 31                  .4045266061

}

IE: Called-Party-Number = {

81 34 30 34 35 32 36 36  30 36 32                  .4045266062

}

IE: User-User = {

20 80 06 00 08 91 4a 00  04 28 00 b5 00 00 12 40    .....J..(.....@

01 3c 05 01 00 00 6e 94  64 b4 15 ee 11 cc 80 b0   .<....n.d.......

b3 29 5d 92 df 6c 00 cd  1d 82 00 07 00 ac 10 04   .)]..l..........

0d 4d b0 11 00 6e 95 00  dc 15 ee 11 cc 80 b2 b3   .M...n..........

29 5d 92 df 6c 34 02 13  00 00 00 0c 60 13 80 0b   )]..l4......`...

05 00 01 00 ac 10 04 0d  44 4b 00 1e 40 00 00 06   ........DK..@...

04 01 00 4c 60 13 80 12  15 00 01 00 ac 10 04 0d   ...L`...........

44 4a 00 ac 10 04 0d 44  4b 00 01 00 01 00 01 80   DJ.....DK.......

01 80 01 00 10 a8 01 80  52 01 40 b5 00 00 12 4b   ........R.@....K

e0 01 12 00 01 1c 3b 1c  39 9e 01 00 03 67 74 64   ......;.9....gtd

00 00 00 2e 49 41 4d 2c  0d 0a 47 43 49 2c 36 65   ....IAM,..GCI,6e

39 34 36 34 62 34 31 35  65 65 31 31 63 63 38 30   9464b415ee11cc80

62 30 62 33 32 39 35 64  39 32 64 66 36 63 0d 0a   b0b3295d92df6c..

0d 0a 06 80 06 00 04 00  00 00 03 35 02 04 67 74   ...........5..gt

64 01 2e 49 41 4d 2c 0d  0a 47 43 49 2c 36 65 39   d..IAM,..GCI,6e9

34 36 34 62 34 31 35 65  65 31 31 63 63 38 30 62   464b415ee11cc80b

30 62 33 32 39 35 64 39  32 64 66 36 63 0d 0a 0d   0b3295d92df6c...

0a                                                 .

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = setup {

protocolIdentifier = 0.0.8.2250.0.4

sourceInfo = {

vendor = {

vendor = {

t35CountryCode = 181

t35Extension = 0

manufacturerCode = 18

}

}

gateway = {

protocol = 1 entries {

[0]=voice {

supportedPrefixes = 0 entries {

}

}

}

}

mc = FALSE

undefinedNode = FALSE

}

activeMC = FALSE

conferenceID =  16 octets {

6e 94 64 b4 15 ee 11 cc  80 b0 b3 29 5d 92 df 6c   n.d........)]..l

}

conferenceGoal = create <<null>>

callType = pointToPoint <<null>>

sourceCallSignalAddress = ipAddress {

ip =  4 octets {

ac 10 04 0d                                        ....

}

port = 19888

}

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

fastStart = 2 entries {

[0]= 19 octets {

00 00 00 0c 60 13 80 0b  05 00 01 00 ac 10 04 0d   ....`...........

44 4b 00                                           DK.

}

[1]= 30 octets {

40 00 00 06 04 01 00 4c  60 13 80 12 15 00 01 00   @......L`.......

ac 10 04 0d 44 4a 00 ac  10 04 0d 44 4b 00         ....DJ.....DK.

}

}

mediaWaitForConnect = FALSE

canOverlapSend = FALSE

multipleCalls = TRUE

maintainConnection = TRUE

symmetricOperationRequired = <<null>>

}

h245Tunneling = TRUE

nonStandardControl = 1 entries {

[0]={

nonStandardIdentifier = h221NonStandard {

t35CountryCode = 181

t35Extension = 0

manufacturerCode = 18

}

data =  75 octets {

e0 01 12 00 01 1c 3b 1c  39 9e 01 00 03 67 74 64   ......;.9....gtd

00 00 00 2e 49 41 4d 2c  0d 0a 47 43 49 2c 36 65   ....IAM,..GCI,6e

39 34 36 34 62 34 31 35  65 65 31 31 63 63 38 30   9464b415ee11cc80

62 30 62 33 32 39 35 64  39 32 64 66 36 63 0d 0a   b0b3295d92df6c..

0d 0a 06 80 06 00 04 00  00 00 03                  ...........

}

}

}

tunnelledSignallingMessage = {

tunnelledProtocolID = {

id = tunnelledProtocolAlternateID {

protocolType = "gtd"

}

}

messageContent = 1 entries {

[0]= 46 octets {

49 41 4d 2c 0d 0a 47 43  49 2c 36 65 39 34 36 34   IAM,..GCI,6e9464

62 34 31 35 65 65 31 31  63 63 38 30 62 30 62 33   b415ee11cc80b0b3

32 39 35 64 39 32 64 66  36 63 0d 0a 0d 0a         295d92df6c....

}

}

}

}

}

}

Raw PDU:

08 02 00 2e 05 04 03 90  90 a3 6c 0b 80 34 30 34   ..........l..404

35 32 36 36 30 36 31 70  0b 81 34 30 34 35 32 36   5266061p..404526

36 30 36 32 7e 01 12 05  20 80 06 00 08 91 4a 00   6062~... .....J.

04 28 00 b5 00 00 12 40  01 3c 05 01 00 00 6e 94   .(.....@.<....n.

64 b4 15 ee 11 cc 80 b0  b3 29 5d 92 df 6c 00 cd   d........)]..l..

1d 82 00 07 00 ac 10 04  0d 4d b0 11 00 6e 95 00   .........M...n..

dc 15 ee 11 cc 80 b2 b3  29 5d 92 df 6c 34 02 13   ........)]..l4..

00 00 00 0c 60 13 80 0b  05 00 01 00 ac 10 04 0d   ....`...........

44 4b 00 1e 40 00 00 06  04 01 00 4c 60 13 80 12   DK..@......L`...

15 00 01 00 ac 10 04 0d  44 4a 00 ac 10 04 0d 44   ........DJ.....D

4b 00 01 00 01 00 01 80  01 80 01 00 10 a8 01 80   K...............

52 01 40 b5 00 00 12 4b  e0 01 12 00 01 1c 3b 1c   R.@....K......;.

39 9e 01 00 03 67 74 64  00 00 00 2e 49 41 4d 2c   9....gtd....IAM,

0d 0a 47 43 49 2c 36 65  39 34 36 34 62 34 31 35   ..GCI,6e9464b415

65 65 31 31 63 63 38 30  62 30 62 33 32 39 35 64   ee11cc80b0b3295d

39 32 64 66 36 63 0d 0a  0d 0a 06 80 06 00 04 00   92df6c..........

00 00 03 35 02 04 67 74  64 01 2e 49 41 4d 2c 0d   ...5..gtd..IAM,.

0a 47 43 49 2c 36 65 39  34 36 34 62 34 31 35 65   .GCI,6e9464b415e

65 31 31 63 63 38 30 62  30 62 33 32 39 35 64 39   e11cc80b0b3295d9

32 64 66 36 63 0d 0a 0d  0a                        2df6c....

2:40.346          H225 Answer:90f4718   transports.cxx(1170)  H225    Incoming call, first PDU: callReference=46

2:40.347          H225 Answer:90f4718      rfc2833.cxx(81)    RFC2833 Handler created

== New H.323 Connection created.

2:40.347          H225 Answer:90f4718       h323ep.cxx(2456)  H323    Created new connection: ip$172.16.4.13:19888/46

Received non-standard Cisco extension data  75 octets {

e0 01 12 00 01 1c 3b 1c  39 9e 01 00 03 67 74 64   ......;.9....gtd

00 00 00 2e 49 41 4d 2c  0d 0a 47 43 49 2c 36 65   ....IAM,..GCI,6e

39 34 36 34 62 34 31 35  65 65 31 31 63 63 38 30   9464b415ee11cc80

62 30 62 33 32 39 35 64  39 32 64 66 36 63 0d 0a   b0b3295d92df6c..

0d 0a 06 80 06 00 04 00  00 00 03                  ...........

}

H323_UU_NonStdInfo = {

version = 18

protoParam = {

qsigNonStdInfo = {

iei = 28

rawMesg =  59 octets {

1c 39 9e 01 00 03 67 74  64 00 00 00 2e 49 41 4d   .9....gtd....IAM

2c 0d 0a 47 43 49 2c 36  65 39 34 36 34 62 34 31   ,..GCI,6e9464b41

35 65 65 31 31 63 63 38  30 62 30 62 33 32 39 35   5ee11cc80b0b3295

64 39 32 64 66 36 63 0d  0a 0d 0a                  d92df6c....

}

}

}

progIndParam = {

progIndIEinfo = {

progIndIE =  4 octets {

00 00 00 03                                        ....

}

}

}

}

Information elements collected:{

protocolDiscriminator = 8

callReference = 0

from = originator

messageType = Escape

IE: Facility = {

9e 01 00 03 67 74 64 00  00 00 2e 49 41 4d 2c 0d   ....gtd....IAM,.

0a 47 43 49 2c 36 65 39  34 36 34 62 34 31 35 65   .GCI,6e9464b415e

65 31 31 63 63 38 30 62  30 62 33 32 39 35 64 39   e11cc80b0b3295d9

32 64 66 36 63 0d 0a 0d  0a                        2df6c....

}

}

2:40.348          H225 Answer:90f4718         h323.cxx(1876)  H225    Handling PDU: Setup callRef=46

2:40.349          H225 Answer:90f4718         h323.cxx(1918)  H225    Set remote application name: "Cisco IOS 12.x    181/18"

--Received SETUP message

-- Setting up Call

--          Call token:  [ip$172.16.4.13:19888/46]

--          Calling party name:  []

--          Calling party number:  [4045266061]

--          Called party name:  [4045266062]

--          Called party number:  [4045266062]

--          Calling party IP:  [172.16.4.13]

2:40.352          H225 Answer:90f4718         h323.cxx(2142)  H225    Set protocol version to 4 and implying H.245 version 7

2:40.352          H225 Answer:90f4718         h323.cxx(2401)  H225    Set remote application name: "Cisco IOS 12.2    181/18"

Setting capabilities to 0x4 (ulaw)

Capabilities in preference order is (ulaw)

2:40.354          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: G.711-uLaw-64k <1>

2:40.354          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: UserInput/hookflash <2>

DTMF mode is 1

2:40.354          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: UserInput/RFC2833 <3>

Allowed Codecs for ip$172.16.4.13:19888/46 (ip$172.16.4.56:1720):

Table:

G.711-uLaw-64k <1>

UserInput/hookflash <2>

UserInput/RFC2833 <3>

Set:

0:

0:

G.711-uLaw-64k <1>

1:

UserInput/hookflash <2>

2:

UserInput/RFC2833 <3>

2:40.355          H225 Answer:90f4718         h323.cxx(2249)  H225    Sending call proceeding PDU

2:40.358          H225 Answer:90f4718      h323pdu.cxx(553)   H225    Sending PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = destination

messageType = CallProceeding

IE: Display = {

34 30 34 35 32 36 36 30  36 32 00                  4045266062.

}

IE: User-User = {

21 80 06 00 08 91 4a 00  04 22 c0 09 00 00 3d 36   !.....J.."....=6

54 68 65 20 4e 75 46 6f  6e 65 20 4e 65 74 77 6f   The NuFone Netwo

72 6b 73 20 48 2e 33 32  33 20 43 68 61 6e 6e 65   rks H.323 Channe

6c 20 44 72 69 76 65 72  20 66 6f 72 20 41 73 74   l Driver for Ast

65 72 69 73 6b 00 00 19  31 2e 30 2e 30 20 28 4f   erisk...1.0.0 (O

70 65 6e 48 33 32 33 20  76 31 2e 31 38 2e 30 29   penH323 v1.18.0)

00 00 00 d0 c0 11 00 6e  95 00 dc 15 ee 11 cc 80   .......n........

b2 b3 29 5d 92 df 6c 01  00 01 00 02 80 01 80      ..)]..l........

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = callProceeding {

protocolIdentifier = 0.0.8.2250.0.4

destinationInfo = {

vendor = {

vendor = {

t35CountryCode = 9

t35Extension = 0

manufacturerCode = 61

}

productId =  55 octets {

54 68 65 20 4e 75 46 6f  6e 65 20 4e 65 74 77 6f   The NuFone Netwo

72 6b 73 20 48 2e 33 32  33 20 43 68 61 6e 6e 65   rks H.323 Channe

6c 20 44 72 69 76 65 72  20 66 6f 72 20 41 73 74   l Driver for Ast

65 72 69 73 6b 00 00                               erisk..

}

versionId =  26 octets {

31 2e 30 2e 30 20 28 4f  70 65 6e 48 33 32 33 20   1.0.0 (OpenH323

76 31 2e 31 38 2e 30 29  00 00                     v1.18.0)..

}

}

terminal = {

}

mc = FALSE

undefinedNode = FALSE

}

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

multipleCalls = FALSE

maintainConnection = FALSE

}

h245Tunneling = TRUE

}

}

}

Raw PDU:

08 02 80 2e 02 28 0b 34  30 34 35 32 36 36 30 36   .....(.404526606

32 00 7e 00 80 05 21 80  06 00 08 91 4a 00 04 22   2.~...!.....J.."

c0 09 00 00 3d 36 54 68  65 20 4e 75 46 6f 6e 65   ....=6The NuFone

20 4e 65 74 77 6f 72 6b  73 20 48 2e 33 32 33 20    Networks H.323

43 68 61 6e 6e 65 6c 20  44 72 69 76 65 72 20 66   Channel Driver f

6f 72 20 41 73 74 65 72  69 73 6b 00 00 19 31 2e   or Asterisk...1.

30 2e 30 20 28 4f 70 65  6e 48 33 32 33 20 76 31   0.0 (OpenH323 v1

2e 31 38 2e 30 29 00 00  00 d0 c0 11 00 6e 95 00   .18.0).......n..

dc 15 ee 11 cc 80 b2 b3  29 5d 92 df 6c 01 00 01   ........)]..l...

00 02 80 01 80                                     .....

2:40.360          H225 Answer:90f4718         h323.cxx(2288)  H225    Incoming call accepted

2:40.360          H225 Answer:90f4718         h323.cxx(2150)  H225    Fast start detected

2:40.361          H225 Answer:90f4718         h323.cxx(2157)  H225    Fast start open:

{

forwardLogicalChannelNumber = 1

forwardLogicalChannelParameters = {

dataType = audioData g711Ulaw64k 20

multiplexParameters = h2250LogicalChannelParameters {

sessionID = 1

mediaControlChannel = unicastAddress iPAddress {

network =  4 octets {

ac 10 04 0d                                        ....

}

tsapIdentifier = 17483

}

silenceSuppression = FALSE

}

}

}

2:40.365          H225 Answer:90f4718         h323.cxx(4691)  H323    CreateLogicalChannel - forward channel

2:40.365          H225 Answer:90f4718     h323caps.cxx(2330)  H323    FindCapability: audioData

2:40.366          H225 Answer:90f4718     h323caps.cxx(1017)  H323    Capability tx frames reduced from 150 to 20

2:40.366          H225 Answer:90f4718     h323caps.cxx(2375)  H323    Found capability: G.711-uLaw-64k <1>

2:40.366          H225 Answer:90f4718     h323caps.cxx(1022)  H323    Capability rx frames left at 20 as remote allows 20

== Using TOS bits 0

== Using CoS mark 0

2:40.368          H225 Answer:90f4718     channels.cxx(979)   H323RTP OnReceivedPDU for channel: R-1

2:40.368          H225 Answer:90f4718     h323caps.cxx(1022)  H323    Capability rx frames left at 20 as remote allows 20

2:40.369          H225 Answer:90f4718         h323.cxx(2157)  H225    Fast start open:

{

forwardLogicalChannelNumber = 1

forwardLogicalChannelParameters = {

dataType = nullData <<null>>

multiplexParameters = none <<null>>

}

reverseLogicalChannelParameters = {

dataType = audioData g711Ulaw64k 20

multiplexParameters = h2250LogicalChannelParameters {

sessionID = 1

mediaChannel = unicastAddress iPAddress {

network =  4 octets {

ac 10 04 0d                                        ....

}

tsapIdentifier = 17482

}

mediaControlChannel = unicastAddress iPAddress {

network =  4 octets {

ac 10 04 0d                                        ....

}

tsapIdentifier = 17483

}

silenceSuppression = FALSE

}

}

}

2:40.369          H225 Answer:90f4718         h323.cxx(4676)  H323    CreateLogicalChannel - reverse channel

2:40.370          H225 Answer:90f4718     h323caps.cxx(2330)  H323    FindCapability: audioData

2:40.371          H225 Answer:90f4718     h323caps.cxx(1017)  H323    Capability tx frames reduced from 150 to 20

2:40.372          H225 Answer:90f4718     h323caps.cxx(2375)  H323    Found capability: G.711-uLaw-64k <1>

2:40.372          H225 Answer:90f4718     h323caps.cxx(1017)  H323    Capability tx frames reduced from 150 to 20

2:40.372          H225 Answer:90f4718     h323caps.cxx(2267)  H323    FindCapability: G.711-uLaw-64k <1>

2:40.372          H225 Answer:90f4718     h323caps.cxx(2153)  H323    Added capability: G.711-uLaw-64k <1>

2:40.373          H225 Answer:90f4718     channels.cxx(979)   H323RTP OnReceivedPDU for channel: T-0

2:40.373          H225 Answer:90f4718     h323caps.cxx(1022)  H323    Capability tx frames left at 20 as remote allows 20

2:40.373          H225 Answer:90f4718         h323.cxx(2171)  H225    Opened 2 fast start channels

=-= In OnAnswerCall for call 46

- Progress Indicator: 0

- Inserting PI of 0 into ALERTING message

2:40.375          H225 Answer:90f4718         h323.cxx(2792)  H323    Answering call: AnswerCallDeferredWithMedia

2:40.376          H225 Answer:90f4718         h323.cxx(4445)  H245    Default OnSelectLogicalChannels, FastStartResponse

-- Started logical channel: sending G.711-uLaw-64k

-- channelsOpen = 1

External RTP Session Starting

RTP channel id 1 parameters:

-- remoteIpAddress: 172.16.4.13

-- remotePort: 17482

-- ExternalIpAddress: 172.16.4.56

-- ExternalPort: 15896

-- Started logical channel: receiving G.711-uLaw-64k

-- channelsOpen = 2

External RTP Session Starting

RTP channel id 1 parameters:

-- remoteIpAddress: 172.16.4.13

-- remotePort: 17482

-- ExternalIpAddress: 172.16.4.56

-- ExternalPort: 15896

2:40.377          H225 Answer:90f4718         h323.cxx(3376)  H225    Accepting fastStart for 2 channels

2:40.378          H225 Answer:90f4718     channels.cxx(920)   H323RTP OnSendingPDU

2:40.380          H225 Answer:90f4718         h323.cxx(2094)  H225    Build fastStart:

{

forwardLogicalChannelNumber = 1

forwardLogicalChannelParameters = {

dataType = audioData g711Ulaw64k 150

multiplexParameters = h2250LogicalChannelParameters {

sessionID = 1

mediaChannel = unicastAddress iPAddress {

network =  4 octets {

ac 10 04 38                                        ...8

}

tsapIdentifier = 15896

}

mediaGuaranteedDelivery = FALSE

mediaControlChannel = unicastAddress iPAddress {

network =  4 octets {

ac 10 04 38                                        ...8

}

tsapIdentifier = 15897

}

silenceSuppression = FALSE

}

}

}

2:40.380          H225 Answer:90f4718         h323.cxx(2099)  H225    Built fastStart for G.711-uLaw-64k <1>

2:40.381          H225 Answer:90f4718     channels.cxx(920)   H323RTP OnSendingPDU

2:40.382          H225 Answer:90f4718         h323.cxx(2094)  H225    Build fastStart:

{

forwardLogicalChannelNumber = 101

forwardLogicalChannelParameters = {

dataType = nullData <<null>>

multiplexParameters = none <<null>>

}

reverseLogicalChannelParameters = {

dataType = audioData g711Ulaw64k 20

multiplexParameters = h2250LogicalChannelParameters {

sessionID = 1

mediaGuaranteedDelivery = FALSE

mediaControlChannel = unicastAddress iPAddress {

network =  4 octets {

ac 10 04 38                                        ...8

}

tsapIdentifier = 15897

}

silenceSuppression = FALSE

}

}

}

2:40.382          H225 Answer:90f4718         h323.cxx(2099)  H225    Built fastStart for G.711-uLaw-64k <1>

-- Executing [4045266062@n_DstGW:1] NoOp("H323/ip$172.16.4.13:19888/46", "DstGW") in new stack

-- Executing [4045266062@n_DstGW:2] Ringing("H323/ip$172.16.4.13:19888/46", "") in new stack

Sending alerting

2:40.386          H225 Answer:90f4718      h323pdu.cxx(553)   H225    Sending PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = destination

messageType = Progress

IE: Progress-Indicator = {

80 88                                              ..

}

IE: Display = {

34 30 34 35 32 36 36 30  36 32 00                  4045266062.

}

IE: User-User = {

28 00 80 ac 84 06 00 08  91 4a 00 04 22 c0 09 00   (........J.."...

00 3d 36 54 68 65 20 4e  75 46 6f 6e 65 20 4e 65   .=6The NuFone Ne

74 77 6f 72 6b 73 20 48  2e 33 32 33 20 43 68 61   tworks H.323 Cha

6e 6e 65 6c 20 44 72 69  76 65 72 20 66 6f 72 20   nnel Driver for

41 73 74 65 72 69 73 6b  00 00 19 31 2e 30 2e 30   Asterisk...1.0.0

20 28 4f 70 65 6e 48 33  32 33 20 76 31 2e 31 38    (OpenH323 v1.18

2e 30 29 00 00 00 6e 95  00 dc 15 ee 11 cc 80 b2   .0)...n.........

b3 29 5d 92 df 6c 02 1a  00 00 00 0c 60 95 80 12   .)]..l......`...

1d 00 01 00 ac 10 04 38  3e 18 00 ac 10 04 38 3e   .......8>.....8>

19 00 17 40 00 64 06 04  01 00 4c 60 13 80 0b 0d   ...@.d....L`....

00 01 00 ac 10 04 38 3e  19 00 03 80 01 00 01 00   ......8>........

02 80 01 80                                        ....

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = progress {

protocolIdentifier = 0.0.8.2250.0.4

destinationInfo = {

vendor = {

vendor = {

t35CountryCode = 9

t35Extension = 0

manufacturerCode = 61

}

productId =  55 octets {

54 68 65 20 4e 75 46 6f  6e 65 20 4e 65 74 77 6f   The NuFone Netwo

72 6b 73 20 48 2e 33 32  33 20 43 68 61 6e 6e 65   rks H.323 Channe

6c 20 44 72 69 76 65 72  20 66 6f 72 20 41 73 74   l Driver for Ast

65 72 69 73 6b 00 00                               erisk..

}

versionId =  26 octets {

31 2e 30 2e 30 20 28 4f  70 65 6e 48 33 32 33 20   1.0.0 (OpenH323

76 31 2e 31 38 2e 30 29  00 00                     v1.18.0)..

}

}

terminal = {

}

mc = FALSE

undefinedNode = FALSE

}

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

fastStart = 2 entries {

[0]= 26 octets {

00 00 00 0c 60 95 80 12  1d 00 01 00 ac 10 04 38   ....`..........8

3e 18 00 ac 10 04 38 3e  19 00                     >.....8>..

}

[1]= 23 octets {

40 00 64 06 04 01 00 4c  60 13 80 0b 0d 00 01 00   @.d....L`.......

ac 10 04 38 3e 19 00                               ...8>..

}

}

multipleCalls = FALSE

maintainConnection = FALSE

}

h245Tunneling = TRUE

}

}

}

Raw PDU:

08 02 80 2e 03 1e 02 80  88 28 0b 34 30 34 35 32   .........(.40452

36 36 30 36 32 00 7e 00  b5 05 28 00 80 ac 84 06   66062.~...(.....

00 08 91 4a 00 04 22 c0  09 00 00 3d 36 54 68 65   ...J.."....=6The

20 4e 75 46 6f 6e 65 20  4e 65 74 77 6f 72 6b 73    NuFone Networks

20 48 2e 33 32 33 20 43  68 61 6e 6e 65 6c 20 44    H.323 Channel D

72 69 76 65 72 20 66 6f  72 20 41 73 74 65 72 69   river for Asteri

73 6b 00 00 19 31 2e 30  2e 30 20 28 4f 70 65 6e   sk...1.0.0 (Open

48 33 32 33 20 76 31 2e  31 38 2e 30 29 00 00 00   H323 v1.18.0)...

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

02 1a 00 00 00 0c 60 95  80 12 1d 00 01 00 ac 10   ......`.........

04 38 3e 18 00 ac 10 04  38 3e 19 00 17 40 00 64   .8>.....8>...@.d

06 04 01 00 4c 60 13 80  0b 0d 00 01 00 ac 10 04   ....L`..........

38 3e 19 00 03 80 01 00  01 00 02 80 01 80         8>............

2:40.388          H225 Answer:90f4718         h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=AwaitingLocalAnswer fastStartState=FastStartAcknowledged

2:40.389          H225 Answer:90f4718         h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=AwaitingLocalAnswer fastStartState=FastStartAcknowledged

2:40.389          H225 Answer:90f4718         h323.cxx(1816)  H225    Reading PDUs: callRef=46

2:40.405      ThreadID=0xf5eacbb0             h323.cxx(2792)  H323    Answering call: AnswerCallPending

2:40.405      ThreadID=0xf5eacbb0             h323.cxx(2881)  H225    Sending Alerting PDU

2:40.406      ThreadID=0xf5eacbb0          h323pdu.cxx(553)   H225    Sending PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = destination

messageType = Alerting

IE: Display = {

34 30 34 35 32 36 36 30  36 32 00                  4045266062.

}

IE: User-User = {

23 80 06 00 08 91 4a 00  04 22 c0 09 00 00 3d 36   #.....J.."....=6

54 68 65 20 4e 75 46 6f  6e 65 20 4e 65 74 77 6f   The NuFone Netwo

72 6b 73 20 48 2e 33 32  33 20 43 68 61 6e 6e 65   rks H.323 Channe

6c 20 44 72 69 76 65 72  20 66 6f 72 20 41 73 74   l Driver for Ast

65 72 69 73 6b 00 00 19  31 2e 30 2e 30 20 28 4f   erisk...1.0.0 (O

70 65 6e 48 33 32 33 20  76 31 2e 31 38 2e 30 29   penH323 v1.18.0)

00 00 00 d0 c0 11 00 6e  95 00 dc 15 ee 11 cc 80   .......n........

b2 b3 29 5d 92 df 6c 01  00 01 00 02 80 01 80      ..)]..l........

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = alerting {

protocolIdentifier = 0.0.8.2250.0.4

destinationInfo = {

vendor = {

vendor = {

t35CountryCode = 9

t35Extension = 0

manufacturerCode = 61

}

productId =  55 octets {

54 68 65 20 4e 75 46 6f  6e 65 20 4e 65 74 77 6f   The NuFone Netwo

72 6b 73 20 48 2e 33 32  33 20 43 68 61 6e 6e 65   rks H.323 Channe

6c 20 44 72 69 76 65 72  20 66 6f 72 20 41 73 74   l Driver for Ast

65 72 69 73 6b 00 00                               erisk..

}

versionId =  26 octets {

31 2e 30 2e 30 20 28 4f  70 65 6e 48 33 32 33 20   1.0.0 (OpenH323

76 31 2e 31 38 2e 30 29  00 00                     v1.18.0)..

}

}

terminal = {

}

mc = FALSE

undefinedNode = FALSE

}

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

multipleCalls = FALSE

maintainConnection = FALSE

}

h245Tunneling = TRUE

}

}

}

Raw PDU:

08 02 80 2e 01 28 0b 34  30 34 35 32 36 36 30 36   .....(.404526606

32 00 7e 00 80 05 23 80  06 00 08 91 4a 00 04 22   2.~...#.....J.."

c0 09 00 00 3d 36 54 68  65 20 4e 75 46 6f 6e 65   ....=6The NuFone

20 4e 65 74 77 6f 72 6b  73 20 48 2e 33 32 33 20    Networks H.323

43 68 61 6e 6e 65 6c 20  44 72 69 76 65 72 20 66   Channel Driver f

6f 72 20 41 73 74 65 72  69 73 6b 00 00 19 31 2e   or Asterisk...1.

30 2e 30 20 28 4f 70 65  6e 48 33 32 33 20 76 31   0.0 (OpenH323 v1

2e 31 38 2e 30 29 00 00  00 d0 c0 11 00 6e 95 00   .18.0).......n..

dc 15 ee 11 cc 80 b2 b3  29 5d 92 df 6c 01 00 01   ........)]..l...

00 02 80 01 80                                     .....

2:40.407      ThreadID=0xf5eacbb0             h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=AwaitingLocalAnswer fastStartState=FastStartAcknowledged

-- Executing [4045266062@n_DstGW:3] Wait("H323/ip$172.16.4.13:19888/46", "1") in new stack

-- Executing [4045266062@n_DstGW:4] Dial("H323/ip$172.16.4.13:19888/46", "Zap/1|15|o") in new stack

-- Called 1

-- Zap/1-1 is ringing

Sending alerting

2:41.480      ThreadID=0xf5eacbb0             h323.cxx(2792)  H323    Answering call: AnswerCallPending

2:41.480      ThreadID=0xf5eacbb0             h323.cxx(2881)  H225    Sending Alerting PDU

2:41.481      ThreadID=0xf5eacbb0          h323pdu.cxx(553)   H225    Sending PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = destination

messageType = Alerting

IE: Display = {

34 30 34 35 32 36 36 30  36 32 00                  4045266062.

}

IE: User-User = {

23 80 06 00 08 91 4a 00  04 22 c0 09 00 00 3d 36   #.....J.."....=6

54 68 65 20 4e 75 46 6f  6e 65 20 4e 65 74 77 6f   The NuFone Netwo

72 6b 73 20 48 2e 33 32  33 20 43 68 61 6e 6e 65   rks H.323 Channe

6c 20 44 72 69 76 65 72  20 66 6f 72 20 41 73 74   l Driver for Ast

65 72 69 73 6b 00 00 19  31 2e 30 2e 30 20 28 4f   erisk...1.0.0 (O

70 65 6e 48 33 32 33 20  76 31 2e 31 38 2e 30 29   penH323 v1.18.0)

00 00 00 d0 c0 11 00 6e  95 00 dc 15 ee 11 cc 80   .......n........

b2 b3 29 5d 92 df 6c 01  00 01 00 02 80 01 80      ..)]..l........

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = alerting {

protocolIdentifier = 0.0.8.2250.0.4

destinationInfo = {

vendor = {

vendor = {

t35CountryCode = 9

t35Extension = 0

manufacturerCode = 61

}

productId =  55 octets {

54 68 65 20 4e 75 46 6f  6e 65 20 4e 65 74 77 6f   The NuFone Netwo

72 6b 73 20 48 2e 33 32  33 20 43 68 61 6e 6e 65   rks H.323 Channe

6c 20 44 72 69 76 65 72  20 66 6f 72 20 41 73 74   l Driver for Ast

65 72 69 73 6b 00 00                               erisk..

}

versionId =  26 octets {

31 2e 30 2e 30 20 28 4f  70 65 6e 48 33 32 33 20   1.0.0 (OpenH323

76 31 2e 31 38 2e 30 29  00 00                     v1.18.0)..

}

}

terminal = {

}

mc = FALSE

undefinedNode = FALSE

}

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

multipleCalls = FALSE

maintainConnection = FALSE

}

h245Tunneling = TRUE

}

}

}

Raw PDU:

08 02 80 2e 01 28 0b 34  30 34 35 32 36 36 30 36   .....(.404526606

32 00 7e 00 80 05 23 80  06 00 08 91 4a 00 04 22   2.~...#.....J.."

c0 09 00 00 3d 36 54 68  65 20 4e 75 46 6f 6e 65   ....=6The NuFone

20 4e 65 74 77 6f 72 6b  73 20 48 2e 33 32 33 20    Networks H.323

43 68 61 6e 6e 65 6c 20  44 72 69 76 65 72 20 66   Channel Driver f

6f 72 20 41 73 74 65 72  69 73 6b 00 00 19 31 2e   or Asterisk...1.

30 2e 30 20 28 4f 70 65  6e 48 33 32 33 20 76 31   0.0 (OpenH323 v1

2e 31 38 2e 30 29 00 00  00 d0 c0 11 00 6e 95 00   .18.0).......n..

dc 15 ee 11 cc 80 b2 b3  29 5d 92 df 6c 01 00 01   ........)]..l...

00 02 80 01 80                                     .....

2:41.482      ThreadID=0xf5eacbb0             h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=AwaitingLocalAnswer fastStartState=FastStartAcknowledged

-- Zap/1-1 is ringing

-- Zap/1-1 answered H323/ip$172.16.4.13:19888/46

Answering call ip$172.16.4.13:19888/46

2:44.717      ThreadID=0xf5eacbb0             h323.cxx(2792)  H323    Answering call: AnswerCallNow

2:44.717      ThreadID=0xf5eacbb0             h323.cxx(3592)  H245    Started control channel

2:44.717      ThreadID=0xf5eacbb0          h323neg.cxx(560)   H245    Sending TerminalCapabilitySet: outSeq=1

-- Receiving RFC2833 on payload 101

2:44.720      ThreadID=0xf5eacbb0          h323pdu.cxx(553)   H245    Sending PDU [(noaddr)/(noaddr)] :

request terminalCapabilitySet {

sequenceNumber = 1

protocolIdentifier = 0.0.8.245.0.11

multiplexCapability = h2250Capability {

maximumAudioDelayJitter = 250

receiveMultipointCapability = {

multicastCapability = FALSE

multiUniCastConference = FALSE

mediaDistributionCapability = 1 entries {

[0]={

centralizedControl = FALSE

distributedControl = FALSE

centralizedAudio = FALSE

distributedAudio = FALSE

centralizedVideo = FALSE

distributedVideo = FALSE

}

}

}

transmitMultipointCapability = {

multicastCapability = FALSE

multiUniCastConference = FALSE

mediaDistributionCapability = 1 entries {

[0]={

centralizedControl = FALSE

distributedControl = FALSE

centralizedAudio = FALSE

distributedAudio = FALSE

centralizedVideo = FALSE

distributedVideo = FALSE

}

}

}

receiveAndTransmitMultipointCapability = {

multicastCapability = FALSE

multiUniCastConference = FALSE

mediaDistributionCapability = 1 entries {

[0]={

centralizedControl = FALSE

distributedControl = FALSE

centralizedAudio = FALSE

distributedAudio = FALSE

centralizedVideo = FALSE

distributedVideo = FALSE

}

}

}

mcCapability = {

centralizedConferenceMC = FALSE

decentralizedConferenceMC = FALSE

}

rtcpVideoControlCapability = FALSE

mediaPacketizationCapability = {

h261aVideoPacketization = FALSE

}

logicalChannelSwitchingCapability = FALSE

t120DynamicPortCapability = TRUE

}

capabilityTable = 3 entries {

[0]={

capabilityTableEntryNumber = 1

capability = receiveAudioCapability g711Ulaw64k 20

}

[1]={

capabilityTableEntryNumber = 2

capability = receiveUserInputCapability hookflash <<null>>

}

[2]={

capabilityTableEntryNumber = 3

capability = receiveRTPAudioTelephonyEventCapability {

dynamicRTPPayloadType = 101

audioTelephoneEvent = "0-16"

}

}

}

capabilityDescriptors = 1 entries {

[0]={

capabilityDescriptorNumber = 1

simultaneousCapabilities = 3 entries {

[0]=1 entries {

[0]=1

}

[1]=1 entries {

[0]=2

}

[2]=1 entries {

[0]=3

}

}

}

}

}

Raw PDU:

size=44 pos=7fffffff.0 {

000 02 70 01 06 00 08 81 75 00 0b 80 13 80 00 fa 00    p     u

01 00 00 01 00 00 01 00 00 0c c0 01 00 01 80 02

80 00 00 20 c0 13 80 00 01 83 01 50 80 00 02 8a              P

06 14 04 30 2d 31 36 00 80 01 02 00 00 00 00 00      0-16

01 00 00 02

}

2:44.721      ThreadID=0xf5eacbb0          h323neg.cxx(323)   H245    Sending MasterSlaveDetermination

2:44.724      ThreadID=0xf5eacbb0          h323pdu.cxx(553)   H245    Sending PDU [(noaddr)/(noaddr)] :

request masterSlaveDetermination {

terminalType = 50

statusDeterminationNumber = 5698548

}

Raw PDU:

size=7 pos=7fffffff.0 {

000 01 00 32 80 56 f3 f4                                2 V

}

2:44.726      ThreadID=0xf5eacbb0          h323pdu.cxx(553)   H225    Sending PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = destination

messageType = Connect

IE: Bearer-Capability = {

80 90 a5                                           ...

}

IE: Display = {

34 30 34 35 32 36 36 30  36 32 00                  4045266062.

}

IE: User-User = {

22 80 06 00 08 91 4a 00  04 22 c0 09 00 00 3d 36   ".....J.."....=6

54 68 65 20 4e 75 46 6f  6e 65 20 4e 65 74 77 6f   The NuFone Netwo

72 6b 73 20 48 2e 33 32  33 20 43 68 61 6e 6e 65   rks H.323 Channe

6c 20 44 72 69 76 65 72  20 66 6f 72 20 41 73 74   l Driver for Ast

65 72 69 73 6b 00 00 19  31 2e 30 2e 30 20 28 4f   erisk...1.0.0 (O

70 65 6e 48 33 32 33 20  76 31 2e 31 38 2e 30 29   penH323 v1.18.0)

00 00 00 6e 94 64 b4 15  ee 11 cc 80 b0 b3 29 5d   ...n.d........)]

92 df 6c 17 0c 20 11 00  6e 95 00 dc 15 ee 11 cc   ..l.. ..n.......

80 b2 b3 29 5d 92 df 6c  01 00 01 00 01 00 04 c0   ...)]..l........

01 80 4e 02 44 02 70 01  06 00 08 81 75 00 0b 80   ..N.D.p.....u...

13 80 00 fa 00 01 00 00  01 00 00 01 00 00 0c c0   ................

01 00 01 80 02 80 00 00  20 c0 13 80 00 01 83 01   ........ .......

50 80 00 02 8a 06 14 04  30 2d 31 36 00 80 01 02   P.......0-16....

00 00 00 00 00 01 00 00  02 07 01 00 32 80 56 f3   ............2.V.

f4                                                 .

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = connect {

protocolIdentifier = 0.0.8.2250.0.4

destinationInfo = {

vendor = {

vendor = {

t35CountryCode = 9

t35Extension = 0

manufacturerCode = 61

}

productId =  55 octets {

54 68 65 20 4e 75 46 6f  6e 65 20 4e 65 74 77 6f   The NuFone Netwo

72 6b 73 20 48 2e 33 32  33 20 43 68 61 6e 6e 65   rks H.323 Channe

6c 20 44 72 69 76 65 72  20 66 6f 72 20 41 73 74   l Driver for Ast

65 72 69 73 6b 00 00                               erisk..

}

versionId =  26 octets {

31 2e 30 2e 30 20 28 4f  70 65 6e 48 33 32 33 20   1.0.0 (OpenH323

76 31 2e 31 38 2e 30 29  00 00                     v1.18.0)..

}

}

terminal = {

}

mc = FALSE

undefinedNode = FALSE

}

conferenceID =  16 octets {

6e 94 64 b4 15 ee 11 cc  80 b0 b3 29 5d 92 df 6c   n.d........)]..l

}

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

multipleCalls = FALSE

maintainConnection = FALSE

fastConnectRefused = <<null>>

}

h245Tunneling = TRUE

h245Control = 2 entries {

[0]= 68 octets {

02 70 01 06 00 08 81 75  00 0b 80 13 80 00 fa 00   .p.....u........

01 00 00 01 00 00 01 00  00 0c c0 01 00 01 80 02   ................

80 00 00 20 c0 13 80 00  01 83 01 50 80 00 02 8a   ... .......P....

06 14 04 30 2d 31 36 00  80 01 02 00 00 00 00 00   ...0-16.........

01 00 00 02                                        ....

}

[1]= 7 octets {

01 00 32 80 56 f3 f4                               ..2.V..

}

}

}

}

}

Raw PDU:

08 02 80 2e 07 04 03 80  90 a5 28 0b 34 30 34 35   ..........(.4045

32 36 36 30 36 32 00 7e  00 e2 05 22 80 06 00 08   266062.~..."....

91 4a 00 04 22 c0 09 00  00 3d 36 54 68 65 20 4e   .J.."....=6The N

75 46 6f 6e 65 20 4e 65  74 77 6f 72 6b 73 20 48   uFone Networks H

2e 33 32 33 20 43 68 61  6e 6e 65 6c 20 44 72 69   .323 Channel Dri

76 65 72 20 66 6f 72 20  41 73 74 65 72 69 73 6b   ver for Asterisk

00 00 19 31 2e 30 2e 30  20 28 4f 70 65 6e 48 33   ...1.0.0 (OpenH3

32 33 20 76 31 2e 31 38  2e 30 29 00 00 00 6e 94   23 v1.18.0)...n.

64 b4 15 ee 11 cc 80 b0  b3 29 5d 92 df 6c 17 0c   d........)]..l..

20 11 00 6e 95 00 dc 15  ee 11 cc 80 b2 b3 29 5d    ..n..........)]

92 df 6c 01 00 01 00 01  00 04 c0 01 80 4e 02 44   ..l..........N.D

02 70 01 06 00 08 81 75  00 0b 80 13 80 00 fa 00   .p.....u........

01 00 00 01 00 00 01 00  00 0c c0 01 00 01 80 02   ................

80 00 00 20 c0 13 80 00  01 83 01 50 80 00 02 8a   ... .......P....

06 14 04 30 2d 31 36 00  80 01 02 00 00 00 00 00   ...0-16.........

01 00 00 02 07 01 00 32  80 56 f3 f4               .......2.V..

2:44.728      ThreadID=0xf5eacbb0             h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=HasExecutedSignalConnect fastStartState=FastStartDisabled

2:44.808          H225 Answer:90f4718      h323pdu.cxx(553)   H225    Receiving PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = originator

messageType = Facility

IE: Facility = {

}

IE: Display = {

63 69 73 63 6f 20 53 79  73 74 65 6d 73 2c 20 49   cisco Systems, I

6e 63 2e                                           nc.

}

IE: User-User = {

26 90 06 00 08 91 4a 00  04 6e 94 64 b4 15 ee 11   &.....J..n.d....

cc 80 b0 b3 29 5d 92 df  6c 86 01 00 1f 01 80 11   ....)]..l.......

00 6e 95 00 dc 15 ee 11  cc 80 b2 b3 29 5d 92 df   .n..........)]..

6c 01 00 01 00 10 c0 01  80 70 01 6e 02 70 01 06   l........p.n.p..

00 08 81 75 00 07 80 13  80 00 14 00 01 00 00 01   ...u............

00 00 01 00 00 0c c0 01  00 01 00 05 80 00 1d 8a   ................

06 14 04 30 2d 31 36 80  00 13 48 10 b5 00 00 12   ...0-16...H.....

0c 52 74 70 44 74 6d 66  52 65 6c 61 79 00 00 80   .RtpDtmfRelay...

00 19 83 01 50 80 00 18  83 01 40 80 00 15 83 01   ....P.....@.....

10 80 00 00 20 c0 13 00  80 01 02 00 00 00 03 00   .... ...........

1d 00 18 00 15 00 13 00  00 19                     ..........

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = facility {

protocolIdentifier = 0.0.8.2250.0.4

conferenceID =  16 octets {

6e 94 64 b4 15 ee 11 cc  80 b0 b3 29 5d 92 df 6c   n.d........)]..l

}

reason = transportedInformation <<null>>

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

multipleCalls = FALSE

maintainConnection = FALSE

}

h245Tunneling = TRUE

h245Control = 1 entries {

[0]= 110 octets {

02 70 01 06 00 08 81 75  00 07 80 13 80 00 14 00   .p.....u........

01 00 00 01 00 00 01 00  00 0c c0 01 00 01 00 05   ................

80 00 1d 8a 06 14 04 30  2d 31 36 80 00 13 48 10   .......0-16...H.

b5 00 00 12 0c 52 74 70  44 74 6d 66 52 65 6c 61   .....RtpDtmfRela

79 00 00 80 00 19 83 01  50 80 00 18 83 01 40 80   y.......P.....@.

00 15 83 01 10 80 00 00  20 c0 13 00 80 01 02 00   ........ .......

00 00 03 00 1d 00 18 00  15 00 13 00 00 19         ..............

}

}

}

}

}

Raw PDU:

08 02 00 2e 62 1c 00 28  13 63 69 73 63 6f 20 53   ....b..(.cisco S

79 73 74 65 6d 73 2c 20  49 6e 63 2e 7e 00 ab 05   ystems, Inc.~...

26 90 06 00 08 91 4a 00  04 6e 94 64 b4 15 ee 11   &.....J..n.d....

cc 80 b0 b3 29 5d 92 df  6c 86 01 00 1f 01 80 11   ....)]..l.......

00 6e 95 00 dc 15 ee 11  cc 80 b2 b3 29 5d 92 df   .n..........)]..

6c 01 00 01 00 10 c0 01  80 70 01 6e 02 70 01 06   l........p.n.p..

00 08 81 75 00 07 80 13  80 00 14 00 01 00 00 01   ...u............

00 00 01 00 00 0c c0 01  00 01 00 05 80 00 1d 8a   ................

06 14 04 30 2d 31 36 80  00 13 48 10 b5 00 00 12   ...0-16...H.....

0c 52 74 70 44 74 6d 66  52 65 6c 61 79 00 00 80   .RtpDtmfRelay...

00 19 83 01 50 80 00 18  83 01 40 80 00 15 83 01   ....P.....@.....

10 80 00 00 20 c0 13 00  80 01 02 00 00 00 03 00   .... ...........

1d 00 18 00 15 00 13 00  00 19                     ..........

2:44.810          H225 Answer:90f4718         h323.cxx(1876)  H225    Handling PDU: Facility callRef=46

-- Received Facility message...

2:44.810          H225 Answer:90f4718         h323.cxx(2142)  H225    Set protocol version to 4 and implying H.245 version 7

2:44.812          H225 Answer:90f4718      h323pdu.cxx(553)   H245    Receiving PDU [(noaddr)/(noaddr)] :

request terminalCapabilitySet {

sequenceNumber = 1

protocolIdentifier = 0.0.8.245.0.7

multiplexCapability = h2250Capability {

maximumAudioDelayJitter = 20

receiveMultipointCapability = {

multicastCapability = FALSE

multiUniCastConference = FALSE

mediaDistributionCapability = 1 entries {

[0]={

centralizedControl = FALSE

distributedControl = FALSE

centralizedAudio = FALSE

distributedAudio = FALSE

centralizedVideo = FALSE

distributedVideo = FALSE

}

}

}

transmitMultipointCapability = {

multicastCapability = FALSE

multiUniCastConference = FALSE

mediaDistributionCapability = 1 entries {

[0]={

centralizedControl = FALSE

distributedControl = FALSE

centralizedAudio = FALSE

distributedAudio = FALSE

centralizedVideo = FALSE

distributedVideo = FALSE

}

}

}

receiveAndTransmitMultipointCapability = {

multicastCapability = FALSE

multiUniCastConference = FALSE

mediaDistributionCapability = 1 entries {

[0]={

centralizedControl = FALSE

distributedControl = FALSE

centralizedAudio = FALSE

distributedAudio = FALSE

centralizedVideo = FALSE

distributedVideo = FALSE

}

}

}

mcCapability = {

centralizedConferenceMC = FALSE

decentralizedConferenceMC = FALSE

}

rtcpVideoControlCapability = FALSE

mediaPacketizationCapability = {

h261aVideoPacketization = FALSE

}

logicalChannelSwitchingCapability = FALSE

t120DynamicPortCapability = FALSE

}

capabilityTable = 6 entries {

[0]={

capabilityTableEntryNumber = 30

capability = receiveRTPAudioTelephonyEventCapability {

dynamicRTPPayloadType = 101

audioTelephoneEvent = "0-16"

}

}

[1]={

capabilityTableEntryNumber = 20

capability = receiveAndTransmitDataApplicationCapability {

application = nonStandard {

nonStandardIdentifier = h221NonStandard {

t35CountryCode = 181

t35Extension = 0

manufacturerCode = 18

}

data =  12 octets {

52 74 70 44 74 6d 66 52  65 6c 61 79               RtpDtmfRelay

}

}

maxBitRate = 0

}

}

[2]={

capabilityTableEntryNumber = 26

capability = receiveUserInputCapability hookflash <<null>>

}

[3]={

capabilityTableEntryNumber = 25

capability = receiveUserInputCapability dtmf <<null>>

}

[4]={

capabilityTableEntryNumber = 22

capability = receiveUserInputCapability basicString <<null>>

}

[5]={

capabilityTableEntryNumber = 1

capability = receiveAudioCapability g711Ulaw64k 20

}

}

capabilityDescriptors = 1 entries {

[0]={

capabilityDescriptorNumber = 1

simultaneousCapabilities = 3 entries {

[0]=1 entries {

[0]=1

}

[1]=4 entries {

[0]=30

[1]=25

[2]=22

[3]=20

}

[2]=1 entries {

[0]=26

}

}

}

}

}

Raw PDU:

size=6e pos=6e.0 {

000 02 70 01 06 00 08 81 75 00 07 80 13 80 00 14 00    p     u

01 00 00 01 00 00 01 00 00 0c c0 01 00 01 00 05

80 00 1d 8a 06 14 04 30 2d 31 36 80 00 13 48 10          0-16   H

b5 00 00 12 0c 52 74 70 44 74 6d 66 52 65 6c 61        RtpDtmfRela

79 00 00 80 00 19 83 01 50 80 00 18 83 01 40 80   y       P     @

00 15 83 01 10 80 00 00 20 c0 13 00 80 01 02 00

00 00 03 00 1d 00 18 00 15 00 13 00 00 19

}

2:44.813          H225 Answer:90f4718         h323.cxx(3767)  H245    Set protocol version to 7

2:44.813          H225 Answer:90f4718      h323neg.cxx(591)   H245    Received TerminalCapabilitySet: state=InProgress pduSeq=1 inSeq=4294967295

2:44.813          H225 Answer:90f4718     h323caps.cxx(2153)  H323    Added capability: G.711-uLaw-64k <1>

2:44.814          H225 Answer:90f4718     h323caps.cxx(2153)  H323    Added capability: UserInput/hookflash <2>

2:44.814          H225 Answer:90f4718     h323caps.cxx(2153)  H323    Added capability: UserInput/RFC2833 <3>

2:44.814          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.711-ALaw-64k"

2:44.815          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: G.711-ALaw-64k <4>

2:44.815          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.711-ALaw-64k{hw}"

2:44.815          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.711-ALaw-64k{sw}"

2:44.816          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: G.711-ALaw-64k <5>

2:44.816          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.711-uLaw-64k"

2:44.816          H225 Answer:90f4718     h323caps.cxx(2240)  H323    Found capability: G.711-uLaw-64k <1>

2:44.817          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.711-uLaw-64k{hw}"

2:44.817          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.711-uLaw-64k{sw}"

2:44.817          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: G.711-uLaw-64k <6>

2:44.817          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.723.1"

2:44.818          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: G.723.1A <7>

2:44.818          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.723.1(5.3k){hw}"

2:44.818          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.723.1A(5.3k){hw}"

2:44.819          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.723.1A(6.3k)-Cisco{hw}"

2:44.819          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.723.1A(6.3k){hw}"

2:44.819          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.723.1{hw}"

2:44.819          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.728{hw}"

2:44.820          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.729"

2:44.820          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: G.729 <8>

2:44.820          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.729A"

2:44.820          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: G.729A <9>

2:44.821          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.729A/B{hw}"

2:44.821          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.729A{hw}"

2:44.821          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.729B{hw}"

2:44.821          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G.729{hw}"

2:44.821          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "G726r32"

2:44.822          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: G726r32 <10>

2:44.822          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "GSM-06.10"

2:44.823          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: GSM-06.10 <11>

2:44.823          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "GSM-06.10{hw}"

2:44.823          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "UserInput/RtpDtmfRelay"

2:44.824          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: UserInput/hookflash <12>

2:44.824          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: UserInput/basicString <13>

2:44.824          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: UserInput/dtmf <14>

2:44.824          H225 Answer:90f4718     h323caps.cxx(2143)  H323    Added capability: UserInput/RFC2833 <15>

2:44.825          H225 Answer:90f4718     h323caps.cxx(2282)  H323    FindCapability: receiveRTPAudioTelephonyEventCapability

2:44.825          H225 Answer:90f4718     h323caps.cxx(2533)  H323    FindCapability: UserInput subtype=10000

2:44.825          H225 Answer:90f4718     h323caps.cxx(2539)  H323    Found capability: UserInput/RFC2833 <3>

2:44.825          H225 Answer:90f4718     h323caps.cxx(2282)  H323    FindCapability: receiveAndTransmitDataApplicationCapability

2:44.826          H225 Answer:90f4718     h323caps.cxx(2514)  H323    FindCapability: Data nonStandard

2:44.826          H225 Answer:90f4718     h323caps.cxx(2282)  H323    FindCapability: receiveUserInputCapability

2:44.826          H225 Answer:90f4718     h323caps.cxx(2533)  H323    FindCapability: UserInput subtype=5

2:44.826          H225 Answer:90f4718     h323caps.cxx(2539)  H323    Found capability: UserInput/hookflash <2>

2:44.826          H225 Answer:90f4718     h323caps.cxx(2282)  H323    FindCapability: receiveUserInputCapability

2:44.826          H225 Answer:90f4718     h323caps.cxx(2533)  H323    FindCapability: UserInput subtype=4

2:44.827          H225 Answer:90f4718     h323caps.cxx(2539)  H323    Found capability: UserInput/dtmf <14>

2:44.827          H225 Answer:90f4718     h323caps.cxx(2282)  H323    FindCapability: receiveUserInputCapability

2:44.827          H225 Answer:90f4718     h323caps.cxx(2533)  H323    FindCapability: UserInput subtype=1

2:44.827          H225 Answer:90f4718     h323caps.cxx(2539)  H323    Found capability: UserInput/basicString <13>

2:44.827          H225 Answer:90f4718     h323caps.cxx(2282)  H323    FindCapability: receiveAudioCapability

2:44.827          H225 Answer:90f4718     h323caps.cxx(2533)  H323    FindCapability: Audio subtype=3

2:44.827          H225 Answer:90f4718     h323caps.cxx(2539)  H323    Found capability: G.711-uLaw-64k <1>

2:44.828          H225 Answer:90f4718     h323caps.cxx(997)   H323    Capability tx frames left at 20 as remote allows 20

2:44.828          H225 Answer:90f4718     h323caps.cxx(2267)  H323    FindCapability: UserInput/RFC2833 <30>

2:44.829          H225 Answer:90f4718     h323caps.cxx(2153)  H323    Added capability: UserInput/RFC2833 <30>

2:44.829          H225 Answer:90f4718     h323caps.cxx(2267)  H323    FindCapability: UserInput/hookflash <26>

2:44.829          H225 Answer:90f4718     h323caps.cxx(2153)  H323    Added capability: UserInput/hookflash <26>

2:44.829          H225 Answer:90f4718     h323caps.cxx(2267)  H323    FindCapability: UserInput/dtmf <25>

2:44.829          H225 Answer:90f4718     h323caps.cxx(2153)  H323    Added capability: UserInput/dtmf <25>

2:44.829          H225 Answer:90f4718     h323caps.cxx(2267)  H323    FindCapability: UserInput/basicString <22>

2:44.830          H225 Answer:90f4718     h323caps.cxx(2153)  H323    Added capability: UserInput/basicString <22>

2:44.830          H225 Answer:90f4718     h323caps.cxx(2267)  H323    FindCapability: G.711-uLaw-64k <1>

2:44.830          H225 Answer:90f4718     h323caps.cxx(2153)  H323    Added capability: G.711-uLaw-64k <1>

2:44.830          H225 Answer:90f4718     h323caps.cxx(2215)  H323    FindCapability: 1

2:44.830          H225 Answer:90f4718     h323caps.cxx(2219)  H323    Found capability: G.711-uLaw-64k <1>

2:44.831          H225 Answer:90f4718     h323caps.cxx(2215)  H323    FindCapability: 30

2:44.831          H225 Answer:90f4718     h323caps.cxx(2219)  H323    Found capability: UserInput/RFC2833 <30>

2:44.831          H225 Answer:90f4718     h323caps.cxx(2215)  H323    FindCapability: 25

2:44.831          H225 Answer:90f4718     h323caps.cxx(2219)  H323    Found capability: UserInput/dtmf <25>

2:44.831          H225 Answer:90f4718     h323caps.cxx(2215)  H323    FindCapability: 22

2:44.831          H225 Answer:90f4718     h323caps.cxx(2219)  H323    Found capability: UserInput/basicString <22>

2:44.831          H225 Answer:90f4718     h323caps.cxx(2215)  H323    FindCapability: 26

2:44.832          H225 Answer:90f4718     h323caps.cxx(2219)  H323    Found capability: UserInput/hookflash <26>

2:44.832          H225 Answer:90f4718     h323caps.cxx(2628)  H245    Capability merge result:

Table:

UserInput/RFC2833 <30>

UserInput/hookflash <26>

UserInput/dtmf <25>

UserInput/basicString <22>

G.711-uLaw-64k <1>

Set:

0:

0:

G.711-uLaw-64k <1>

1:

UserInput/RFC2833 <30>

UserInput/dtmf <25>

UserInput/basicString <22>

2:

UserInput/hookflash <26>

2:44.832          H225 Answer:90f4718     h323caps.cxx(2630)  H245    Received capability set, is accepted

2:44.832          H225 Answer:90f4718      h323neg.cxx(546)   H245    TerminalCapabilitySet already in progress: outSeq=1

2:44.832          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "UserInput/RFC2833"

2:44.832          H225 Answer:90f4718     h323caps.cxx(2240)  H323    Found capability: UserInput/RFC2833 <30>

2:44.833          H225 Answer:90f4718         h323.cxx(4257)  H323    User Input RFC2833 payload type set to [pt=101]

Peer capability is UserInput/RFC2833 <30>

-- Outbound RFC2833 on payload [pt=101]

Peer capability is UserInput/hookflash <26>

Peer capability is UserInput/dtmf <25>

Peer capability is UserInput/basicString <22>

Peer capability is G.711-uLaw-64k <1>

Found peer capability G.711-uLaw-64k <1>, Asterisk code is 4, frame size (in ms) is 20

Peer capabilities = 0x4 (ulaw), ordered list is (ulaw)

2:44.834          H225 Answer:90f4718      h323pdu.cxx(553)   H245    Sending PDU [(noaddr)/(noaddr)] :

response terminalCapabilitySetAck {

sequenceNumber = 1

}

Raw PDU:

size=3 pos=7fffffff.0 {

000 21 80 01                                          !

}

2:44.835          H225 Answer:90f4718      h323pdu.cxx(553)   H225    Sending PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = destination

messageType = Facility

IE: Facility = {

}

IE: User-User = {

28 10 01 00 04 c0 01 80  05 01 03 21 80 01         (..........!..

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = empty <<null>>

h245Tunneling = TRUE

h245Control = 1 entries {

[0]= 3 octets {

21 80 01                                           !..

}

}

}

}

}

Raw PDU:

08 02 80 2e 62 1c 00 7e  00 0f 05 28 10 01 00 04   ....b..~...(....

c0 01 80 05 01 03 21 80  01                        ......!..

2:44.835          H225 Answer:90f4718         h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=HasExecutedSignalConnect fastStartState=FastStartDisabled

2:44.836          H225 Answer:90f4718         h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=HasExecutedSignalConnect fastStartState=FastStartDisabled

2:44.839          H225 Answer:90f4718      h323pdu.cxx(553)   H225    Receiving PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = originator

messageType = Facility

IE: Facility = {

}

IE: Display = {

63 69 73 63 6f 20 53 79  73 74 65 6d 73 2c 20 49   cisco Systems, I

6e 63 2e                                           nc.

}

IE: User-User = {

26 90 06 00 08 91 4a 00  04 6e 94 64 b4 15 ee 11   &.....J..n.d....

cc 80 b0 b3 29 5d 92 df  6c 86 01 00 1f 01 80 11   ....)]..l.......

00 6e 95 00 dc 15 ee 11  cc 80 b2 b3 29 5d 92 df   .n..........)]..

6c 01 00 01 00 10 c0 01  80 08 01 06 01 00 3c 40   l.............<@

17 54                                              .T

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = facility {

protocolIdentifier = 0.0.8.2250.0.4

conferenceID =  16 octets {

6e 94 64 b4 15 ee 11 cc  80 b0 b3 29 5d 92 df 6c   n.d........)]..l

}

reason = transportedInformation <<null>>

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

multipleCalls = FALSE

maintainConnection = FALSE

}

h245Tunneling = TRUE

h245Control = 1 entries {

[0]= 6 octets {

01 00 3c 40 17 54                                  ..<@.T

}

}

}

}

}

Raw PDU:

08 02 00 2e 62 1c 00 28  13 63 69 73 63 6f 20 53   ....b..(.cisco S

79 73 74 65 6d 73 2c 20  49 6e 63 2e 7e 00 43 05   ystems, Inc.~.C.

26 90 06 00 08 91 4a 00  04 6e 94 64 b4 15 ee 11   &.....J..n.d....

cc 80 b0 b3 29 5d 92 df  6c 86 01 00 1f 01 80 11   ....)]..l.......

00 6e 95 00 dc 15 ee 11  cc 80 b2 b3 29 5d 92 df   .n..........)]..

6c 01 00 01 00 10 c0 01  80 08 01 06 01 00 3c 40   l.............<@

17 54                                              .T

2:44.840          H225 Answer:90f4718         h323.cxx(1876)  H225    Handling PDU: Facility callRef=46

-- Received Facility message...

2:44.840          H225 Answer:90f4718         h323.cxx(2122)  H225    Set protocol version to 4

2:44.841          H225 Answer:90f4718      h323pdu.cxx(553)   H245    Receiving PDU [(noaddr)/(noaddr)] :

request masterSlaveDetermination {

terminalType = 60

statusDeterminationNumber = 5972

}

Raw PDU:

size=6 pos=6.0 {

000 01 00 3c 40 17 54                                   <@ T

}

2:44.841          H225 Answer:90f4718      h323neg.cxx(355)   H245    Received MasterSlaveDetermination: state=Outgoing

2:44.842          H225 Answer:90f4718      h323neg.cxx(386)   H245    MasterSlaveDetermination: local is slave

2:44.842          H225 Answer:90f4718      h323pdu.cxx(553)   H245    Sending PDU [(noaddr)/(noaddr)] :

response masterSlaveDeterminationAck {

decision = master <<null>>

}

Raw PDU:

size=2 pos=7fffffff.0 {

000 20 80

}

2:44.843          H225 Answer:90f4718      h323pdu.cxx(553)   H225    Sending PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = destination

messageType = Facility

IE: Facility = {

}

IE: User-User = {

28 10 01 00 04 c0 01 80  04 01 02 20 80            (.......... .

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = empty <<null>>

h245Tunneling = TRUE

h245Control = 1 entries {

[0]= 2 octets {

20 80                                               .

}

}

}

}

}

Raw PDU:

08 02 80 2e 62 1c 00 7e  00 0e 05 28 10 01 00 04   ....b..~...(....

c0 01 80 04 01 02 20 80                            ...... .

2:44.844          H225 Answer:90f4718         h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=HasExecutedSignalConnect fastStartState=FastStartDisabled

2:44.845          H225 Answer:90f4718         h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=HasExecutedSignalConnect fastStartState=FastStartDisabled

2:44.854          H225 Answer:90f4718      h323pdu.cxx(553)   H225    Receiving PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = originator

messageType = Facility

IE: Facility = {

}

IE: Display = {

63 69 73 63 6f 20 53 79  73 74 65 6d 73 2c 20 49   cisco Systems, I

6e 63 2e                                           nc.

}

IE: User-User = {

26 90 06 00 08 91 4a 00  04 6e 94 64 b4 15 ee 11   &.....J..n.d....

cc 80 b0 b3 29 5d 92 df  6c 86 01 00 1f 01 80 11   ....)]..l.......

00 6e 95 00 dc 15 ee 11  cc 80 b2 b3 29 5d 92 df   .n..........)]..

6c 01 00 01 00 10 c0 01  80 05 01 03 21 80 01      l...........!..

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = facility {

protocolIdentifier = 0.0.8.2250.0.4

conferenceID =  16 octets {

6e 94 64 b4 15 ee 11 cc  80 b0 b3 29 5d 92 df 6c   n.d........)]..l

}

reason = transportedInformation <<null>>

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

multipleCalls = FALSE

maintainConnection = FALSE

}

h245Tunneling = TRUE

h245Control = 1 entries {

[0]= 3 octets {

21 80 01                                           !..

}

}

}

}

}

Raw PDU:

08 02 00 2e 62 1c 00 28  13 63 69 73 63 6f 20 53   ....b..(.cisco S

79 73 74 65 6d 73 2c 20  49 6e 63 2e 7e 00 40 05   ystems, Inc.~.@.

26 90 06 00 08 91 4a 00  04 6e 94 64 b4 15 ee 11   &.....J..n.d....

cc 80 b0 b3 29 5d 92 df  6c 86 01 00 1f 01 80 11   ....)]..l.......

00 6e 95 00 dc 15 ee 11  cc 80 b2 b3 29 5d 92 df   .n..........)]..

6c 01 00 01 00 10 c0 01  80 05 01 03 21 80 01      l...........!..

2:44.855          H225 Answer:90f4718         h323.cxx(1876)  H225    Handling PDU: Facility callRef=46

-- Received Facility message...

2:44.856          H225 Answer:90f4718         h323.cxx(2122)  H225    Set protocol version to 4

2:44.856          H225 Answer:90f4718      h323pdu.cxx(553)   H245    Receiving PDU [(noaddr)/(noaddr)] :

response terminalCapabilitySetAck {

sequenceNumber = 1

}

Raw PDU:

size=3 pos=3.0 {

000 21 80 01                                          !

}

2:44.857          H225 Answer:90f4718      h323neg.cxx(633)   H245    Received TerminalCapabilitySetAck: state=InProgress pduSeq=1 outSeq=1

2:44.857          H225 Answer:90f4718      h323neg.cxx(643)   H245    TerminalCapabilitySet Sent.

2:44.857          H225 Answer:90f4718         h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=HasExecutedSignalConnect fastStartState=FastStartDisabled

2:44.857          H225 Answer:90f4718         h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=HasExecutedSignalConnect fastStartState=FastStartDisabled

2:44.877          H225 Answer:90f4718      h323pdu.cxx(553)   H225    Receiving PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = originator

messageType = Facility

IE: Facility = {

}

IE: Display = {

63 69 73 63 6f 20 53 79  73 74 65 6d 73 2c 20 49   cisco Systems, I

6e 63 2e                                           nc.

}

IE: User-User = {

26 90 06 00 08 91 4a 00  04 6e 94 64 b4 15 ee 11   &.....J..n.d....

cc 80 b0 b3 29 5d 92 df  6c 86 01 00 1f 01 80 11   ....)]..l.......

00 6e 95 00 dc 15 ee 11  cc 80 b2 b3 29 5d 92 df   .n..........)]..

6c 01 00 01 00 10 c0 01  80 04 01 02 20 a0         l........... .

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = facility {

protocolIdentifier = 0.0.8.2250.0.4

conferenceID =  16 octets {

6e 94 64 b4 15 ee 11 cc  80 b0 b3 29 5d 92 df 6c   n.d........)]..l

}

reason = transportedInformation <<null>>

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

multipleCalls = FALSE

maintainConnection = FALSE

}

h245Tunneling = TRUE

h245Control = 1 entries {

[0]= 2 octets {

20 a0                                               .

}

}

}

}

}

Raw PDU:

08 02 00 2e 62 1c 00 28  13 63 69 73 63 6f 20 53   ....b..(.cisco S

79 73 74 65 6d 73 2c 20  49 6e 63 2e 7e 00 3f 05   ystems, Inc.~.?.

26 90 06 00 08 91 4a 00  04 6e 94 64 b4 15 ee 11   &.....J..n.d....

cc 80 b0 b3 29 5d 92 df  6c 86 01 00 1f 01 80 11   ....)]..l.......

00 6e 95 00 dc 15 ee 11  cc 80 b2 b3 29 5d 92 df   .n..........)]..

6c 01 00 01 00 10 c0 01  80 04 01 02 20 a0         l........... .

2:44.878          H225 Answer:90f4718         h323.cxx(1876)  H225    Handling PDU: Facility callRef=46

-- Received Facility message...

2:44.878          H225 Answer:90f4718         h323.cxx(2122)  H225    Set protocol version to 4

2:44.879          H225 Answer:90f4718      h323pdu.cxx(553)   H245    Receiving PDU [(noaddr)/(noaddr)] :

response masterSlaveDeterminationAck {

decision = slave <<null>>

}

Raw PDU:

size=2 pos=1.3 {

000 20 a0

}

2:44.879          H225 Answer:90f4718      h323neg.cxx(414)   H245    Received MasterSlaveDeterminationAck: state=Incoming

2:44.880          H225 Answer:90f4718         h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=HasExecutedSignalConnect fastStartState=FastStartDisabled

2:44.880          H225 Answer:90f4718     h323caps.cxx(2231)  H323    FindCapability: "T.120"

=-= In OnConnectionEstablished for call 46

-- Connection Established with "172.16.4.13"

2:44.880          H225 Answer:90f4718         h323.cxx(4374)  H323    InternalEstablishedConnectionCheck: connectionState=EstablishedConnection fastStartState=FastStartDisabled

-- Hungup 'Zap/1-1'

== Spawn extension (n_DstGW, 4045266062, 4) exited non-zero on 'H323/ip$172.16.4.13:19888/46'

2:51.278      ThreadID=0xf5eacbb0             h323.cxx(1629)  H323    Call end reason for ip$172.16.4.13:19888/46 set to EndedByRemoteUser

2:51.278      ThreadID=0xf5eacbb0             h323.cxx(1647)  H225    Sending release complete PDU: callRef=46

-- Sending RELEASE COMPLETE

2:51.279      ThreadID=0xf5eacbb0          h323pdu.cxx(553)   H245    Sending PDU [(noaddr)/(noaddr)] :

command endSessionCommand disconnect <<null>>

Raw PDU:

size=2 pos=7fffffff.0 {

000 4a 40                                             J@

}

2:51.280      ThreadID=0xf5eacbb0          h323pdu.cxx(553)   H225    Sending PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = destination

messageType = ReleaseComplete

IE: Cause - Normal call clearing = {

80 90                                              ..

}

IE: User-User = {

25 80 06 00 08 91 4a 00  04 01 11 00 6e 95 00 dc   %.....J.....n...

15 ee 11 cc 80 b2 b3 29  5d 92 df 6c 04 c0 01 80   .......)]..l....

04 01 02 4a 40                                     ...J@

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = releaseComplete {

protocolIdentifier = 0.0.8.2250.0.4

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

}

h245Tunneling = TRUE

h245Control = 1 entries {

[0]= 2 octets {

4a 40                                              J@

}

}

}

}

}

Raw PDU:

08 02 80 2e 5a 08 02 80  90 7e 00 26 05 25 80 06   ....Z....~.&.%..

00 08 91 4a 00 04 01 11  00 6e 95 00 dc 15 ee 11   ...J.....n......

cc 80 b2 b3 29 5d 92 df  6c 04 c0 01 80 04 01 02   ....)]..l.......

4a 40                                              J@

-- ClearCall: Request to clear call with token ip$172.16.4.13:19888/46, cause EndedByRemoteUser

2:51.281      ThreadID=0xf5eacbb0           h323ep.cxx(2119)  H323    Clearing connection ip$172.16.4.13:19888/46 reason=EndedByRemoteUser

2:51.284                 H323 Cleaner       h323ep.cxx(2176)  H323    Cleaning up connections

2:51.284                 H323 Cleaner         h323.cxx(1689)  H323    Connection ip$172.16.4.13:19888/46 closing: connectionState=EstablishedConnection

2:51.284                 H323 Cleaner      h323neg.cxx(340)   H245    Stopping MasterSlaveDetermination: state=Idle

2:51.284                 H323 Cleaner      h323neg.cxx(572)   H245    Stopping TerminalCapabilitySet: state=Sent

2:51.285                 H323 Cleaner     channels.cxx(685)   LogChan Cleaning up R-1

channelsOpen = 1

2:51.285                 H323 Cleaner       h323ep.cxx(2509)  H323    Stopped receiving logical channel: G.711-uLaw-64k <1>

2:51.285                 H323 Cleaner     channels.cxx(713)   LogChan Cleaned up R-1

2:51.285                 H323 Cleaner     channels.cxx(685)   LogChan Cleaning up T-101

channelsOpen = 0

2:51.285                 H323 Cleaner       h323ep.cxx(2509)  H323    Stopped sending logical channel: G.711-uLaw-64k <1>

2:51.285                 H323 Cleaner     channels.cxx(713)   LogChan Cleaned up T-101

ExternalRTPChannel Destroyed

2:51.289                 H323 Cleaner         h323.cxx(4932)  H323    Bandwidth request: -0.0kb/s, available: 128.0kb/s

ExternalRTPChannel Destroyed

2:51.291                 H323 Cleaner         h323.cxx(4932)  H323    Bandwidth request: -0.0kb/s, available: 128.0kb/s

2:51.291                 H323 Cleaner         h323.cxx(1732)  H323    Awaiting end session from remote for 9.988 seconds

2:51.317          H225 Answer:90f4718      h323pdu.cxx(553)   H225    Receiving PDU [ip$172.16.4.56:1720/ip$172.16.4.13:19888] :

{

q931pdu = {

protocolDiscriminator = 8

callReference = 46

from = originator

messageType = ReleaseComplete

IE: Cause - Normal call clearing = {

80 90                                              ..

}

IE: User-User = {

25 80 06 00 08 91 4a 00  04 11 00 11 00 6e 95 00   %.....J......n..

dc 15 ee 11 cc 80 b2 b3  29 5d 92 df 6c 10 80 01   ........)]..l...

80                                                 .

}

}

h225pdu = {

h323_uu_pdu = {

h323_message_body = releaseComplete {

protocolIdentifier = 0.0.8.2250.0.4

callIdentifier = {

guid =  16 octets {

6e 95 00 dc 15 ee 11 cc  80 b2 b3 29 5d 92 df 6c   n..........)]..l

}

}

}

h245Tunneling = TRUE

}

}

}

Raw PDU:

08 02 00 2e 5a 08 02 80  90 7e 00 22 05 25 80 06   ....Z....~.".%..

00 08 91 4a 00 04 11 00  11 00 6e 95 00 dc 15 ee   ...J......n.....

11 cc 80 b2 b3 29 5d 92  df 6c 10 80 01 80         .....)]..l....

2:51.318          H225 Answer:90f4718         h323.cxx(1876)  H225    Handling PDU: ReleaseComplete callRef=46

-- ClearCall: Request to clear call with token ip$172.16.4.13:19888/46, cause EndedByTransportFail

2:51.318          H225 Answer:90f4718       h323ep.cxx(2119)  H323    Clearing connection ip$172.16.4.13:19888/46 reason=EndedByTransportFail

2:51.319          H225 Answer:90f4718         h323.cxx(1866)  H225    Signal channel closed.

2:51.321                 H323 Cleaner   transports.cxx(1143)  H323    H323Transport::Close

2:51.323                 H323 Cleaner   transports.cxx(1240)  H323    H323Transport::CleanUpOnTermination for H225 Answer:90f4718

2:51.324                 H323 Cleaner         h323.cxx(1757)  H323    Connection ip$172.16.4.13:19888/46 terminated.

-- 172.16.4.13 has cleared the call

== H.323 Connection deleted.

2:51.325                 H323 Cleaner         h323.cxx(1579)  H323    Connection ip$172.16.4.13:19888/46 deleted.

2:51.326                 H323 Cleaner       h323ep.cxx(2176)  H323    Cleaning up connections