M. Spencer

Internet-Draft                                              Digium, Inc.

Expires: April 26, 2007                                       B. Capouch

Saint Joseph's College

E. Guy, (Editor)

E-MC Software

F. Miller

Cornfed Systems, Inc.

K. Shumard

October 23, 2006

IAX2: Inter-Asterisk eXchange Version 2

draft-guy-iax-02

Status of this Memo

By submitting this Internet-Draft, each author represents that any

applicable patent or other IPR claims of which he or she is aware

have been or will be disclosed, and any of which he or she becomes

aware will be disclosed, in accordance with Section 6 of BCP 79.

This document may not be modified, and derivative works of it may not

be created.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF), its areas, and its working groups.  Note that

other groups may also distribute working documents as Internet-

Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time.  It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

The list of current Internet-Drafts can be accessed at

http://www.ietf.org/ietf/1id-abstracts.txt.

The list of Internet-Draft Shadow Directories can be accessed at

http://www.ietf.org/shadow.html.

This Internet-Draft will expire on April 26, 2007.

Copyright Notice

Copyright (C) The Internet Society (2006).

Abstract

Spencer, et al.          Expires April 26, 2007                 [Page 1]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

This document describes the Inter-Asterisk eXchange protocol, Version

2, (IAX2) an application-layer control and media protocol for

creating, modifying, and terminating multimedia sessions over

Internet Protocol (IP) networks.  IAX2 is targeted primarily at the

control of Voice over Internet Protocol (VoIP) calls, but can be used

with streaming video or any other type of multimedia.

IAX2 is an "all in one" protocol for handling multimedia in Internet

Protocol (IP) networks.  It combines both control and media services

in the same protocol.  IAX2's compact encoding decreases bandwidth

usage and since IAX2 uses a single, static, UDP port, it facilitates

native support for Network Address Translation (NAT) transparency.

IAX2 is well suited for Internet telephony service, but, is open

enough to support additional services.

Table of Contents

1.  Introduction  . . . . . . . . . . . . . . . . . . . . . . . .   6

2.  IAX2 Terminology  . . . . . . . . . . . . . . . . . . . . . .   7

3.  Overview of IAX2 Protocol . . . . . . . . . . . . . . . . . .   8

4.  Naming Conventions  . . . . . . . . . . . . . . . . . . . . .  10

5.  IAX2 Uniform Resource Identifiers . . . . . . . . . . . . . .  11

5.1.  IAX2 URI Scheme Registration  . . . . . . . . . . . . . .  11

5.2.  URI Comparison  . . . . . . . . . . . . . . . . . . . . .  13

6.  Peer Behavior and Messages  . . . . . . . . . . . . . . . . .  15

6.1.  Registration (OPTIONAL) . . . . . . . . . . . . . . . . .  15

6.1.1.  Overview  . . . . . . . . . . . . . . . . . . . . . .  15

6.1.2.  REGREQ Registration Request Message . . . . . . . . .  18

6.1.3.  REGAUTH Registration Authentication Response

Message . . . . . . . . . . . . . . . . . . . . . . .  19

6.1.4.  REGACK Registration Acknowledgment Message  . . . . .  19

6.1.5.  REGREJ Registration Rejection Message . . . . . . . .  20

6.1.6.  REGREL Registration Release Request Message . . . . .  21

6.2.  Call Leg Management . . . . . . . . . . . . . . . . . . .  22

6.2.1.  Overview  . . . . . . . . . . . . . . . . . . . . . .  23

6.2.2.  NEW Request Message . . . . . . . . . . . . . . . . .  24

6.2.3.  ACCEPT Response Message . . . . . . . . . . . . . . .  26

6.2.4.  REJECT Response Message . . . . . . . . . . . . . . .  26

6.2.5.  HANGUP Request Message  . . . . . . . . . . . . . . .  27

6.2.6.  AUTHREP Authentication Reply Message  . . . . . . . .  27

6.2.7.  AUTHREQ Authentication Request Message  . . . . . . .  28

6.3.  Call Control  . . . . . . . . . . . . . . . . . . . . . .  28

6.3.1.  Overview  . . . . . . . . . . . . . . . . . . . . . .  28

6.3.2.  PROCEEDING Response Message . . . . . . . . . . . . .  29

6.3.3.  RINGING Response Message  . . . . . . . . . . . . . .  29

6.3.4.  ANSWER Response Message . . . . . . . . . . . . . . .  30

6.4.  Mid-Call Link Operations  . . . . . . . . . . . . . . . .  30

Spencer, et al.          Expires April 26, 2007                 [Page 2]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.4.1.  FLASH Request Message . . . . . . . . . . . . . . . .  30

6.4.2.  HOLD Request Message  . . . . . . . . . . . . . . . .  30

6.4.3.  UNHOLD Request Message  . . . . . . . . . . . . . . .  30

6.4.4.  QUELCH Request Message  . . . . . . . . . . . . . . .  31

6.4.5.  UNQUELCH Request Message  . . . . . . . . . . . . . .  31

6.4.6.  TRANSFER Request Message  . . . . . . . . . . . . . .  31

6.5.  Call Path Optimization  . . . . . . . . . . . . . . . . .  32

6.5.1.  TXREQ Transfer Request Message  . . . . . . . . . . .  34

6.5.2.  TXCNT Transfer Connectivity Response Message  . . . .  35

6.5.3.  TXACC Response Message  . . . . . . . . . . . . . . .  35

6.5.4.  TXREADY Transfer Ready Response Message . . . . . . .  35

6.5.5.  TXREL Transfer Release Response Message . . . . . . .  36

6.5.6.  TXMEDIA Transfer Media Message  . . . . . . . . . . .  36

6.5.7.  TXREJ Transfer Rejection Response Message . . . . . .  37

6.6.  Call Tear Down  . . . . . . . . . . . . . . . . . . . . .  37

6.7.  Network Monitoring  . . . . . . . . . . . . . . . . . . .  37

6.7.1.  POKE Request Message  . . . . . . . . . . . . . . . .  37

6.7.2.  PING Request Message  . . . . . . . . . . . . . . . .  38

6.7.3.  PONG Response Message . . . . . . . . . . . . . . . .  38

6.7.4.  LAGRQ Lag Request Message . . . . . . . . . . . . . .  38

6.7.5.  LAGRP Lag Response Message  . . . . . . . . . . . . .  38

6.8.  Digit Dialing . . . . . . . . . . . . . . . . . . . . . .  38

6.8.1.  DPREQ Dial Plan Request Message . . . . . . . . . . .  39

6.8.2.  DPREP Dial Plan Response Message  . . . . . . . . . .  39

6.8.3.  DIAL Request Message  . . . . . . . . . . . . . . . .  40

6.9.  Firmware Download . . . . . . . . . . . . . . . . . . . .  40

6.9.1.  FWDOWNL Firmware Download Request Message . . . . . .  41

6.9.2.  FWDATA Firmware Data Response Message . . . . . . . .  41

6.10. Provisioning  . . . . . . . . . . . . . . . . . . . . . .  42

6.10.1. PROVISION Request Message . . . . . . . . . . . . . .  42

6.11. Miscellaneous . . . . . . . . . . . . . . . . . . . . . .  42

6.11.1. ACK acknowledgement Message . . . . . . . . . . . . .  42

6.11.2. INVAL Invalid Response Message  . . . . . . . . . . .  43

6.11.3. VNAK Voice Negative Acknowledgement Message . . . . .  43

6.11.4. MWI Message Waiting Indicator Request Message . . . .  43

6.11.5. UNSUPPORT Unsupported Response Message  . . . . . . .  44

6.12. Media Messages  . . . . . . . . . . . . . . . . . . . . .  44

6.12.1. DTMF Media Message  . . . . . . . . . . . . . . . . .  44

6.12.2. Voice Media Message . . . . . . . . . . . . . . . . .  44

6.12.3. Video Media Message . . . . . . . . . . . . . . . . .  45

6.12.4. Text Media Message  . . . . . . . . . . . . . . . . .  45

6.12.5. Image Media Message . . . . . . . . . . . . . . . . .  45

6.12.6. HTML Media Message  . . . . . . . . . . . . . . . . .  45

6.12.7. Comfort Noise  Media Message  . . . . . . . . . . . .  45

7.  Message Transport . . . . . . . . . . . . . . . . . . . . . .  46

7.1.  Trunking  . . . . . . . . . . . . . . . . . . . . . . . .  46

7.2.  Timers  . . . . . . . . . . . . . . . . . . . . . . . . .  47

7.2.1.  Retransmission Timer  . . . . . . . . . . . . . . . .  47

Spencer, et al.          Expires April 26, 2007                 [Page 3]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

7.2.2.  Registration Period Timer . . . . . . . . . . . . . .  47

7.3.  NAT Considerations  . . . . . . . . . . . . . . . . . . .  47

7.4.  Encryption  . . . . . . . . . . . . . . . . . . . . . . .  48

8.  Message Encoding  . . . . . . . . . . . . . . . . . . . . . .  49

8.1.  Full Frames . . . . . . . . . . . . . . . . . . . . . . .  49

8.2.  Mini frames . . . . . . . . . . . . . . . . . . . . . . .  51

8.3.  Meta frames . . . . . . . . . . . . . . . . . . . . . . .  52

8.3.1.  Meta Video Frames . . . . . . . . . . . . . . . . . .  52

8.3.2.  Meta Trunk Frames . . . . . . . . . . . . . . . . . .  54

8.4.  Frame Types . . . . . . . . . . . . . . . . . . . . . . .  56

8.4.1.  DTMF Frame  . . . . . . . . . . . . . . . . . . . . .  56

8.4.2.  Voice Frame . . . . . . . . . . . . . . . . . . . . .  57

8.4.3.  Video Frame . . . . . . . . . . . . . . . . . . . . .  57

8.4.4.  Control Frame . . . . . . . . . . . . . . . . . . . .  57

8.4.5.  Null Frame  . . . . . . . . . . . . . . . . . . . . .  57

8.4.6.  IAX2 Frame  . . . . . . . . . . . . . . . . . . . . .  57

8.4.7.  Text Frame  . . . . . . . . . . . . . . . . . . . . .  57

8.4.8.  Image Frame . . . . . . . . . . . . . . . . . . . . .  57

8.4.9.  HTML Frame  . . . . . . . . . . . . . . . . . . . . .  58

8.4.10. Comfort Noise Frame . . . . . . . . . . . . . . . . .  58

8.4.11. Control Frames  . . . . . . . . . . . . . . . . . . .  59

8.4.12. IAX2 Frames . . . . . . . . . . . . . . . . . . . . .  60

8.5.  HTML Command Subclasses . . . . . . . . . . . . . . . . .  62

8.6.  Information Elements  . . . . . . . . . . . . . . . . . .  62

8.6.1.  CALLED NUMBER . . . . . . . . . . . . . . . . . . . .  65

8.6.2.  CALLING NUMBER  . . . . . . . . . . . . . . . . . . .  66

8.6.3.  CALLING ANI . . . . . . . . . . . . . . . . . . . . .  66

8.6.4.  CALLING NAME  . . . . . . . . . . . . . . . . . . . .  67

8.6.5.  CALLED CONTEXT  . . . . . . . . . . . . . . . . . . .  67

8.6.6.  USERNAME  . . . . . . . . . . . . . . . . . . . . . .  68

8.6.7.  CAPABILITY  . . . . . . . . . . . . . . . . . . . . .  68

8.6.8.  FORMAT  . . . . . . . . . . . . . . . . . . . . . . .  68

8.6.9.  LANGUAGE  . . . . . . . . . . . . . . . . . . . . . .  69

8.6.10. VERSION . . . . . . . . . . . . . . . . . . . . . . .  69

8.6.11. ADSICPE . . . . . . . . . . . . . . . . . . . . . . .  70

8.6.12. DNID  . . . . . . . . . . . . . . . . . . . . . . . .  70

8.6.13. AUTHMETHODS . . . . . . . . . . . . . . . . . . . . .  71

8.6.14. CHALLENGE . . . . . . . . . . . . . . . . . . . . . .  71

8.6.15. MD5 RESULT  . . . . . . . . . . . . . . . . . . . . .  72

8.6.16. RSA RESULT  . . . . . . . . . . . . . . . . . . . . .  72

8.6.17. APPARENT ADDR . . . . . . . . . . . . . . . . . . . .  73

8.6.18. REFRESH . . . . . . . . . . . . . . . . . . . . . . .  75

8.6.19. DPSTATUS  . . . . . . . . . . . . . . . . . . . . . .  75

8.6.20. CALLNO  . . . . . . . . . . . . . . . . . . . . . . .  76

8.6.21. CAUSE . . . . . . . . . . . . . . . . . . . . . . . .  76

8.6.22. IAX2 UNKNOWN  . . . . . . . . . . . . . . . . . . . .  77

8.6.23. MSGCOUNT  . . . . . . . . . . . . . . . . . . . . . .  77

8.6.24. AUTOANSWER  . . . . . . . . . . . . . . . . . . . . .  78

Spencer, et al.          Expires April 26, 2007                 [Page 4]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.6.25. MUSICONHOLD . . . . . . . . . . . . . . . . . . . . .  78

8.6.26. TRANSFERID  . . . . . . . . . . . . . . . . . . . . .  79

8.6.27. RDNIS . . . . . . . . . . . . . . . . . . . . . . . .  79

8.6.28. PROVISIONING  . . . . . . . . . . . . . . . . . . . .  80

8.6.29. AESPROVISIONING . . . . . . . . . . . . . . . . . . .  80

8.6.30. DATETIME  . . . . . . . . . . . . . . . . . . . . . .  81

8.6.31. DEVICETYPE  . . . . . . . . . . . . . . . . . . . . .  81

8.6.32. SERVICEIDENT  . . . . . . . . . . . . . . . . . . . .  82

8.6.33. FIRMWAREVER . . . . . . . . . . . . . . . . . . . . .  82

8.6.34. FWBLOCKDESC . . . . . . . . . . . . . . . . . . . . .  83

8.6.35. FWBLOCKDATA . . . . . . . . . . . . . . . . . . . . .  83

8.6.36. PROVVER . . . . . . . . . . . . . . . . . . . . . . .  84

8.6.37. CALLINGPRES . . . . . . . . . . . . . . . . . . . . .  84

8.6.38. CALLINGTON  . . . . . . . . . . . . . . . . . . . . .  85

8.6.39. CALLINGTNS  . . . . . . . . . . . . . . . . . . . . .  86

8.6.40. SAMPLINGRATE  . . . . . . . . . . . . . . . . . . . .  87

8.6.41. CAUSECODE . . . . . . . . . . . . . . . . . . . . . .  87

8.6.42. ENCRYPTION  . . . . . . . . . . . . . . . . . . . . .  90

8.6.43. ENCKEY  . . . . . . . . . . . . . . . . . . . . . . .  91

8.6.44. CODEC PREFS . . . . . . . . . . . . . . . . . . . . .  91

8.6.45. RR JITTER . . . . . . . . . . . . . . . . . . . . . .  92

8.6.46. RR LOSS . . . . . . . . . . . . . . . . . . . . . . .  92

8.6.47. RR PKTS . . . . . . . . . . . . . . . . . . . . . . .  93

8.6.48. RR DELAY  . . . . . . . . . . . . . . . . . . . . . .  93

8.6.49. RR DROPPED  . . . . . . . . . . . . . . . . . . . . .  93

8.6.50. RR OOO  . . . . . . . . . . . . . . . . . . . . . . .  94

8.7.  Media Formats . . . . . . . . . . . . . . . . . . . . . .  95

9.  Example Message Flows . . . . . . . . . . . . . . . . . . . .  96

9.1.  Ping/Pong . . . . . . . . . . . . . . . . . . . . . . . .  96

9.2.  Lagrq/Lagrp . . . . . . . . . . . . . . . . . . . . . . .  96

9.3.  Registration  . . . . . . . . . . . . . . . . . . . . . .  97

9.4.  Registration  . . . . . . . . . . . . . . . . . . . . . .  97

9.5.  Provisioning  . . . . . . . . . . . . . . . . . . . . . .  98

9.6.  Firmware Download . . . . . . . . . . . . . . . . . . . .  98

9.7.  Call Path Optimization  . . . . . . . . . . . . . . . . .  99

9.8.  IAX2 Media Call . . . . . . . . . . . . . . . . . . . . .  99

9.9.  IAX2 Media Call via an IAX2 Device  . . . . . . . . . . . 101

10. Security Considerations . . . . . . . . . . . . . . . . . . . 103

11. IANA Considerations . . . . . . . . . . . . . . . . . . . . . 104

12. Implementation Notes  . . . . . . . . . . . . . . . . . . . . 105

13. Acknowledgments . . . . . . . . . . . . . . . . . . . . . . . 106

14. References  . . . . . . . . . . . . . . . . . . . . . . . . . 106

Authors' Addresses  . . . . . . . . . . . . . . . . . . . . . . . 108

Intellectual Property and Copyright Statements  . . . . . . . . . 110

Spencer, et al.          Expires April 26, 2007                 [Page 5]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

1.  Introduction

Numerous protocols have been specified by the Internet community to

support control or signaling of multimedia sessions.  In general,

these protocols are designed to offer full support for many types of

media transmission.  This flexible approach adds some overhead to the

protocol headers, but allows for the protocol use well beyond the

current application.  Typically, these protocols reference, but do

not specify the media transmission protocol to be used to carry the

actual stream.  This method allows for greater flexibility, but again

leads to more overhead.  Furthermore, multimedia solutions which use

different, perhaps dynamic, network addresses for signaling and media

transmission frequently suffer from Network Address Translation (NAT)

traversal and security challenges.

IAX2 is general enough that it can handle most common types of media

streams.  However, the protocol is highly optimized for VoIP calls

where low overhead and low bandwidth consumption are priorities.

This pragmatic aspect makes IAX2 much more efficient for VoIP than

protocols which consider possibilities far beyond current needs and

specify many more details than are strictly necessary to describe or

transport a point-to-point call.  Furthermore, because IAX2 is

designed to be lightweight and VoIP-friendly, it consumes

comparatively less bandwidth.  Because IAX2 uses the same UDP port

for both its signaling and media messages, and because all

communications regarding the call are done over a the same point-to-

point path, NAT traversal is much simpler for IAX2 than for other

commonly deployed protocols.

Spencer, et al.          Expires April 26, 2007                 [Page 6]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

2.  IAX2 Terminology

The keywords "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT",

"SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this

document are to be interpreted as described in [RFC2119].

Additionally, this document uses the following terminology:

Peer: A host or device that implements the IAX2 protocol.

Call: A call is a relationship between two or more parties (i.e.,

resources such as devices, user agents, or programs) that exists

for some time for the purpose of exchanging real-time media.  In

the context of this document, a call is an end to end relationship

where at least the one leg of call path is implemented using the

IAX2 protocol.

Calling Party: A device or program that initiates a call.

Called Party: A device or program to which a call is directed.

Context: A context is a named partition of a Dialplan.

Dialplan: A Dialplan is a set of rules for associating provided names

and numbers with a particular called party.

Frame: The atomic communication unit between two IAX2 peers.  All

IAX2 messages are carried within frames.

Information Element (IE): A discrete data unit appended to an IAX2

frame which specifies user or call-specific data.

Registrant: A registrant is a peer that makes REGISTER requests in

order to advertise the address of a resource, i.e., a device or

program to which a call may be directed.

Registrar: A registrar is a peer that processes REGISTER requests and

places the information it receives in those requests into the

location service.  [RFC3261].

Spencer, et al.          Expires April 26, 2007                 [Page 7]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

3.  Overview of IAX2 Protocol

IAX2 is a peer-to-peer, VoIP-oriented, protocol.  IAX2 includes both

control and media functions.  It can register locations, create,

modify, terminate multimedia sessions, and carry the actual media

streams specified by the sessions it manages.  The protocol is

designed and optimized for describing and transporting multimedia

calls using Internet Protocol.

The basic design approach for IAX2 multiplexes signaling and multiple

media streams over a single UDP association between two hosts.  This

is accomplished by using the same "well-known" UDP port, 4569, for

all types of IAX2 traffic.  IAX2's unified signaling and media paths

achieve NAT transparency, which is an advantage of IAX2 over

alternative media transport protocols such as SIP.

IAX2 is coded as a binary protocol.  One major benefit of using a

binary protocol is bandwidth efficiency because the quality of voice

calls is frequently related to the amount of bandwidth consumed.

This is one way the protocol is specifically optimized to make

efficient use of bandwidth for individual voice calls.  The bandwidth

efficiency for other stream types is sacrificed for the sake of

individual voice calls.  Other benefits of a binary protocol are

robustness against buffer overrun attacks and compact implementation

capability, which reduces interoperability issues related to parsing.

The atomic communication unit in IAX2 is the "Frame".  There are

multiple classes of Frames, each of which is described below.  In

general, "Full Frames" carry signaling/control data, while "Mini

Frames" carry media stream data.  Full Frames enclose optional

'Information Elements' (IEs).  IEs describe various types of user- or

call-specific data.  "Meta Frames" are used for call trunking or

video stream transmission.

An IAX2-based call may consist of many call legs, or segments.  Each

call leg may be implemented using different protocols, e.g., SIP to

IAX2 to ISDN.  IAX2 is responsible for setting up one or more legs of

a complete call path, not necessarily the end-to-end call.

IAX2 is an optimized peer-to-peer protocol.  If two adjacent call

legs utilize the IAX2 protocol and if the intermediate peer

determines that it does not need to remain in the call path, it can

supervise a calling path change such that it removes itself from the

path.  This supervision is complete, a call path is not changed until

all peers in the optimized call path confirm they can properly

communicate.

IAX2 supports security features by allowing multiple methods of user

Spencer, et al.          Expires April 26, 2007                 [Page 8]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

authentication and authorization, as well as during peer

registration.  IAX2 also specifies a generic framework for native

encryption.

Spencer, et al.          Expires April 26, 2007                 [Page 9]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

4.  Naming Conventions

Call Identifier: A call leg is marked with two unique integers, one

assigned by each peer involved in creating the call leg.

Number: The Calling and Called Numbers are a set of digits and

letters identifying a call originator and the desired terminating

resource.  The term 'Number' is historic and has been expanded to

include letters.  A peer is responsible for defining its own

dialplan.  A peer MAY define its dialplan according to ITU-T

Recommendation E.164.  [E164] However, this is not required.

Username: A username is a string used for identification purposes.

Spencer, et al.          Expires April 26, 2007                [Page 10]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

5.  IAX2 Uniform Resource Identifiers

5.1.  IAX2 URI Scheme Registration

This section registers IAX2 according to the guidelines in [RFC4395].

URI scheme name:

iax2.

Status:

Permanent.

URI scheme syntax:

The "iax2:" scheme follows the guidelines in [RFC3986].

The general form is as follows:

iax2:[username@]host[:port][/number[?context]]

where these tokens have the following meanings:

iax2: The literal 'iax2:'.

username: A string used for identification purposes.

host: The domain of the resource.  The host part contains

either a fully-qualified domain name or numeric IPv4 or IPv6

address.  An IPv6 address must be enclosed within brackets

(i.e., '[2001:db8::1]') as defined in [RFC3986].  Using the

fully-qualified domain name form is RECOMMENDED whenever

possible.

port: The numeric UDP port number.

number: The name or number identifying the resource on that

host.

context: The name of the host partition in which the service is

identified or processed.

Spencer, et al.          Expires April 26, 2007                [Page 11]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

Examples

iax2:atlanta.com/alice

iax2:atlanta.com:4569/alice

iax2:atlanta.com:4570/alice?friends

iax2:192.0.2.4:4569/alice?friends

iax2:[2001:db8::1]:4569/alice?friends

iax2:whitehouse.gov/12022561414

iax2:johnQ@whitehouse.gov/12022561414

ABNF Certain values are included by reference from [RFC3986]:

iax2-uri = "iax2:" [userinfo "@" ]host[":" port][ "/" number[

"?" context]]

userinfo = // as specified in RFC 3986

host = // as specified in RFC 3986

port = // as specified in RFC 3986

number = *(unreserved | sub-delims | pct-encoded )

context = *(unreserved | sub-delims | pct-encoded )

unreserved = // as specified in RFC 3986

sub-delims = // as specified in RFC 3986

pct-encoded = // as specified in RFC 3986

URI Scheme Semantics:

An IAX2 URI identifies a communications resource capable of

communicating using the IAX2 Version 2 protocol defined in this

document.  Within this document, we refer to IAX2 Version 2

protocol URI as IAX2.  An IAX2 URI contains enough information to

initiate an IAX2-based call with that resource.

IAX2 URIs are associated with server resources to which calls may

be routed.  For instance, an IAX2 URI may represent an appearance

on a phone, a voice-mail box on a messaging service, an

interactive program, a PSTN address or gateway, or any group of

the above.

The iax2 uri scheme translates into a location that may be used by

the iax2 protocol to establish a new call using the uri scheme

components described in the previous section.  This new call

function is the only defined operation.

Spencer, et al.          Expires April 26, 2007                [Page 12]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

Encoding considerations:

IAX2 URI scheme encoding conforms to the encoding rules

established for URIs in [RFC3986].

Applications/protocols that use this URI scheme name:

The scheme is used by ENUM DDDS services to specify resources that

support the IAX2 protocol.  The IAX2 protocol provides

application-layer control and media protocol for creating,

modifying, and terminating multimedia sessions over Internet

Protocol (IP) networks.

Interoperability considerations:

None.

Security considerations:

The IAX2 URI Scheme does not introduce any new security concerns

except that it provides a uniform syntax for describing IAX2

resources and that, when published, these addresses are subject to

various denial os service attacks.

Contact:

Ed Guy, edguy@emcsw.com, +1.973.437.4519.

Author/Change controller

Not Applicable.

References:

Spencer, M., Shumard, K., Capouch, B., and E. Guy, 'IAX2: Inter-

Asterisk eXchange Version 2,' draft-guy-iax-02 Work In Progress,

October 2006.

5.2.  URI Comparison

Some operations in this specification require determining whether two

IAX2 URIs are equivalent.  IAX2 URIs are compared for equality

according to the following rules:

All components of the URI MUST be identical except:

A host in domain form and in IP address form are considered

identical if and only if the host name resolves to exactly one

Spencer, et al.          Expires April 26, 2007                [Page 13]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

address record and that address record matches the given IP

address.

The port, if omitted, is considered to be the same as the default,

4569.

Only the user and password fields are case sensitive.

The URIs within each of the following sets are equivalent:

iax2:atlanta.com/alice

iax2:AtLaNtA.com/ALicE

iax2:atlanta.com:4569/alice

iax2:atlanta.com:4569/alice

iax2:192.168.1.1/alice (if atlanta.com resolves to 192.168.1.1)

The URIs within each of the following sets are not equivalent:

iax2:ALICE@atlanta.com/alice

iax2:alice@atlanta.com/alice

Spencer, et al.          Expires April 26, 2007                [Page 14]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.  Peer Behavior and Messages

Messages are divided into two categories: reliable and non-

guaranteed.  The reliable messages are referred to as "Full Frames."

In addition to a message type indicator and facilities to ensure

reliability, see Section 7, they include the full call identifier.

It consists of each of peer's identifiers for the call.  Additional

attributes, "Information Elements" or "IEs", MAY be associated with

the Full Frame messages.

The non-guaranteed messages are referred to as "Mini-Frames" and

"Meta Frames" and these more compact messages only have the

originating peer's call identifier and may not have any "Information

Elements."

Peer behavior is presented in several partitions divided by the

following functional areas:

Registration (OPTIONAL)

Call Link Management

Call Path Optimization (OPTIONAL)

Mid-Call Behavior

Call Tear Down

Network Monitoring

Digit Dialing (OPTIONAL)

Firmware Download(OPTIONAL)

Provisioning (OPTIONAL)

Miscellaneous

Media Messages

Each of these behavior topics and the messages involved are described

in the sections which follow.

6.1.  Registration (OPTIONAL)

6.1.1.  Overview

In order for one IAX2 peer to be reachable by another IAX2 peer, the

Spencer, et al.          Expires April 26, 2007                [Page 15]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

calling peer needs the network address of the receiving peer.  This

address may be manually provisioned, determined through a shared

directory, e.g. an ENUM-like service, [RFC3761] or configured using

the IAX2 protocol.  IAX2 provides a facility for one peer to register

its address and credentials with another so that others may locate

it.  The IAX2 registration facility is optional.  If implemented, the

IAX2 registration protocol MAY be done in parts, e.g., an analog

telephone adapter MAY only implement the registrant portion of the

protocol.

IAX2 allows user authentication via multiple methods.  MD5 Message-

Digest authentication [RFC1321] uses a challenge/response md5 sum

arrangement, but still requires that both ends have plaintext access

to the secret.  Rivest, Shamir and Adleman's (RSA) algorithm

[RFC2437] allows unidirectional secret knowledge through public/

private key pairs.  IAX2 Private keys SHOULD always be Triple Data

Encryption Standard (3DES) encrypted [RFC1851].

Spencer, et al.          Expires April 26, 2007                [Page 16]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

________________

|                |

|  Unregistered  |<--------------------------\

|________________|                           |

|                                    |

/Init         |                                    |

------------  |                                    |

snd REGAUTH   |    +--------+                      |

|    |        | rec REGAUTH          |

_______V____V___     | -----------          |

|                |    | snd REGREQ           |

|   Reg Sent     +----+                      |

|________________+----------+                |

|    ^              | rec REGAUTH    |

rec REGACK   |    |              | /No Credentials|

------------  |    | REG timeout  | -------------- |

snd ack      |    | -------      | snd ack        |

|    | REGREQ     __V___             |

_______V____|___        |      |            |

|                |       |  No  |            |

|   Registered   |       | Auth |            |

|________________|       |______|            |

|                   ^                |

|                   | rec REGAUTH    |

| release           | /No Credentials|

| -------           | -------------- |

+-------+     | snd REGREL        | snd ack        |

rec REGAUTH  |       |     |                   |                |

-----------  |      _V_____V________           |                |

snd REGREL   |     |                |----------+                |

+-----+   Releasing    |---------------------------+

|________________|      rec ACK

-------

x

__________

rec  REGREJ     |          |

----------   *->| Rejected |

snd   ack       |__________|

Figure 1: Registrant State Diagram

Registration, illustrated in Figure 1, is performed by a registrant

that sends a username and a registration 'refresh' period to the

registrar.  This is accomplished with a REGREQ message.  If

Spencer, et al.          Expires April 26, 2007                [Page 17]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

authentication is required, the registrar responds with the REGAUTH

message which indicates the types of authentication supported by the

registrar.  In response, the registrant resends a REGREQ with one of

the supported authentications.  If the registrant can not

authenticate, no further action is necessary.  If accepted, the

registrar sends a REGACK message which MUST indicate the 'apparent

address' and SHOULD indicate the 'refresh'/expire time.  If no

'refresh' is sent a default registration expiration of 60 seconds

MUST be assumed by both peers.  At any time during this exchange, the

registrar may send a REGREJ message to indicate a failure.

A registration has a specified time period associated with it for

which it is valid.  This time period begins when the registrar sends

a REGACK message.  A registrant may extend this time period by

sending another REGREQ message and performing the messaging described

above before this time period expires.  A registrant MAY also force

an expiration in the registrar by sending the REGREL message.  This

message may be challenged with REGAUTH or if sufficient credentials

were included, it will be accepted with REGACK.  In response to a

REGAUTH, a REGREL message SHOULD be resent using the specified

credentials.

See sections Section 9.3 and Section 9.4 for example call flows.

6.1.2.  REGREQ Registration Request Message

The REGREQ occurs independently of any media-carrying call.  A REGREQ

MUST include the 'username' IE and SHOULD include the 'refresh' IE.

A REGREQ is used both for an initial registration request as well as

for a reply to a REGAUTH.  As a reply to a REGAUTH message, it MUST

include credentials such as a response to a REGAUTH's challenge.

Upon receipt of a REGREQ message which has credentials, a registrar

MUST determine their validity.  If valid, it MUST respond with a

REGACK message indicating the time period for which this registration

is valid.  If the provided credentials are not valid or the registrar

cannot validate the credentials, the registrar MUST respond with a

REGREJ message.  If credentials are not provided, the registrar MUST

respond with a REGAUTH message that indicates the available

authentication methods.

Registrants MUST implement this message and registrars MUST be able

to process it.

Spencer, et al.          Expires April 26, 2007                [Page 18]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

The following table specifies IEs for this message:

+------------+----------------+-------------+-------------+

| IE         | Section        | Status      | Comments    |

+------------+----------------+-------------+-------------+

| Username   | Section 8.6.6  | Required    |             |

|            |                |             |             |

| MD5 Result | Section 8.6.15 | Conditional | per REGAUTH |

|            |                |             |             |

| RSA Result | Section 8.6.16 | Conditional | per REGAUTH |

|            |                |             |             |

| Refresh    | Section 8.6.18 | Optional    |             |

+------------+----------------+-------------+-------------+

6.1.3.  REGAUTH Registration Authentication Response Message

A REGAUTH is a response to a REGREQ or REGREL.  It is sent when a

registrar requires authentication to permit registration.  A REGAUTH

message MUST include the 'authentication methods' and 'username' IEs,

and the 'MD5 challenge' or 'RSA challenge' IE if the authentication

methods include MD5 or RSA.

Upon receipt of a REGAUTH message, the registrant MUST resend the

REGREQ or REGREL message with one of the requested credentials, if it

has the specified credentials.

Registrars MUST implement this message and registrants MUST be able

to process it.

The following table specifies IEs for this message:

+--------------+----------------+-------------+---------------+

| IE           | Section        | Status      | Comments      |

+--------------+----------------+-------------+---------------+

| Username     | Section 8.6.6  | Required    |               |

|              |                |             |               |

| Auth Methods | Section 8.6.13 | Required    |               |

|              |                |             |               |

| Challenge    | Section 8.6.14 | Conditional | If RSA or MD5 |

+--------------+----------------+-------------+---------------+

6.1.4.  REGACK Registration Acknowledgment Message

A REGACK is sent in response to a REGREQ.  A REGACK typically

includes the 'refresh' IE specifying the number of seconds before the

registration will expire.  If the 'refresh' IE is not included with a

REGACK, a default registration expiration of 60 seconds MUST be

assumed.  A REGACK MAY also include the 'username' and 'apparent

Spencer, et al.          Expires April 26, 2007                [Page 19]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

address' IEs to indicate how the peer identifies the registrant.  IEs

related to caller identification or the time the registration

occurred MAY be sent as well.

Receipt of a REGACK message requires an ACK in response.

Registrars MUST be able to send this message and registrants MUST be

able to process it.

The following table specifies IEs for this message:

+------------------+----------------+----------+----------+

| IE               | Section        | Status   | Comments |

+------------------+----------------+----------+----------+

| Username         | Section 8.6.6  | Required |          |

|                  |                |          |          |

| Date Time        | Section 8.6.30 | Required |          |

|                  |                |          |          |

| Apparent Address | Section 8.6.17 | Required |          |

|                  |                |          |          |

| Message Count    | Section 8.6.23 | Optional |          |

|                  |                |          |          |

| Calling Number   | Section 8.6.2  | Optional |          |

|                  |                |          |          |

| Calling Name     | Section 8.6.4  | Optional |          |

|                  |                |          |          |

| Firmware Version | Section 8.6.33 | Optional |          |

|                  |                |          |          |

| Refresh          | Section 8.6.18 | Optional |          |

+------------------+----------------+----------+----------+

6.1.5.  REGREJ Registration Rejection Message

A REGREJ indicates that a registration request has been rejected.

This rejection can occur for several reasons.  A REGREJ MUST include

the 'causecode' and 'cause' IEs to specify why registration was

rejected.

Upon receipt of a REGREJ message, the registrant MUST consider

registration process unsuccessful and no further interaction is

required.  A peer MAY reinitiate the the process at later time

accounting for potential configuration changes on the registrar or

registrant.

Both registrants and registrars MUST be capable of sending and

processing this message.

Spencer, et al.          Expires April 26, 2007                [Page 20]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

The following table specifies IEs for this message:

+------------+----------------+----------+----------+

| IE         | Section        | Status   | Comments |

+------------+----------------+----------+----------+

| Cause      | Section 8.6.21 | Required |          |

|            |                |          |          |

| Cause Code | Section 8.6.41 | Required |          |

+------------+----------------+----------+----------+

6.1.6.  REGREL Registration Release Request Message

A REGREL is used by a registrant for a forced release of a prior

registration.  It MUST include the 'username' IE to identify the

registrant to be released, and MAY include the 'causecode' and

'cause' IEs to specify why registration is being released.

Upon receipt of this message, a peer MUST authenticate the sender

using the provided credentials or send a REGAUTH message requesting

them.  If authenticated it MUST immediately purge its registration of

the specified registrant or send a REGREJ message if the registration

is not found.

Registrants SHOULD be capable of sending this message and registrars

MUST be able to process it.

The following table specifies IEs for this message:

+----------+----------------+-------------+-------------------------+

| IE       | Section        | Status      | Comments                |

+----------+----------------+-------------+-------------------------+

| Username | Section 8.6.6  | Required    |                         |

|          |                |             |                         |

| MD5      | Section 8.6.15 | Conditional | MD5 or RSA Result is    |

| Result   |                |             | required                |

|          |                |             |                         |

| RSA      | Section 8.6.16 | Conditional |                         |

| Result   |                |             |                         |

|          |                |             |                         |

| Cause    | Section 8.6.21 | Optional    |                         |

|          |                |             |                         |

| Cause    | Section 8.6.41 | Optional    |                         |

| Code     |                |             |                         |

+----------+----------------+-------------+-------------------------+

Spencer, et al.          Expires April 26, 2007                [Page 21]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.2.  Call Leg Management

+--------+  HANGUP/ack

|        |

_____________|__      |

|                |     |

+--------->|    Initial     |<----+

|          |________________|<---------------------+

|                  |                               ^

|       start call |                               |

|       ---------- |                               |

|       send NEW   |  +-------+                    |

|                  |  |       |  rec AUTHREQ       |

|             _____V__V__     |  -----------       |

|            |           |    |  snd AUTHREP       |

+------------|  Waiting  |----+                    |

rec REJECT           |___________|------------------------>+

----------                  |                              |

ack                       |              rec HANGUP      |

|              ---------       |

|              snd ack         |

|                              |

rec ACCEPT    |                              |

----------    |   +------+                   |

snd ack       |   |      | PROCEEDING / ack  |

_________V___V      | RINGING / ack     |

|              |     |                   |

|     Linked   |-----+                   |

|______________|------------------------>+

|               rec HANGUP      |

rec ANSWER   |               ----------      |

------------  |               snd ack         |

snd ack      |                               |

|                               |

|               rec HANGUP      |

_______V________       ---------       |

|                |       snd ack        |

|      UP        |--------------------->+

|________________|--------------------->+

finish

------

snd HANGUP

Figure 2: Call Origination State Diagram

Spencer, et al.          Expires April 26, 2007                [Page 22]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

+--------+ rec HANGUP/ack

|        |

_____________V__      | rec NEW(no Auth)/snd AUTHREQ

|                |     |

|    Initial     |-----+ rec NEW(not Auth)/snd REJECT

|                |

|________________|<--------------------+

|                              |

rec NEW       |                              |

(valid credentials)|                              |

----------    |   +------+                   |

snd ACCEPT    |   |      | snd PROCEEDING    |

_________V___V      | snd RINGING       |

|              |     |                   |

|     Linked   |-----+                   |

|              |

|______________|------------------------>+

|               rec HANGUP      |

/answered   |               ----------      |

-----------  |               snd ack         |

snd ANSWER   |                               |

|               rec HANGUP      |

_______V________       ---------       |

|                |       snd ack        |

|      UP        |--------------------->+

|________________|--------------------->+

finish

------

snd HANGUP

Figure 3: Call Termination State Diagram

6.2.1.  Overview

The IAX2 protocol can be used to setup 'links' or 'call legs' between

two peers for the purposes of placing a call.  The process,

illustrated in Figure 2 and Figure 3, starts when a peer sends a NEW

message indicating the destination 'number' (or name) of a Called

Party on the remote peer.  The remote peer can respond with either a

credentials challenge (AUTHREQ), a REJECT message, or an ACCEPT

message.  The AUTHREQ message indicates the permitted authentication

schemes and SHOULD result in the sending of an AUTHREP message with

the requested credentials.  The REJECT message indicates the call

cannot be established at this time.  And ACCEPT indicates that the

call leg between these two peers is established and that Higher level

call signaling (Section 6.3) MAY proceed.  After sending or receiving

the ACCEPT message, the Call Leg is in the 'Linked' state and is used

Spencer, et al.          Expires April 26, 2007                [Page 23]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

to pass call control message until the call is completed.  Further

detail on messages used for this process can be found in Section 6.3.

Call Legs are labeled with a pair of identifiers.  Each end of the

call leg assigns the source or destination identifier during the call

leg creation process.

6.2.2.  NEW Request Message

A NEW message is sent to initiate a call.  It is the first call-

specific message sent to initiate an actual media exchange between

two peers.  'NEW' messages are unique with respect to compared to

other Call Supervision messages in that they do not require a

destination call identifier in their header.  This absence is because

the remote peer's source call identifier is not created until after

receipt of this frame.  Before sending a NEW message, the local IAX2

peer MUST assign a source call identifier that is not currently being

used for another call.  A time-stamp MUST also be assigned for the

call, beginning at 0 and incrementing each millisecond.  Sequence

numbers for a NEW message, described in the transport section,

(Section 7) are both set to 0.

A NEW message MUST include the 'version' IE, and it MUST be the first

IE; the order of other IEs is unspecified.  A NEW SHOULD generally

include IEs to indicate routing on the remote peer, e.g., via the

'called number' IE or to indicate a peer partition or ruleset, the

'called context' IE.  Caller identification and CODEC negotiation IEs

MAY also be included.

Upon receipt of a NEW message, the receiving peer examines the

destination and MUST perform one of the following actions:

Send a REJECT response,

Challenge the caller with an AUTHREQ response,

Accept the call using an ACCEPT message, or

Abort the connection using a HANGUP message, although the REJECT

message is preferred at this point in call.

If the call is accepted, the peer MUST progress the call and further

respond with one of PROCEEDING, RINGING, BUSY or ANSWER depending on

the status of the called party on the peer.  See Section 6.3 for

further detail.

Spencer, et al.          Expires April 26, 2007                [Page 24]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

The following table specifies IEs for the NEW message:

+--------------+----------------+-------------+---------------------+

| IE           | Section        | Status      | Comments            |

+--------------+----------------+-------------+---------------------+

| Version      | Section 8.6.10 | Required    |                     |

|              |                |             |                     |

| Called       | Section 8.6.1  | Required    |                     |

| Number       |                |             |                     |

|              |                |             |                     |

| Auto Answer  | Section 8.6.24 | Optional    |                     |

|              |                |             |                     |

| Codecs Prefs | Section 8.6.44 | Required    |                     |

|              |                |             |                     |

| Calling      | Section 8.6.37 | Required    |                     |

| Presentation |                |             |                     |

|              |                |             |                     |

| Calling      | Section 8.6.2  | Optional    |                     |

| Number       |                |             |                     |

|              |                |             |                     |

| Calling TON  | Section 8.6.38 | Required    |                     |

|              |                |             |                     |

| Calling TNS  | Section 8.6.39 | Required    |                     |

|              |                |             |                     |

| Calling Name | Section 8.6.4  | Optional    |                     |

|              |                |             |                     |

| ANI          | Section 8.6.3  | Optional    |                     |

|              |                |             |                     |

| Language     | Section 8.6.9  | Optional    |                     |

|              |                |             |                     |

| DNID         | Section 8.6.12 | Optional    |                     |

|              |                |             |                     |

| Called       | Section 8.6.5  | Conditional | 'Default' assumed   |

| Context      |                |             | if IE excluded      |

|              |                |             |                     |

| Username     | Section 8.6.6  | Optional    |                     |

|              |                |             |                     |

| RSA Result   | Section 8.6.16 | Conditional | If challenged with  |

|              |                |             | RSA.                |

|              |                |             |                     |

| MD5 Result   | Section 8.6.15 | Conditional | If challenged with  |

|              |                |             | MD5                 |

|              |                |             |                     |

| Format       | Section 8.6.8  | Required    |                     |

|              |                |             |                     |

| Capability   | Section 8.6.7  | Required    |                     |

|              |                |             |                     |

| ADSICPE      | Section 8.6.11 | Required    |                     |

Spencer, et al.          Expires April 26, 2007                [Page 25]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

| Date Time    | Section 8.6.30 | Optional    | Suggested           |

|              |                |             |                     |

| OSP Token    | Section 8.6.51 | Optional    |                     |

+--------------+----------------+-------------+---------------------+

6.2.3.  ACCEPT Response Message

An ACCEPT response is issued when a NEW message is received, and

authentication has taken place (if required).  It acknowledges

receipt of a NEW message and indicates that the call leg has been

setup on the terminating side, including assigning a CODEC.  An

ACCEPT message MUST include the 'format' IE to indicate its desired

CODEC to the originating peer.  The CODEC format MUST be one of the

formats sent in the associated NEW command.

Upon receipt of an ACCEPT, an ACK MUST be sent and the CODEC for the

call MAY be configured using the 'format' IE from the received

ACCEPT.  The call then waits for an ANSWER, HANGUP or other call

control signal.  (See Section 6.3.)  If a subsequent ACCEPT message

is received for a call which has already started, or has not sent a

NEW message, the message MUST be ignored.

The following table specifies IEs for this message:

+--------+---------------+----------+----------+

| IE     | Section       | Status   | Comments |

+--------+---------------+----------+----------+

| Format | Section 8.6.8 | Required |          |

+--------+---------------+----------+----------+

6.2.4.  REJECT Response Message

A REJECT response is sent to indicate that a NEW, AUTHREP, DIAL, or

ACCEPT request has been denied.  It MAY be due to an authentication

failure, an invalid username or if a peer cannot provide a valid

password or response to an issued challenge.  It MAY also be used to

notify a peer of a call setup failure, e.g., when IAX2 peers cannot

negotiate a CODEC to use or when a firmware download request cannot

be satisfied.  Upon receipt of a REJECT message, the call leg is

destroyed and no further action is required.  (Note: REJECT messages

require an explicit ACK.)

REJECT messages MAY include the 'causecode' and 'cause' IEs to

indicate the rejection reason.

Spencer, et al.          Expires April 26, 2007                [Page 26]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

The following table specifies IEs for this message:

+------------+----------------+----------+----------+

| IE         | Section        | Status   | Comments |

+------------+----------------+----------+----------+

| Cause      | Section 8.6.21 | Optional |          |

|            |                |          |          |

| Cause Code | Section 8.6.41 | Optional |          |

+------------+----------------+----------+----------+

6.2.5.  HANGUP Request Message

A HANGUP message is sent by either peer and indicates a call tear-

down.  It MAY include the 'causecode' and 'cause' IEs to indicate the

reason for terminating the call.  Upon receipt of a HANGUP message,

an IAX2 peer MUST immediately respond with an ACK, and then destroy

the call leg at its end.  After a HANGUP message has been received

for a call leg, any messages received which reference that call leg

(i.e., have the same source/destination call identifiers) MUST be

answered with an INVAL message.  This indicates that the received

message is invalid because the call no longer exists.

After sending a HANGUP message, the sender MUST destroy the call and

respond to subsequent messages regarding this call with an INVAL

message.

The following table specifies IEs for this message:

+------------+----------------+----------+----------+

| IE         | Section        | Status   | Comments |

+------------+----------------+----------+----------+

| Cause      | Section 8.6.21 | Optional |          |

|            |                |          |          |

| Cause Code | Section 8.6.41 | Optional |          |

+------------+----------------+----------+----------+

6.2.6.  AUTHREP Authentication Reply Message

An AUTHREP MUST include the appropriate challenge response or

password IE, and is only sent in response to an AUTHREQ.  An AUTHREP

requires a response of either an ACCEPT or a REJECT.

Typical reasons for rejecting an AUTHREP include 'destination does

not exist' and 'suitable bearer not found'.

Spencer, et al.          Expires April 26, 2007                [Page 27]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

The following table specifies IEs for this message:

+------------+----------------+-------------+----------+

| IE         | Section        | Status      | Comments |

+------------+----------------+-------------+----------+

| RSA Result | Section 8.6.16 | Conditional | If RSA   |

|            |                |             |          |

| MD5 Result | Section 8.6.15 | Conditional | If MD5   |

+------------+----------------+-------------+----------+

6.2.7.  AUTHREQ Authentication Request Message

The AUTHREQ message is sent in response to a NEW message if

authentication is required for the call to be accepted.  It MUST

include the 'authentication methods' and 'username' IEs, and the

'challenge' IE if MD5 or RSA authentication is specified.

Upon receiving an AUTHREQ message, the receiver MUST respond with an

AUTHREP or HANGUP message.

The following table specifies IEs for this message:

+--------------+----------------+----------+----------+

| IE           | Section        | Status   | Comments |

+--------------+----------------+----------+----------+

| Username     | Section 8.6.6  | Required |          |

|              |                |          |          |

| Auth Methods | Section 8.6.13 | Required |          |

|              |                |          |          |

| Challenge    | Section 8.6.14 | Required |          |

+--------------+----------------+----------+----------+

6.3.  Call Control

6.3.1.  Overview

IAX2's call control messages provide end-to-end signaling functions

common to other telephony control protocols.  The messages include

RINGING, ANSWER, BUSY, and PROCEEDING.  These messages MUST only be

sent after an IAX2 call leg has been ACCEPTed.

In response to an exchange starting with a NEW message, typically,

the first call control message is RINGING, however, a PROCEEDING

message MAY precede it or the call MAY proceed directly to the ANSWER

message.  If the call is answered, an ANSWER message will be sent.

Other possibilities include a "BUSY" indication, or if the called

party's service cannot be reached, the call will be town down using

the link-level HANGUP and an appropriate cause code.

Spencer, et al.          Expires April 26, 2007                [Page 28]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

If the link was started with a DIAL message, the sequence is an

optional PROCEEDING, then optional RINGING, then ANSWER or BUSY.  Of

course, a link level HANGUP MAY occur at any time.

Various extensions to IAX2 Control messages have been deployed for

passing application-specific data over IAX2 control link.  One such

extensions is an application that controls ham radio transceivers.

An IAX2 peer that receives a control message that is not understood

MUST respond with the UNSUPPORT message.

The mandatory IAX2 control messages are explained below.

6.3.2.  PROCEEDING Response Message

The PROCEEDING message SHOULD sent to a calling party when their call

request is being processed by a further network element but has not

yet reached the called party.

Upon receipt of a PROCEEDING message, the peer SHOULD perform

protocol-specific actions to indicate this fact to the calling party,

e.g., tones, an ISUP Proceeding message, etc.  If the prior call leg

is utilizing the IAX2 protocol, a PROCEEDING message MUST be sent to

that peer.  The processing of this message at an originating or

transcoding peer is not specified, however, if possible, the status

may be displayed to the calling party.

The PROCEEDING message does not require any IEs.

6.3.3.  RINGING Response Message

This message is sent from a terminating party to indicate that that

the called party's service has processed the call request and is

being alerted to the call.  A IAX2 RINGING message MUST be sent to an

IAX2-based calling party when the peer determines that the called

party is being alerted, e.g., when their phone is ringing.

Upon receipt of an IAX2 RINGING message, the peer MUST pass this

indication to the calling party, unless the calling party has already

received such indication.  For an initiating peer, this is typically

done by starting the ring-back tone, however, many implementations

start ringback before ringing in order to meet user expectations.  If

the calling party is using the IAX2 protocol, a RINGING message MUST

be passed to this caller.

The RINGING message does not require any IEs.

Spencer, et al.          Expires April 26, 2007                [Page 29]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.3.4.  ANSWER Response Message

This message is sent from the called party to indicate that party has

accepted the call request and is communicating with the calling

party.  Upon receipt of this message, any ring-back or other progress

tones MUST be terminated and the communications channel MUST be

opened.

The ANSWER message does not require any IEs.

6.4.  Mid-Call Link Operations

6.4.1.  FLASH Request Message

The FLASH message is sent to indicate a mid call feature.  Its

interpretation is system dependent and if it is not expected, it

SHOULD be ignored.  Typically, this message is only sent from Analog

Telephone adapters when a brief circuit interruption is made during

an answered call.

The FLASH message does not require any IEs.

6.4.2.  HOLD Request Message

The HOLD message is sent to cause the remote system to stop

transmitting audio on this channel, and optionally replace the audio

with music or other sounds.  If the remote system cannot perform this

request, it SHOULD be ignored.

The HOLD message SHOULD only be sent in IAX2 calls which are started

using the DIAL message.

The HOLD message does not require any IEs.

6.4.3.  UNHOLD Request Message

The UNHOLD message is sent to cause the remote system to stop

transmitting audio on this channel, and optionally replace the audio

with music or other sounds.  If the remote system cannot perform this

request, it SHOULD be ignored.

The UNHOLD message SHOULD only be sent in IAX2 calls after the HOLD

message.

The UNHOLD message does not require any IEs.

Spencer, et al.          Expires April 26, 2007                [Page 30]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.4.4.  QUELCH Request Message

The QUELCH message is sent to cause the remote peer to squelch or

stop transmitting audio on this channel.  It MAY replace the audio

sent to the further party with music or other sounds.  If the remote

system cannot perform this request, it SHOULD be ignored.

The QUELCH message MUST only be sent in IAX2 calls after an ACCEPT is

sent or received; it SHOULD only be used on calls which are started

using the NEW message.

The QUELCH message does not require any IEs.

6.4.5.  UNQUELCH Request Message

The UNQUELCH message is sent to cause the remote system to resume

transmitting audio on this channel.  If it previously replaced the

audio with music or other sounds, it MUST discontinue it immediately.

If the remote system cannot perform this request, it SHOULD be

ignored.

The UNQUELCH message SHOULD only be sent in IAX2 calls after the

QUELCH message.

The UNQUELCH message does not require any IEs.

6.4.6.  TRANSFER Request Message

The TRANSFER message causes the receiving peer to restart the call

using another specified number.  The receiving peer MUST be on the

calling side of this call leg and the new call behavior is

unspecified.  After processing this message, a HANGUP message SHOULD

be sent and the call leg torn down.

When sending a TRANSFER message, the new number to which the call is

being transferred MUST be included in the CALLED_NUMBER IE and a

CALLED_CONTEXT IE MAY be included.  The call leg MUST not be used for

anything else and MAY be torn down.

The following table specifies IEs for this message:

+----------+---------------+----------+-----------------------------+

| IE       | Section       | Status   | Comments                    |

+----------+---------------+----------+-----------------------------+

| Called   | Section 8.6.1 | Required |                             |

| Number   |               |          |                             |

|          |               |          |                             |

Spencer, et al.          Expires April 26, 2007                [Page 31]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

| Called   | Section 8.6.5 | Optional | Use this IE if context is   |

| Context  |               |          | other than default.         |

+----------+---------------+----------+-----------------------------+

6.5.  Call Path Optimization

If a peer is handling a call between two other IAX2 peers and the

peer no longer has any need to monitor the progress, content, or

duration of the call, it MAY remove itself from the call by directing

the other two peers to communicate directly.  This call path

optimization, or "supervised transfer," is done in a manner that

ensures the call will not be lost in the process; the initiating peer

does not give up control of the process until it has confirmed the

other two peers are communicating.  Note: the parties involved in the

call are not aware of this operation; it is purely a network

operation.

Spencer, et al.          Expires April 26, 2007                [Page 32]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

________________

rec  TXREJ              |                |     rec TXREL

----------   *--------->|      None      |<-----------------+

snd  TXREJ              |________________|        ack       ^

to other                  |           |                     |

|           V                     |

|                                 |

|           *   (From All)        |

/Init Transfer |           | rec TXREQ           |

------------  |           | ---------           |

snd TXREQ   |           | snd TXCNT           |

to both     |           |                     |

_v___________v__                   |

|                |                  |

|     Begin      |----------------->+

|________________|                  |

|           |                     |

rec TXACC |           | rec TXREADY         |

--------- |           | ---------           |

snd TXREADY |           |     x               |

|           |                     |

_v___________v__                   |

|                |----------------->+

----------|     Ready      |----------        |

|          |________________|          |       |

|                   |                  |       |

/Both Legs Ready|   /Both Legs Ready|       rec TXMEDIA|       |

and not media-only|    and media-only |                  |       |

------------  |    ------------   |       -----------|       |

snd TXREL     |     snd TXMEDIA   |            x     |       |

|                   |                  |       |

____V____          _____V___            ___V_____  |

|         |        |         |          |         | |

| Release |        |  Media  |          | Media   | |

|_________|        |_________|          |  Pass   | |

|              |_________| |

|                  |       |

V                  V       |

rec  TXCNT                           +------------------------->+

----------  (In any state)

snd  TXACC

Figure 4: Call Path Optimization State Diagram

When a peer initiates this procedure, both call legs MUST be in the

Spencer, et al.          Expires April 26, 2007                [Page 33]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

UP state, i.e., they MUST have sent or received the ACCEPT message

for that call leg.  To start, it sends a TXREQ message with the

addresses and information from the other remote peers to each its

neighbors.  If capable of performing this procedure, they begin

transmitting all channel information to both the initiating peer and

the new remote peer.  They also send a TXCNT message indicating

packet counts for the call leg to the new remote peer.  Each TXCNT

message is acknowledged with a TXACC message.  The peers respond by

sending a TXREADY message to the initiator indicating that they have

confirmed the new communications path.  When all remote peers have

sent the initiator a TXREADY message, the transfer is successful and

the initiator responds with a TXREL and has finished its involvement

with the call.  If during the transfer process, the two remote peers

cannot communicate, they send a TXREJ message to the initiator.  An

example is shown in Section 9.7.

These messages are described in the sections which follow:

6.5.1.  TXREQ Transfer Request Message

The TXREQ message is sent by a peer to initiate the transfer process.

When sent, It MUST be sent to both adjacent peers involved in the

call.

It MUST include the following Information Elements:

+------------------+----------------+----------+----------+

| IE               | Section        | Status   | Comments |

+------------------+----------------+----------+----------+

| Apparent Address | Section 8.6.17 | Required |          |

|                  |                |          |          |

| Call Number      | Section 8.6.20 | Required |          |

|                  |                |          |          |

| Transfer ID      | Section 8.6.26 | Required |          |

+------------------+----------------+----------+----------+

The Apparent Address is the IP address data structure address for the

other remote peer.  The Call Number IE is The callid used by the

other remote peer and the Transfer ID is a unique number assigned by

the initiator.

Upon receipt of a TXREQ message for a valid call from the

corresponding remote peer, a peer MUST respond by attempting to

communicate with the newly specified remote peer.  This task is

accomplished by sending a TXCNT message directly to the peer at the

address specified in the Apparent Address parameter.

Spencer, et al.          Expires April 26, 2007                [Page 34]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.5.2.  TXCNT Transfer Connectivity Response Message

The TXCNT message is used to verify connectivity with a potential

replacement peer for a call.  It MUST include the TRANSFERID IE.

Upon receipt on a message of this type, and if the peer has

previously received a TXREQ for this call leg, the peer MUST respond

with a TXACC message.

If the TXCNT Message is not successfully transmitted or if a TXACC

message is not received in response to it, the transfer process MUST

be aborted by sending a TXREJ message to the initiating host.

It MUST include the following Information Element:

+----------+----------------+----------+----------------------------+

| IE       | Section        | Status   | Comments                   |

+----------+----------------+----------+----------------------------+

| Transfer | Section 8.6.26 | Required | A unique number assigned   |

| ID       |                |          | by the initiator.          |

+----------+----------------+----------+----------------------------+

6.5.3.  TXACC Response Message

Like the TXCNT message, the TXACC message is used to verify

connectivity with a potential replacement peer.  It MUST include the

TRANSFERID IE.  Upon receipt on a message of this type if the peer is

attempting to transfer this call leg, the peer stops sending call

related media to the initiating peer and sends a TXREADY message to

it.

It MUST include the following Information Element:

+----------+----------------+----------+----------------------------+

| IE       | Section        | Status   | Comments                   |

+----------+----------------+----------+----------------------------+

| Transfer | Section 8.6.26 | Required | A unique number assigned   |

| ID       |                |          | by the initiator.          |

+----------+----------------+----------+----------------------------+

6.5.4.  TXREADY Transfer Ready Response Message

The TXREADY message indicates that the sending peer has verified

connectivity with the peer which it was instructed to transfer the

call.  It MUST include the TRANSFERID IE.  When TXREADY messages are

received from both remote peers, it MUST discontinue media transport

and send a TXREL message to each peer.

Spencer, et al.          Expires April 26, 2007                [Page 35]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

It MUST include the following Information Element:

+----------+----------------+----------+----------------------------+

| IE       | Section        | Status   | Comments                   |

+----------+----------------+----------+----------------------------+

| Transfer | Section 8.6.26 | Required | A unique number assigned   |

| ID       |                |          | by the initiator.          |

+----------+----------------+----------+----------------------------+

6.5.5.  TXREL Transfer Release Response Message

The TXREL message indicates that the transfer process has

successfully completed.  After sending and upon receipt of this

message, no further interaction (other than an ACK, of course) is

needed between the peers on this call-leg.  The TXREL is also used to

revert a split-media call (one where the media and signaling follow

different paths) to a call where the media and signalling follow the

same path.

It MUST include the following Information Element:

+-------------+----------------+----------+----------+

| IE          | Section        | Status   | Comments |

+-------------+----------------+----------+----------+

| Call Number | Section 8.6.20 | Required |          |

+-------------+----------------+----------+----------+

6.5.6.  TXMEDIA Transfer Media Message

The TXREL message indicates that the MEDIA transfer process has

successfully completed.  After sending and upon processing of this

message, full frames MUST continue to follow the original signaling

path and media frames MUST follow the newly negotiated path.  This

split-path process continues until the call ends with a HANGUP or

peer receives a TXREL message for the call leg.  A peer MAY force the

paths to rejoin by sending a TXREL message.

It MUST include the following Information Element:

+-------------+----------------+----------+----------+

| IE          | Section        | Status   | Comments |

+-------------+----------------+----------+----------+

| Call Number | Section 8.6.20 | Required |          |

+-------------+----------------+----------+----------+

Spencer, et al.          Expires April 26, 2007                [Page 36]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.5.7.  TXREJ Transfer Rejection Response Message

The TXREJ MAY be sent at anytime during the transfer process to

indicate that the transfer cannot proceed.  Upon receiving a TXREJ

message, if the receiver is the initiating peer, it MUST form a TXREJ

message and send it to the other remote peer.

The TXREJ message does not require any IEs.

6.6.  Call Tear Down

The messages used to finish a call vary depending on the particular

process the call is in at the time.  The terminal messages for a call

are:

HANGUP.  See Section 6.2.5.

REJECT.  See Section 6.2.4.

TRANSFER.  See Section 6.4.6.

TXREADY.  See Section 6.5.4.

These messages are discussed in their respective sections.  Also, if

the reliable transport procedures determines that messaging cannot be

maintained, the call leg MUST be torn down without any other

indications over the errant IAX2 call leg.

6.7.  Network Monitoring

The IAX2 protocol has various tools to determine the network load.

It uses the POKE message to monitor reachability of remote peer and

the LAGRQ message to measure the quality of a current call leg

including the jitter buffer delay.

6.7.1.  POKE Request Message

A POKE message is sent to test connectivity of a remote IAX2 peer.

It is similar to a PING message, except that it MUST be sent when

there is no existing call to the remote endpoint.  It MAY also be

used to "qualify" a user to a remote peer, so that the remote peer

can maintain awareness of the state of the user.  A POKE MUST have 0

as its destination call number.

Upon receiving a POKE message, the peer MUST respond with a PONG

message.

This message does not require any IEs.

Spencer, et al.          Expires April 26, 2007                [Page 37]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.7.2.  PING Request Message

A PING message is sent to test connectivity of the remote IAX2

endpoint on an existing call.  Transmission of a PING MAY occur when

a peer-defined number of seconds have passed without receiving an

incoming media frame on a call, or by default every 20 seconds.

Receipt of a PING requires an acknowledging PONG be sent.

This message does not require any IEs.

6.7.3.  PONG Response Message

A PONG message is a response to a PING or a POKE.  It acknowledges

the connection.  The receiver uses the time-stamp of the received

PING or POKE and its times to determine the Round Trip Time of the

connection.  Several receiver report IEs MAY be included with a PONG,

including received jitter, received frames, delay, and dropped

frames.  Receipt of a PONG requires an ACK.

This message does not require any IEs.

6.7.4.  LAGRQ Lag Request Message

A LAGRQ is a lag request.  It is sent to determine the lag between

two IAX2 endpoints, including the amount of time used to process a

frame through a jitterbuffer (if any).  It requires a clock-based

time-stamp, and MUST be answered with a LAGRP, which MUST echo the

LAGRQ's time-stamp.  The lag between the two peers can be computed on

the peer sending the LAGRQ by comparing the time-stamp of the LAGRQ

and the time the LAGRP was received.

This message does not require any IEs.

6.7.5.  LAGRP Lag Response Message

A LAGRP is a lag reply, sent in response to a LAGRQ message.  It MUST

send the same time-stamp it received in the LAGRQ after passing the

received frame through any jitter buffer the peer has configured.

This message does not require any IEs.

6.8.  Digit Dialing

Digit Dialing support is an optional portion of the IAX2 protocol

designed to support devices that do not maintain their own dial

plans, for instance, analog telephone adapters, or ATAs.  The dialing

portion of the IAX2 protocol MAY be implemented for the client/

phone-side, server side or not all.  The exchanges work as a series

Spencer, et al.          Expires April 26, 2007                [Page 38]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

of Dialing Plan requests (DPREQ) each followed by a response (DPREP)

indicating if additional digits SHOULD be collected before sending

the call.  The sections that follow describe these messages and the

rules associated with them.

6.8.1.  DPREQ Dial Plan Request Message

A DPREQ is a request for the server to analyze the passed called

number and determine if there is a valid dialing pattern on the

remote peer.  It MUST include the 'called number' IE to specify what

extension is being queried.  This command is used in the case where a

local peer does not handle its own dialplan/extension switching.  The

local peer can inquire (as a user dials) how the remote peer

perceives the 'called number'.  If a DPREP is received indicating

that the number is valid, a DIAL MAY be sent.

This message MAY be sent by the client and MUST be implemented on

servers which provide IAX2 dialing support.

It MUST include the following Information Element:

+-------------+----------------+----------+----------+

| IE          | Section        | Status   | Comments |

+-------------+----------------+----------+----------+

| Call Number | Section 8.6.20 | Required |          |

+-------------+----------------+----------+----------+

6.8.2.  DPREP Dial Plan Response Message

A DPREP is a reply to a DPREQ, containing the status of the dialplan

entry requested in the 'called number' IE of the DPREQ.  It MUST

include the 'called number', 'dpstatus', and 'refresh' IEs.  The

called number is the same one received in the 'called number' IE of

the DPREQ.  The 'dpstatus' IE contains the status of the dialplan

entry referenced by the received called number.  The status indicates

whether the called number exists, can exist, needs more digits, or is

invalid.  More information can be found in Section 8.6 under the

DPSTATUS information element.  The 'refresh' IE specifies the number

of minutes the 'dpstatus' is valid.  If the 'refresh' IE is not

present, a default 10 minutes period is assumed.

The sending of this message MUST be implemented by servers which

support IAX2 dialing.  Clients which support IAX2 dialing MUST be

capable of receiving such messages.

Spencer, et al.          Expires April 26, 2007                [Page 39]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

It MUST include the following Information Elements:

+---------+----------------+----------+-----------------------------+

| IE      | Section        | Status   | Comments                    |

+---------+----------------+----------+-----------------------------+

| Call    | Section 8.6.20 | Required |                             |

| Number  |                |          |                             |

|         |                |          |                             |

| Dial    | Section 8.6.20 | Required | Indicates if number exists, |

| Plan    |                |          | is a partial match, etc.    |

| Status  |                |          |                             |

|         |                |          |                             |

| Dial    | Section 8.6.20 | Optional | Inclusion is strongly       |

| Plan    |                |          | suggested. The default is   |

| Refresh |                |          | 10 minutes.                 |

+---------+----------------+----------+-----------------------------+

6.8.3.  DIAL Request Message

The DIAL message is used with IAX2 peers that do not maintain their

own dialplan/extension routing.  Once an extension is validated by

one or more DPREQ/DPREP exchanges, the number MAY be dialed in a DIAL

message, using the 'called number' IE to specify the extension it is

attempting to reach.  The remote peer then handles the remaining

aspects of call setup, including ringing the extension and notifying

the local peer when it has been answered following the same

requirements as the NEW command (Section 6.2.2).

The following table specifies the IEs used by this message:

+----------+---------------+----------+-----------------------------+

| IE       | Section       | Status   | Comments                    |

+----------+---------------+----------+-----------------------------+

| Called   | Section 8.6.1 | Required |                             |

| Number   |               |          |                             |

|          |               |          |                             |

| Called   | Section 8.6.5 | Optional | Use this IE if context is   |

| Context  |               |          | other than default.         |

+----------+---------------+----------+-----------------------------+

6.9.  Firmware Download

The IAX2 protocol can download firmware to devices which request it.

This is an optional portion of the protocol designed such that an

embedded device can retrieve binary images from a 'server' peer.

Note Well: There is no security on firmware downloads.

Spencer, et al.          Expires April 26, 2007                [Page 40]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.9.1.  FWDOWNL Firmware Download Request Message

An FWDOWNL message is a request made by an IAX2 device to download a

firmware binary using the IAX2 protocol.  The FWDOWNL message MUST

include the device type and the block IEs.  If the recipient is

capable of transmitting a firmware image, it does so with a FWDATA

message which includes the 'fwblockdesc' (Section 8.6.34 and

'fwblockdata' (Section 8.6.35) IEs.

On receiving this message, the server MUST determine if it has the

firmware for the specified device and respond with a FWDATA message

containing the data block of firmware that was requested.  If the

firmware is not available or the offset is invalid, it MUST respond

with a REJECT message.  (See Section 6.2.4.)

The following table specifies IEs used by this message

+-------------+----------------+----------+----------+

| IE          | Section        | Status   | Comments |

+-------------+----------------+----------+----------+

| DEVTYPE     | Section 8.6.31 | Required |          |

|             |                |          |          |

| FWBLOCKDATA | Section 8.6.35 | Required |          |

+-------------+----------------+----------+----------+

6.9.2.  FWDATA Firmware Data Response Message

An FWDATA message is sent in response to an FWDOWNL message.  It

carries a block of a firmware and IEs that identify which block is

being carried.  The 'fwblockdata' and 'fwblockdesc' IEs are used.  If

the FWDATA is the last of the transaction (that is, 'fwblockdata' is

of length 0), an ACK is required upon receipt.  Otherwise, an FWDOWNL

requesting the next block to download in the 'fwblockdesc' IE is

required.

The ordered set of firmware blocks is concatenated to make up the

downloaded firmware.

The following table specifies IEs used by this message

+-------------+----------------+----------+-------------------------+

| IE          | Section        | Status   | Comments                |

+-------------+----------------+----------+-------------------------+

| FWBLOCKDESC | Section 8.6.34 | Required |                         |

|             |                |          |                         |

| FWBLOCKDATA | Section 8.6.35 | Required | Length 0 at end of      |

|             |                |          | transfer.               |

+-------------+----------------+----------+-------------------------+

Spencer, et al.          Expires April 26, 2007                [Page 41]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.10.  Provisioning

The IAX2 protocol allows provisioning of peer devices.  This is a

useful way to specify configuration information to a device that does

not maintain state.  Provisioning generally allows for configuration

of network information (e.g., server address, UDP port number to use,

type of service setting, etc.), user information (e.g., username,

password, etc.), and CODEC information (i.e., which CODECs the device

MAY use when transmitting media to an IAX2 peer).  Provisioning can

be configured for a particular class or type of device using the

'device type' information element, or for one specific device using

the 'service identifier' information element.

6.10.1.  PROVISION Request Message

A PROVISION message is sent by a provisioning peer to a remote IAX2

device.  It includes either the 'provisioning' or the 'AES

provisioning' IE, which contains the data to be used to perform the

actual provisioning of the device.  The format is device specific.

The following table specifies IEs used by this message

+-----------------+----------------+-------------+------------------+

| IE              | Section        | Status      | Comments         |

+-----------------+----------------+-------------+------------------+

| PROVISIONING    | Section 8.6.28 | Conditional | Either           |

|                 |                |             | PROVISIOINNG or  |

|                 |                |             | AESPROVISIONING  |

|                 |                |             | MUST be          |

|                 |                |             | included.        |

|                 |                |             |                  |

| AESPROVISIONING | Section 8.6.29 | Conditional |                  |

+-----------------+----------------+-------------+------------------+

6.11.  Miscellaneous

6.11.1.  ACK acknowledgement Message

An ACK acknowledges the receipt of an IAX2 message.  An ACK is sent

upon receipt of a full frame which does not have any other protocol-

defined response.  An ACK MUST have both a source call number and

destination call number.  It MUST also not change the sequence number

counters, and MUST return the same time-stamp it received.  This

time-stamp allows the originating peer to determine to which message

the ACK is responding.  Receipt of an ACK requires no action.

An ACK MAY also be sent as an initial acknowledgment of an IAX2

message which requires some other protocol-defined message

Spencer, et al.          Expires April 26, 2007                [Page 42]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

acknowledgment, as long as the required message is also sent within

some peer-defined amount of time.  This allows the acknowledging peer

to delay transmission of the proper IAX2 message, which may add

security against brute-force password attacks during authentication

exchanges.

When the following messages are received, an ACK MUST be sent in

return: NEW, HANGUP, REJECT, ACCEPT, PONG, AUTHREP, REGREL, REGACK,

REGREJ, TXREL.  ACKs SHOULD not be expected by any peer and their

purpose is purely to force the transport layer to be up to date.

The ACK message does not requires any IEs.

6.11.2.  INVAL Invalid Response Message

An INVAL is sent as a response to a received message that is not

valid.  This occurs when an IAX2 peer sends a message on a call after

the remote peer has hungup its end.  Upon receipt of an INVAL, a peer

MUST destroy its side of a call.

The INVAL message does not requires any IEs.

6.11.3.  VNAK Voice Negative Acknowledgement Message

A VNAK is sent when a message is received out of order, particularly

when a mini frame is received before the first full voice frame on a

call.  It is a request for retransmission of dropped messages.  A

message is considered out of sequence if the received iseqno is

different than the expected iseqno.  On receipt of a VNAK, a peer

MUST retransmit all frames with a higher sequence number than the

VNAK message's iseqno.

The VNAK message does not requires any IEs.

6.11.4.  MWI Message Waiting Indicator Request Message

An MWI message is used to indicate to a remote peer that it has one

or more messages waiting.  It MAY include the 'msgcount' IE to

specify how many messages are waiting.

The following table specifies IEs used by this message

+----------+----------------+----------+------------+

| IE       | Section        | Status   | Comments   |

+----------+----------------+----------+------------+

| MSGCOUNT | Section 8.6.23 | Optional | Suggested. |

+----------+----------------+----------+------------+

Spencer, et al.          Expires April 26, 2007                [Page 43]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.11.5.  UNSUPPORT Unsupported Response Message

An UNSUPPORT message is sent in response to a message that is not

supported by an IAX2 peer.  This occurs when an IAX2 command with an

unrecognized or unsupported subclass is received.  No action is

required upon receipt of this message, though the peer SHOULD be

aware that the message referred to in the optionally included 'IAX2

unknown' IE is not supported by the remote peer.

The following table specifies IEs used by this message

+---------+----------------+----------+------------+

| IE      | Section        | Status   | Comments   |

+---------+----------------+----------+------------+

| UNKNOWN | Section 8.6.22 | Optional | Suggested. |

+---------+----------------+----------+------------+

6.12.  Media Messages

The IAX2 protocol supports many types of media and these are

transported through the same UDP port as the other messages.  Voice

and video are unique in that they utilize two different encodings

which use two different procedures to support.  Abbreviated 'Mini

frames' are normally used for audio and video, however, each time the

time-stamp is a multiple of 32,768 (0x8000 hex) a standard or 'Full

Frame' MAY be sent.  This approach facilitates efficiency and

reliability by sending compressed messages without guaranteed

delivery packets most of the time and periodically having reliable

exchanges with the peer which invokes call tear down procedures if

communication is no longer successful.

Upon receiving any media message, except the abbreviated audio and

video mini frames, an ACK message MUST be sent.  The content SHOULD

be passed to the application or further to the next call leg.  The

data MAY be buffered before it is presented to the user.

6.12.1.  DTMF Media Message

The message carries a single digit of DTMF (Dual Tone Multiple

Frequency).  Useful background information about DTMF can be found in

[RFC2833], but, note that IAX2 does not use the RTP protocol.

6.12.2.  Voice Media Message

The message carries voice data and indicates the CODEC used.

Spencer, et al.          Expires April 26, 2007                [Page 44]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

6.12.3.  Video Media Message

The frame carries video data and indicates the video format of the

data.

6.12.4.  Text Media Message

The frame carries a text message.

6.12.5.  Image Media Message

This message carries a single image.  The image MUST fit in one

message in this version of the protocol.

6.12.6.  HTML Media Message

The HTML message class carries HTML and related data as well as

status about the display of that HTML page.  The subclass parameter

indicates the HTML content type.  It MAY be a URL, the start, middle

or end of a data block.

If a peer receives an HTML message for a channel that does not

support HTML, it MUST respond with an HTML message that has the HTML

NOT SUPPORTED indication.

When a devices that supports HTML completes loading the page, it

SHOULD send a LOAD COMPLETE message

6.12.7.  Comfort Noise  Media Message

This message indicates that comfort noise SHOULD be played.  It has a

parameter that indicates the level.  The noise is to be locally

generated.

Spencer, et al.          Expires April 26, 2007                [Page 45]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

7.  Message Transport

IAX2 is sent over UDP and uses an application level protocol to

provide reliable transport where needed.

With respect to transport, there are two messages formats: reliable

or 'Full Frames' and unacknowledged 'Mini' or 'Meta' frames.  All

messages except certain voice and video messages are reliable.

Reliable messages are transported by a scheme which maintains message

counts and time stamps for both peers involved in the call.  The

counts are per call.  Each peer maintains a timer for all reliable

messages and MUST periodically retransmit those messages until they

acknowledge or the retry limit is exceeded.

When starting a call, the outgoing and incoming messages sequence

numbers MUST both be set to zero.  Each reliable message that is sent

increments the message count by one except the ACK, INVAL, TXCNT,

TXACC, and VNAK messages which do not change the message count.  The

message includes the outgoing message count and the highest numbered

incoming message which has been received.  In addition, it contains a

time-stamp which represents the number of milliseconds since the call

started.  Or, in the case of certain network timing messages, it

contains a copy of the time-stamp sent to it.  Timestamps MAY be

approximate, but, MUST be in order.

When any message is received, the timestamps in MUST be checked to

make sure that they are in order.  If a message is received out of

order, it MUST be ignored and a VNAK message sent to resynchronize

the peers.  And if the message is a reliable message, the incoming

message counter MUST be used to acknowledge all the messages up to

that sequence number which have been sent.

If no acknowledgment is received after a locally configured number of

retries, default 4, the call leg SHOULD be considered unusable and

the call MUST be torn down without any further interaction on this

call leg.

7.1.  Trunking

IAX2 allows multiple media exchanges between the same two peers to be

multiplexed into a single trunk call coalescing media payload into a

combined packet.  This decreases bandwidth usage as there are fewer

total packets being transmitted.  Trunking MAY occur in one or both

directions of an IAX2 exchange.  A trunk consists of a trunk header

and one or more trunked IAX2 calls.  The trunk message contains a

time-stamp specifying the time of transmission of the trunk frame.

The audio data from the trunked calls are encapsulated in the trunk

frame following the header.  Each trunked call consists of two octets

Spencer, et al.          Expires April 26, 2007                [Page 46]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

specifying the call's source number, two octets specifying the length

in octets of the media data, and the media data itself.  IAX2 permits

transmitting the timestamps of each encapsulated mini frame as well,

so that accurate timing information can be used for jitter buffers,

etc.  A flag in the meta command header specifies whether the

encapsulated mini frames retain their original timestamps.  If they

do not retain them, they MUST assume the time-stamp in the trunk

header upon being received by the trunk peer.

7.2.  Timers

There are various timers in the the IAX2 protocol.  There are other

application level timers such as the call timers and ring timer which

are beyond the scope of this document.  This section describes the

IAX2 timers and specifies their default values and behavior.

7.2.1.  Retransmission Timer

The message retransmission procedures are described in section

Section 7.  On each call, there is a timer for how long to wait for

an acknowledgment of a message.  This timer starts at twice the

measured round trip time from the last PING/PONG command.  If a

retransmission is needed, it is exponentially increased until it

meets a boundary value.  The maximum retry time period boundary is 10

seconds.

7.2.2.  Registration Period Timer

Registrations are valid for a specified time period.  It is the

client's responsibility to renew this registration before the time

period expires.  The registrations SHOULD be renewed at random

intervals to prevent network congestion.  A registrar MUST monitor

this time period and invalidate the registration if the client/

registrant has not renewed their registration before the timer

elapses.

7.3.  NAT Considerations

IAX2 is very well suited to operating behind NAT due to its single

port approach.  This approach eliminates any start of call media

stream delays while the NAT gateway establishes a bidirectional port

association.  Deploying a single IAX2 server behind a NAT gateway

requires little effort.  If the server acts as a registrar, the IAX2

UDP port on the NAT gateway must be forwarded to the server.  If the

server acts as a registrant, the default, 60 second, REGREQ refresh

timer should be sufficient to maintain a port association in the NAT

gateway, however, a static port mapping is preferred.

Spencer, et al.          Expires April 26, 2007                [Page 47]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

If multiple servers are to be deployed behind a single NAT gateway,

most NAT gateways require each IAX2 server to use different UDP

ports.  Of course, there may be NAT implementations which recognize

when multiple devices utilize the same private port and and manage it

appropriately.

7.4.  Encryption

IAX2 supports call encryption using the symmetric key, Rijndael [AES]

block cipher (also called AES---Advanced Encryption Standard).

Rijndael is a 128-bit block cipher utilizing a shared secret.  IAX2

encrypts on a call-by-call basis starting with a plain-text NEW

message indicating, in addition to the other message parameters, that

the call should be encrypted.  This indication is given by sending

the ENCRYPTION IE Section 8.6.42 in the NEW request message.  If the

called host supports encryption, it will respond with a plain-text

AUTHREQ message which also includes the ENCRYPTION IE.  All

subsequent messages in the call MUST be encrypted.  If the called

host does not support encryption, the AUTHREQ sent in response to the

NEW must not include the ENCRYPTION IE and the calling host MUST

either HANGUP the request or continue with the unencrypted call.

The key to use in encrypting the messages is computed by taking the

the CHALLENGE IE Section 8.6.14 from the AUTHREQ and concatenating

any one of the shared passwords then computing the 128-bit MD5 digest

of this combination.  To decrypt, if there is more than password for

the peer, each must be tried until the message is successfully

decoded.  The key remains constant for the duration of the call.

Only the data portion of the messages are encoded.

Spencer, et al.          Expires April 26, 2007                [Page 48]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.  Message Encoding

This section contains the specification for each type of frame that

IAX2 defines.

8.1.  Full Frames

Full frames can send signaling or media data.  Generally full frames

are used to control initiation, setup, and termination of an IAX2

call, but they can also be used to carry stream data (though this is

generally not optimal).

Full frames are sent reliably, so all full frames require an

immediate acknowledgment upon receipt.  This acknowledgment can be

explicit via an 'ACK' message (see Section 8.4.12) or implicit based

upon receipt of an appropriate response to the full frame issued.

The standard full frame header length is 12 octets.

Field descriptions:

'F' bit

This bit specifies whether the frame is a full frame or not.  If

the 'F' bit is set to 1 the frame is a full frame.  If it is set

to 0 it is not a full frame.

Source call number

This 15-bit value specifies the call number the transmitting

client uses to identify this call.  The source call number for an

active call MUST not be in use by another call on the same client.

Call numbers MAY be reused once a call is no longer active, i.e.

when either there is positive acknowledgment that the call has

been destroyed or when all possible timeouts for the call have

expired.

'R' bit

This bit specifies whether the frame is being retransmitted or

not.  If the 'R' bit is set to 0 the frame is being transmitted

for the first time.  If it is set to 1 the frame is being

retransmitted.  IAX2 does not specify a retransmit timeout; this

is left to the implementor.

Destination call number

Spencer, et al.          Expires April 26, 2007                [Page 49]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

This 15-bit value specifies the call number the transmitting

client uses to reference the call at the remote peer.  This number

is the same as the remote peer's source call number.  The

destination call number uniquely identifies a call on the remote

peer.  The source call number uniquely identifies the call on the

local peer.

time-stamp

The time-stamp field contains a 32-bit time-stamp maintained by an

IAX2 peer for a given call.  The time-stamp is an incrementally

increasing representation of the number of milliseconds since the

first transmission of the call.

OSeqno

The 8-bit OSeqno field is the outbound stream sequence number.

Upon initialization of a call its value is 0.  It increases

incrementally as full frames are sent.  When the counter

overflows, it silently resets to 0.

ISeqno

The 8-bit ISeqno field is the inbound stream sequence number.

Upon initialization of a call its value is 0.  It increases

incrementally as full frames are received.  At any time the ISeqno

of a call represents the next expected inbound stream sequence

number.  When the counter overflows, it silently resets to 0.

Frametype

The Frametype field identifies the type of message carried by the

frame.  See Section 8.4 for more information.

'C' bit

This bit determines how the remaining 7 bits of the Subclass field

are coded.  If the 'C' bit is set to 1, the Subclass value is

interpreted as a power of 2.  If it is not set, the Subclass value

is interpreted as a simple seven bit unsigned integer.

Spencer, et al.          Expires April 26, 2007                [Page 50]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|     Source Call Number      |R|   Destination Call Number   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                            time-stamp                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    OSeqno     |    ISeqno     |   Frame Type  |C|  Subclass   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

:                             Data                              :

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 5: Full Frame Binary Format

8.2.  Mini frames

Mini Frames are so named because their header is a minimal 4 octets.

Mini frames carry no control or signaling data; their sole purpose is

to carry a media stream on an already-established IAX2 call.  They

are sent unreliably.  This decision was made because VoIP calls

typically can miss several frames without significant degradation in

call quality while the incurred overhead in ensuring reliability

increases bandwidth requirements and decreases throughput.  Further,

because voice calls are typically sent in real time, lost frames are

too old to be reintegrated into the audio stream by the time they can

be retransmitted.

Field descriptions:

'F' bit

Mini frames MUST have the 'F' bit set to 0 to specify that they

are not full frames.

Source call number

The source call number is the number that is used by the

transmitting peer to identify the current call.

time-stamp

Mini frames carry a 16-bit time-stamp, which is the lower 16 bits

of the transmitting peer's full 32-bit time-stamp for the call.

The time-stamp allows synchronization of incoming frames so that

they MAY be processed in chronological order instead of the

(possibly different) order in which they are received.  The 16-bit

Spencer, et al.          Expires April 26, 2007                [Page 51]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

time-stamp wraps after 65.536 seconds, at which point a full frame

SHOULD be sent to notify the remote peer that its time-stamp has

been reset.  A call MUST continue to send mini frames starting

with time-stamp 0 even if acknowledgment of the resynchronization

is not received.

The F bit, source call number, and 16-bit time-stamp comprise the

entire four octet header for a full frame.  Following this header is

the actual stream data, of arbitrary length up to the maximum

supported by the network.

Mini frames are implicitly defined to be of type 'voice frame'

(frametype 2; see Section 8.4).  The subclass is implicitly defined

by the most recent full voice frame of a call (i.e. the subclass for

a voice frame specifies the CODEC used with the stream).  The first

voice frame of a call SHOULD be sent using the CODEC agreed upon in

the initial CODEC negotiation.  On-the-fly CODEC negotiation is

permitted by sending a full voice frame specifying the new CODEC to

use in the subclass field.

1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|     Source call number      |            time-stamp         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

:                             Data                              :

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 6: Mini Frame Binary Format

8.3.  Meta frames

Meta frames serve one of two purposes.  Meta video frames allow the

transmission of video streams with an optimized header.  They are

similar in purpose to mini voice frames.  Meta trunk frames are used

for trunking multiple IAX2 media streams between two peers into one

header, to further minimize bandwidth consumption.

8.3.1.  Meta Video Frames

Field descriptions:

'F' bit

Spencer, et al.          Expires April 26, 2007                [Page 52]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

Meta video frames MUST have the 'F' bit set to 0 to indicate that

they are not full frames.

Meta Indicator

The meta indicator is a 15-bit field of all zeroes, used to

indicate that the frame is a meta frame.  Meta frames are

identifiable because the first 16 bits will always be zero in any

meta frame, whereas full or mini frames will have either the 'F'

bit set or some (nonzero) value for the source call number (or

both).

'V' bit

The 'V' bit in a meta video frame is set to 1 to specify that the

frame is a meta video frame.

Source call number

The call number that is used by the transmitting peer to identify

this video call.

time-stamp

Meta video frames carry a 16-bit time-stamp, which is the lower 16

bits of the transmitting peer's full 32-bit time-stamp for the

call.  When this time-stamp wraps, a full frame SHOULD be sent to

notify the remote peer that the time-stamp has been reset to 0.

Following the time-stamp is the actual video stream data.  Meta video

frames are implicitly defined to be of type 'video frame' (frametype

3; see Section 8.4).  The video CODEC used is implicitly defined by

the subclass of the most recent full video frame of a call.

1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|         Meta Indicator      |V|      Source Call Number     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|?|          time-stamp         |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+                               |

|                                         Data                  |

:                                                               :

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 7: Meta Video Frame Binary Format

Spencer, et al.          Expires April 26, 2007                [Page 53]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.3.2.  Meta Trunk Frames

IAX2 natively supports two methods of trunking multiple media streams

between two peers into a single association.  The first method sends

a standard meta header, along with a single 32-bit time-stamp

describing the transmission time of the trunk frame.  Following the

time-stamp are one or more media frames consisting of the call number

and the length in octets of the stream data included in the frame.

The second method of trunking is very similar to the first.  It sends

a standard meta header, including the 32-bit time-stamp describing

the time of transmission of the trunk frame.  But the media frames

included in the trunk are actually complete mini frames, including

the 16-bit time-stamp for each call.  The first method uses slightly

less bandwidth (2 fewer octets per call in the trunk), while the

second method maintains the individual timestamps for each call so

that jitterbuffering can use the actual timestamps associated with a

call instead of the (less accurate) time-stamp representing the

entire trunk.  Either method is permissible for trunking.

Field descriptions:

'F' bit

Meta trunk frames MUST have the 'F' bit set to 0 to indicate that

they are not full frames.

Meta Indicator

The meta indicator is a 15-bit field of all zeroes, used to

indicate that the frame is a meta frame.  Meta frames are

identifiable because the first 16 bits will always be zero in any

meta frame, whereas full or mini frames will have either the 'F'

bit set or some (nonzero) value for the source call number (or

both).

'V' bit

The 'V' bit in a meta trunk frame is set to 0 to specify that the

frame is not a meta video frame.

Meta Command

This seven bit field identifies whether the meta frame is a trunk

or not.  A value of '1' indicates that the frame is a meta trunk

frame.  All other values are reserved for future use.

Command Data

Spencer, et al.          Expires April 26, 2007                [Page 54]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

This 8-bit field specifies flags for options which apply to a

trunked call.  The least significant bit of the field is the

'trunk timestamps' flag.  A value of 0 indicates that the calls in

the trunk do not include their individual timestamps.  A value of

1 indicates that the calls do each include their own time-stamp.

All other bits are reserved for future use.

time-stamp

Meta trunk frames carry a 32-bit time-stamp, which represents the

actual time of transmission of the trunk frame.  This is distinct

from the timestamps of the calls included in the trunk.

Following the 32-bit time-stamp is one or more trunked calls.  If the

'trunk timestamps' flag is set to 0, each entry consists of 2 octets

specifying the source call number of the call, 2 octets specifying

the length in octets of the media data, and then the media data.  If

the 'trunk timestamps' flag is set to 1, each entry consists of 2

octets specifying the length in octets of the media data, and then a

mini frame (2 octets specifying source call number, 2 octets

specifying 16-bit time-stamp, and the media data).  The following 2

diagrams help illustrate this pictorially.

1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|         Meta Indicator      |V|Meta Command | Cmd Data (0)  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                            time-stamp                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|R|      Source Call Number     |     Data Length (in octets)   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

:                             Data                              :

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

.

.

.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|R|      Source Call Number     |     Data Length (in octets)   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                                                               |

:                             Data                              :

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 55]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

Figure 8: Meta Trunk Frame Binary Format (trunk timestamps 0)

1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|F|         Meta Indicator      |V|Meta Command | Cmd Data (1)  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                            time-stamp                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Data Length (in octets)   |R|     Source Call Number      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           time-stamp          |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+                               |

|                                       Data                    |

:                                                               :

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

.

.

.

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Data Length (in octets)   |R|     Source Call Number      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           time-stamp          |                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+                               |

|                                       Data                    |

:                                                               :

|                                                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Figure 9: Meta Trunk Frame Binary Format (trunk timestamps 1)

8.4.  Frame Types

The IAX2 protocol specifies 10 types of possible frames for the

"frametype" field of a full frame.  They are:

8.4.1.  DTMF Frame

The frame carries a single digit of DTMF (Dual Tone Multiple

Frequency).  More information about DTMF can be found in RFC

2833[RFC2833].

For DTMF frames, the subclass is the actual DTMF digit carried by the

frame.

Spencer, et al.          Expires April 26, 2007                [Page 56]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.4.2.  Voice Frame

The frame carries voice data.

The subclass specifies the audio format of the data.  Predefined

voice formats can be found in Section 8.7 below.

8.4.3.  Video Frame

The frame carries video data.

The subclass specifies the video format of the data.  Predefined

video formats can be found in Section 8.7 below.

8.4.4.  Control Frame

The frame carries session control data, i.e. it refers to control of

a device connected to an IAX2 endpoint.

The subclass is a value from Section 8.4.11 describing the device

control signal.

8.4.5.  Null Frame

Frames with the Null value MUST not be transmitted.

8.4.6.  IAX2 Frame

The frame carries control data that provides IAX2 protocol specific

endpoint management.  This frametype is used to manage IAX2 protocol

interactions that are generally independent of the type of endpoints.

The subclass is a value from Section 8.4.12 describing an IAX2 event.

8.4.7.  Text Frame

The frame carries a non-control text message.

All text frames have a subclass of 0.

8.4.8.  Image Frame

The frame carries a single image.

The subclass describes the format of the image from Section 8.7

below.

Spencer, et al.          Expires April 26, 2007                [Page 57]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.4.9.  HTML Frame

The frame carries HTML data.

The subclass is a value from the HTML Subclasses table in

Section 8.5.

8.4.10.  Comfort Noise Frame

The frame carries comfort noise.

The subclass is the level of comfort noise in -dBov.

The following table specifies all valid Frame Type Values:

+------+-------------+---------------------------+------------------+

| TYPE | Description | Subclass Description      | Data Description |

+------+-------------+---------------------------+------------------+

| 0x01 | DTMF        | 0-9, A-D, *, #            | Undefined        |

|      |             |                           |                  |

| 0x02 | Voice       | Audio Compression Format  | Data             |

|      |             |                           |                  |

| 0x03 | Video       | Video Compression Format  | Data             |

|      |             |                           |                  |

| 0x04 | Control     | See Control Frame Types   | Varies with      |

|      |             |                           | subclass         |

|      |             |                           |                  |

| 0x05 | Null        | Undefined                 | Undefined        |

|      |             |                           |                  |

| 0x06 | IAX2        | See IAX2 Protocol         | Information      |

|      | Control     | Messages                  | Elements         |

|      |             |                           |                  |

| 0x07 | Text        | Always 0                  | Raw Text         |

|      |             |                           |                  |

| 0x08 | Image       | Image Compression Format  | Raw image        |

|      |             |                           |                  |

| 0x09 | HTML        | See HTML Frame Types      | Message Specific |

|      |             |                           |                  |

| 0x0A | Comfort     | Level in -dBov of comfort | None             |

|      | Noise       | noise                     |                  |

+------+-------------+---------------------------+------------------+

Spencer, et al.          Expires April 26, 2007                [Page 58]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.4.11.  Control Frames

The following table specifies all valid Control Frame Subclasses:

+-------------+--------------+--------------------------------------+

| VALUE       | Name         | Description                          |

+-------------+--------------+--------------------------------------+

| 0x01        | Hangup       | The call has been hungup at the      |

|             |              | remote end.                          |

|             |              |                                      |

| 0x02        | Reserved     | Reserved for future use              |

|             |              |                                      |

| 0x03        | Ringing      | Remote end is ringing (ringback)     |

|             |              |                                      |

| 0x04        | Answer       | Remote end has answered              |

|             |              |                                      |

| 0x05        | Busy         | Remote end is busy                   |

|             |              |                                      |

| 0x06        | Reserved     | Reserved for future use              |

|             |              |                                      |

| 0x07        | Reserved     | Reserved for future use              |

|             |              |                                      |

| 0x08        | Congestion   | The call is congested.               |

|             |              |                                      |

| 0x09        | Flash Hook   | Flash hook                           |

|             |              |                                      |

| 0x0a        | Reserved     | Reserved for future use              |

|             |              |                                      |

| 0x0b        | Option       | Device-specific options are being    |

|             |              | transmitted                          |

|             |              |                                      |

| 0x0c        | Key Radio    | Key Radio                            |

|             |              |                                      |

| 0x0d        | Unkey Radio  | Unkey Radio                          |

|             |              |                                      |

| 0x0e        | Call         | Call is in progress                  |

|             | Progress     |                                      |

|             |              |                                      |

| 0x0f        | Call         | Call is proceeding                   |

|             | Proceeding   |                                      |

|             |              |                                      |

| 0x10        | Hold         | Call is placed on hold               |

|             |              |                                      |

| 0x11        | Unhold       | Call is taken off hold               |

+-------------+--------------+--------------------------------------+

Spencer, et al.          Expires April 26, 2007                [Page 59]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.4.12.  IAX2 Frames

Frames of type 'IAX2' are used to provide management of IAX2

endpoints.  They handle IAX2 signaling (e.g. call setup, maintenance,

and teardown).  They MAY also handle direct transmission of media

data, but this is not optimal for VoIP calls.  They do not carry

session-specific control (e.g. device state), as this is the purpose

of Control Frames.  The IAX2 commands are listed and described below.

The following table specifies all valid IAX2 Frame Values:

+------+-----------+-----------------------------------------+

| Hex  | Name      | Description                             |

+------+-----------+-----------------------------------------+

| 0x01 | NEW       | Initiate a new call                     |

|      |           |                                         |

| 0x02 | PING      | Ping request                            |

|      |           |                                         |

| 0x03 | PONG      | Ping or poke reply                      |

|      |           |                                         |

| 0x04 | ACK       | Explicit acknowledgment                 |

|      |           |                                         |

| 0x05 | HANGUP    | Initiate call teardown                  |

|      |           |                                         |

| 0x06 | REJECT    | Reject a call                           |

|      |           |                                         |

| 0x07 | ACCEPT    | Accept a call                           |

|      |           |                                         |

| 0x08 | AUTHREQ   | Authentication request                  |

|      |           |                                         |

| 0x09 | AUTHREP   | Authentication reply                    |

|      |           |                                         |

| 0x0a | INVAL     | Invalid message                         |

|      |           |                                         |

| 0x0b | LAGRQ     | Lag request                             |

|      |           |                                         |

| 0x0c | LAGRP     | Lag reply                               |

|      |           |                                         |

| 0x0d | REGREQ    | Registration request                    |

|      |           |                                         |

| 0x0e | REGAUTH   | Registration authentication             |

|      |           |                                         |

| 0x0f | REGACK    | Registration acknowledgement            |

|      |           |                                         |

| 0x10 | REGREJ    | Registration reject                     |

|      |           |                                         |

| 0x11 | REGREL    | Registration release                    |

|      |           |                                         |

Spencer, et al.          Expires April 26, 2007                [Page 60]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

| 0x12 | VNAK      | Video/Voice retransmit request          |

|      |           |                                         |

| 0x13 | DPREQ     | Dialplan request                        |

|      |           |                                         |

| 0x14 | DPREP     | Dialplan reply                          |

|      |           |                                         |

| 0x15 | DIAL      | Dial                                    |

|      |           |                                         |

| 0x16 | TXREQ     | Transfer request                        |

|      |           |                                         |

| 0x17 | TXCNT     | Transfer connect                        |

|      |           |                                         |

| 0x18 | TXACC     | Transfer accept                         |

|      |           |                                         |

| 0x19 | TXREADY   | Transfer ready                          |

|      |           |                                         |

| 0x1a | TXREL     | Transfer release                        |

|      |           |                                         |

| 0x1b | TXREJ     | Transfer reject                         |

|      |           |                                         |

| 0x1c | QUELCH    | Halt audio/video [media] transmission   |

|      |           |                                         |

| 0x1d | UNQUELCH  | Resume audio/video [media] transmission |

|      |           |                                         |

| 0x1e | POKE      | Poke request                            |

|      |           |                                         |

| 0x1f | Reserved  | Reserved for future use                 |

|      |           |                                         |

| 0x20 | MWI       | Message waiting indication              |

|      |           |                                         |

| 0x21 | UNSUPPORT | Unsupported message                     |

|      |           |                                         |

| 0x22 | TRANSFER  | Remote transfer request                 |

|      |           |                                         |

| 0x23 | PROVISION | Provision an IAX2 device                |

|      |           |                                         |

| 0x24 | FWDOWNL   | Download firmware request               |

|      |           |                                         |

| 0x25 | FWDATA    | Transmit firmware data                  |

+------+-----------+-----------------------------------------+

Spencer, et al.          Expires April 26, 2007                [Page 61]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.5.  HTML Command Subclasses

IAX2 HTML Command Subclasses:

+--------+----------------------------+

| NUMBER | DESCRIPTION                |

+--------+----------------------------+

| 1      | Sending a URL              |

|        |                            |

| 2      | Data frame                 |

|        |                            |

| 4      | Beginning frame            |

|        |                            |

| 8      | End frame                  |

|        |                            |

| 16     | Load is complete           |

|        |                            |

| 17     | Peer does not support HTML |

|        |                            |

| 18     | Link URL                   |

|        |                            |

| 19     | Unlink URL                 |

|        |                            |

| 20     | Reject Link URL            |

+--------+----------------------------+

8.6.  Information Elements

IAX2 messages sent as full frames MAY carry information elements to

specify user- or call-specific data.  Information elements are

appended to a frame header in its data field.  Zero, one, or multiple

information elements MAY be included with any IAX2 message.

Information elements are coded as follows:

The first octet of any information element consists of the "IE"

field.  The IE field is an identification number which defines the

particular information element.  Table 31 lists the defined

information elements and each information element is defined below

the table.

The second octet of any information element is the "data length"

field.  It specifies the length in octets of the information

element's data field.

The remaining octet(s) of an information element contain the

actual data being transmitted.  The representation of the data is

dependent on the particular information element as identified by

Spencer, et al.          Expires April 26, 2007                [Page 62]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

its "IE" field.  Some information elements carry binary data, some

carry UTF-8 [RFC3629] data, and some have no data field at all.

The data representation for each information element is described

below.

The following table specifies the Information Element Binary Format:

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      IE       |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:             DATA              :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Following is a table of the information elements IAX2 defines, and a

brief description of each information element's purpose.  More

information about each IE may be found below the table.

+------+-----------------+------------------------------------------+

| HEX  | NAME            | DESCRIPTION                              |

+------+-----------------+------------------------------------------+

| HEX  | NAME            | DESCRIPTION                              |

|      |                 |                                          |

| 0x01 | CALLED NUMBER   | Number/extension being called            |

|      |                 |                                          |

| 0x02 | CALLING NUMBER  | Calling number                           |

|      |                 |                                          |

| 0x03 | CALLING ANI     | Calling number ANI for billing           |

|      |                 |                                          |

| 0x04 | CALLING NAME    | Name of caller                           |

|      |                 |                                          |

| 0x05 | CALLED CONTEXT  | Context for number                       |

|      |                 |                                          |

| 0x06 | USERNAME        | Username (peer or user) for              |

|      |                 | authentication                           |

|      |                 |                                          |

| 0x07 | PASSWORD        | Password for authentication              |

|      |                 |                                          |

| 0x08 | CAPABILITY      | Actual CODEC capability                  |

|      |                 |                                          |

| 0x09 | FORMAT          | Desired CODEC format                     |

|      |                 |                                          |

Spencer, et al.          Expires April 26, 2007                [Page 63]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

| 0x0a | LANGUAGE        | Desired language                         |

|      |                 |                                          |

| 0x0b | VERSION         | Protocol version                         |

|      |                 |                                          |

| 0x0c | ADSICPE         | CPE ADSI capability                      |

|      |                 |                                          |

| 0x0d | DNID            | Originally dialed DNID                   |

|      |                 |                                          |

| 0x0e | AUTHMETHODS     | Authentication method(s)                 |

|      |                 |                                          |

| 0x0f | CHALLENGE       | Challenge data for MD5/RSA               |

|      |                 |                                          |

| 0x10 | MD5 RESULT      | MD5 challenge result                     |

|      |                 |                                          |

| 0x11 | RSA RESULT      | RSA challenge result                     |

|      |                 |                                          |

| 0x12 | APPARENT ADDR   | Apparent address of peer                 |

|      |                 |                                          |

| 0x13 | REFRESH         | When to refresh registration             |

|      |                 |                                          |

| 0x14 | DPSTATUS        | Dialplan status                          |

|      |                 |                                          |

| 0x15 | CALLNO          | Call number of peer                      |

|      |                 |                                          |

| 0x16 | CAUSE           | Cause                                    |

|      |                 |                                          |

| 0x17 | IAX2 UNKNOWN    | Unknown IAX2 command                     |

|      |                 |                                          |

| 0x18 | MSGCOUNT        | How many messages waiting                |

|      |                 |                                          |

| 0x19 | AUTOANSWER      | Request auto-answering                   |

|      |                 |                                          |

| 0x1a | MUSICONHOLD     | Request musiconhold with QUELCH          |

|      |                 |                                          |

| 0x1b | TRANSFERID      | Transfer Request Identifier              |

|      |                 |                                          |

| 0x1c | RDNIS           | Referring DNIS                           |

|      |                 |                                          |

| 0x1d | PROVISIONING    | Provisioning info                        |

|      |                 |                                          |

| 0x1e | AESPROVISIONING | AES Provisioning info                    |

|      |                 |                                          |

| 0x1f | DATETIME        | Date/Time                                |

|      |                 |                                          |

| 0x20 | DEVICETYPE      | Device Type                              |

|      |                 |                                          |

| 0x21 | SERVICEIDENT    | Service Identifier                       |

|      |                 |                                          |

Spencer, et al.          Expires April 26, 2007                [Page 64]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

| 0x22 | FIRMWAREVER     | Firmware revision                        |

|      |                 |                                          |

| 0x23 | FWBLOCKDESC     | Firmware block description               |

|      |                 |                                          |

| 0x24 | FWBLOCKDATA     | Firmware block of data                   |

|      |                 |                                          |

| 0x25 | PROVVER         | Provisioning Version                     |

|      |                 |                                          |

| 0x26 | CALLINGPRES     | Calling presentation                     |

|      |                 |                                          |

| 0x27 | CALLINGTON      | Calling type of number                   |

|      |                 |                                          |

| 0x28 | CALLINGTNS      | Calling transit network select           |

|      |                 |                                          |

| 0x29 | SAMPLINGRATE    | Supported sampling rates                 |

|      |                 |                                          |

| 0x2a | CAUSECODE       | Hangup cause                             |

|      |                 |                                          |

| 0x2b | ENCRYPTION      | Encryption format                        |

|      |                 |                                          |

| 0x2c | ENCKEY          | 128-bit AES encryption key               |

|      |                 |                                          |

| 0x2d | CODEC PREFS     | CODEC Negotiation                        |

|      |                 |                                          |

| 0x2e | RR JITTER       | Received jitter, as in rfc1889           |

|      |                 |                                          |

| 0x2f | RR LOSS         | Received loss, as in rfc1889             |

|      |                 |                                          |

| 0x30 | RR PKTS         | Received frames                          |

|      |                 |                                          |

| 0x31 | RR DELAY        | Max playout delay for received frames in |

|      |                 | ms                                       |

|      |                 |                                          |

| 0x32 | RR DROPPED      | Dropped frames (presumably by            |

|      |                 | jitterbuffer)                            |

|      |                 |                                          |

| 0x33 | RR OOO          | Frames received Out of Order             |

|      |                 |                                          |

| 0x34 | OSPTOKEN        | OSP token block                          |

+------+-----------------+------------------------------------------+

Table 31: Information Element Definitions

8.6.1.  CALLED NUMBER

The purpose of the CALLED NUMBER information element is to indicate

the number or extension being called.  It carries UTF-8-encoded data.

The CALLED NUMBER information element MUST use UTF-8 encoding and not

numeric data because it is not limited to E.164 numbers ([E164]),

national numbers, or even numbers.  It is possible for a number or

Spencer, et al.          Expires April 26, 2007                [Page 65]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

extension to include non-numeric characters.  A CALLED NUMBER MAY be

a SIP URI [RFC3261]

The CALLED NUMBER information element is generally sent with IAX2

NEW, DPREQ, DPREP, DIAL, and TRANSFER messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x01     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:  UTF-8-encoded CALLED NUMBER  :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.2.  CALLING NUMBER

The purpose of the CALLING NUMBER information element is to indicate

the number or extension of the calling entity to the remote peer.  It

carries UTF-8-encoded data.

The CALLING NUMBER information element is usually sent with IAX2 NEW

messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x02     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

: UTF-8-encoded CALLING NUMBER  :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.3.  CALLING ANI

The purpose of the CALLING ANI information element is to indicate the

calling number ANI (Automatic number identification) for billing.  It

carries UTF-8-encoded data.

The CALLING ANI information element MAY be sent with an IAX2 NEW

message, but it is not required.

Spencer, et al.          Expires April 26, 2007                [Page 66]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x03     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:   UTF-8-encoded CALLING ANI   :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.4.  CALLING NAME

The purpose of the CALLING NAME information element is to indicate

the calling name of the transmitting peer.  It carries UTF-8-encoded

data.

The CALLING NAME information element is usually sent with IAX2 NEW

messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x04     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:   UTF-8-encoded CALLING NAME  :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.5.  CALLED CONTEXT

The purpose of the CALLED CONTEXT information element is to indicate

the context in the remote peer's dialplan the call is trying to

reach.  It carries UTF-8-encoded data.

The CALLED CONTEXT information element MAY be sent with IAX2 NEW or

TRANSFER messages, though it is not required.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x05     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

: UTF-8-encoded CALLED CONTEXT  :

|                               |

Spencer, et al.          Expires April 26, 2007                [Page 67]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.6.  USERNAME

The purpose of the USERNAME information element is to specify the

identity of the user participating in an IAX2 message exchange.  It

carries UTF-8-encoded data.

The USERNAME information element MAY be sent with IAX2 NEW, AUTHREQ,

REGREQ, REGAUTH, or REGACK messages, or any time a peer needs to

identify a user.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x06     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:     UTF-8-encoded USERNAME    :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.7.  CAPABILITY

The purpose of the CAPABILITY information element is to indicate the

media CODEC capabilities of an IAX2 peer.  Its data is represented in

a four octet bitmask according to Section 8.7.  Multiple CODECs MAY

be specified by logically OR'ing them into the CAPABILITY information

element.

The CAPABILITY information element is sent with IAX2 NEW messages if

appropriate for the CODEC negotiation method the peer is using.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x08     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| CAPABILITY according to Media |

| Format Subclass Values Table  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.8.  FORMAT

The purpose of the FORMAT information element is to indicate a single

preferred media CODEC.  When sent with a NEW message, the indicated

Spencer, et al.          Expires April 26, 2007                [Page 68]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

CODEC is the desired CODEC an IAX2 peer wishes to use for a call.

When sent with an ACCEPT message, it indicates the actual CODEC that

has been selected for the call.  Its data is represented in a four

octet bitmask according to Section 8.7.  Only one CODEC MUST be

specified in the FORMAT information element.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x09     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   FORMAT according to Media   |

| Format Subclass Values Table  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.9.  LANGUAGE

The purpose of the LANGUAGE information element is to indicate the

language in which the transmitting peer would like the remote peer to

send signaling information.  It carries UTF-8-encoded data.

The LANGUAGE information element MAY be sent with an IAX2 NEW

message.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x0a     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:     UTF-8-encoded LANGUAGE    :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.10.  VERSION

The purpose of the VERSION information element is to indicate the

protocol version the peer is using.  Peers at each end of a call MUST

use the same protocol version.  Currently the only supported version

is 2.  The data field of the VERSION information element is 2 octets

long.

The VERSION information element MUST be sent with an IAX2 NEW

message.

When Sent, the VERSION information element MUST be the first IE in

Spencer, et al.          Expires April 26, 2007                [Page 69]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

the message.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x0b     |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|            0x0002             |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.11.  ADSICPE

The purpose of the ADSICPE information element is to indicate the CPE

ADSI capability.  The data field of the ADSICPE information element

is 2 octets long.

The ADSICPE information element MAY be sent with an IAX2 NEW message.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x0c     |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       ADSICPE Capability      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.12.  DNID

The purpose of the DNID information element is to indicate the Dialed

Number ID, which may differ from the 'called number'.  It carries

UTF-8-encoded data.

The DNID information element MAY be sent with an IAX2 NEW message.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x0d     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:    UTF-8-encoded DNID Data    :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 70]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.6.13.  AUTHMETHODS

The purpose of the AUTHMETHODS information element is to indicate the

authentication methods a peer accepts.  It is sent as a bitmask 2

octets long.  The table below lists the valid authentication methods.

The AUTHMETHODS information element MUST be sent with IAX2 AUTHREQ

and REGAUTH messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x0e     |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Valid Authentication Methods |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The following table lists valid values for authentication:

+--------+--------------------------+

| METHOD | DESCRIPTION              |

+--------+--------------------------+

| 0x0001 | Reserved (was Plaintext) |

|        |                          |

| 0x0002 | MD5                      |

|        |                          |

| 0x0004 | RSA                      |

+--------+--------------------------+

8.6.14.  CHALLENGE

The purpose of the CHALLENGE information element is to offer the MD5

or RSA challenge to be used for authentication.  It carries the

actual UTF-8-encoded challenge data.

The CHALLENGE information element MUST be sent with IAX2 AUTHREQ and

REGAUTH messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x0f     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:  UTF-8-encoded Challenge Data :

|                               |

Spencer, et al.          Expires April 26, 2007                [Page 71]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.15.  MD5 RESULT

The purpose of the MD5 RESULT information element is to offer an MD5

response to an authentication CHALLENGE.  It carries the actual UTF-

8-encoded challenge result.

The MD5 RESULT information element MAY be sent with IAX2 AUTHREP and

REGREQ messages if an AUTHREQ or REGAUTH and appropriate CHALLENGE

has been received.  This information element MUST NOT be sent except

in response to a CHALLENGE.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     0x10      |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:    UTF-8-encoded MD5 Result   :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.16.  RSA RESULT

The purpose of the RSA RESULT information element is to offer an RSA

response to an authentication CHALLENGE.  It carries the actual UTF-

8-encoded challenge result.

The RSA RESULT information element MAY be sent with IAX2 AUTHREP and

REGREQ messages if an AUTHREQ or REGAUTH and appropriate CHALLENGE

have been received.  This information element MUST NOT be sent except

in response to a CHALLENGE.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x11     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:    UTF-8-encoded RSA Result   :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 72]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.6.17.  APPARENT ADDR

The purpose of the APPARENT ADDR information element is to indicate

the perceived network connection information used to reach a peer,

which may differ from the actual address when the peer is behind NAT.

The data field of the APPARENT ADDR information element is the same

as the POSIX sockaddr struct for the address family in use (i.e.

sockaddr_in for IPv4, sockaddr_in6 for IPv6).  The data length

depends on the type of address being represented.

The APPARENT ADDR information element MUST be sent with IAX2 TXREQ

and REGACK messages.  When used with a TXREQ message, the APPARENT

ADDR MUST specify the address of the peer the local peer is trying to

transfer its end of the connection to.  When used with a REGACK

message, the APPARENT ADDR MUST specify the address it uses to reach

the peer (which may be different than the address the peer perceives

itself as in the case of NAT or multi-homed peer machines).

The data field of the APPARENT ADDR information element is the same

as the linux struct sockaddr_in: two octets for the address family,

two octets for the port number, four octets for the IPv4 address, and

8 octets of padding consisting of all bits set to 0.  Thus the total

length of the APPARENT ADDR information element is 18 octets.

The following diagram demonstrates the generic APPARENT ADDR format:

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x12     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|        sockaddr struct        |

:   for address family in use   :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 73]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

The following diagram demonstrates the APPARENT ADDR format for an

IPv4 address:

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x12     |      0x10     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|            0x0200             | <- Address family (INET)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|            0x11d9             | <- Portno (default 4569)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      32-bit IP address        |

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

|      8 octets of all 0s       |

|   (padding in sockaddr_in)    |

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The following diagram demonstrates the APPARENT ADDR format for an

IPv6 address:

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x12     |      0x1C     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|            0x0A00             | <- Address family (INET6)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|            0x11d9             | <- Portno (default 4569)

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           32 bits             | <- Flow information

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      128-bit IP address       | <- Ip6 Address

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|           32 bits             | <- Scope ID

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 74]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.6.18.  REFRESH

The purpose of the REFRESH information element is to indicate the

number of seconds before an event expires.  Its data field is 2

octets long.

The REFRESH information element is used with IAX2 REGREQ, REGACK, and

DPREP messages.  When sent with a REGREQ it is a request that the

peer maintaining the registration set the timeout to REFRESH seconds.

When sent with a DPREP or REGACK, it is informational and tells a

remote peer when the local peer will no longer consider the event

valid.  The REFRESH sent with a DPREP tells a peer how long it SHOULD

store the received dialplan response.

If the REFRESH information element is not received with a DPREP, the

expiration of the cache data is assumed to be 10 minutes.  If the

REFRESH information element is not received with a REGACK,

registration expiration is assumed to occur after 60 seconds.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x13     |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  2 octets specifying refresh  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.19.  DPSTATUS

The purpose of the DPSTATUS information element is to indicate the

status of a CALLED NUMBER in a remote dialplan.  Its data field is a

two octet bitmask specifying flags from the table below.  Exactly one

of the low 3 bits MUST be set, and zero, one, or two of the high two

bits MAY be set.

The DPSTATUS information element MUST be sent with IAX2 DPREP

messages, as it is the payload of the dialplan response.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x14     |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|M|R|                     |N|C|E|

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 75]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

The following table lists the dialplan status flags:

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  FLAG     |  DESCRIPTION                  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  0x0001   |  Exists                       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  0x0002   |  Can exist                    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  0x0004   |  Non-existent                 |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  0x4000   |  Retain dialtone (ignorepat)  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  0x8000   |  More digits may match number |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.20.  CALLNO

The purpose of the CALLNO information element is to indicate the call

number a remote peer needs to use as a destination call number to

identify a call being transferred.  The peer managing a transfer

sends the CALLNO for one transfer endpoint to the other transfer

endpoint so that it knows what call number to specify for the

transfer.  The data field is 2 octets long and specifies a call

number in the same manner as a source call number or destination call

number is specified in a frame header.

The CALLNO information element MUST be sent with IAX2 TXREQ, TXREADY,

and TXREL messages.  Transferring cannot succeed if the CALLNO IE is

not included with the appropriate transfer messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     0x15      |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Callno of transfer recipient |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.21.  CAUSE

The purpose of the CAUSE information element is to indicate the

reason an event occurred.  It carries a description of the CAUSE of

the event as UTF-8-encoded data.  Notification of the event itself is

handled at the message level.

Spencer, et al.          Expires April 26, 2007                [Page 76]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

The CAUSE information element SHOULD be sent with IAX2 HANGUP,

REJECT, REGREJ, and TXREJ messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x16     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:  UTF-8-encoded CAUSE of event :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.22.  IAX2 UNKNOWN

The purpose of the IAX2 UNKNOWN information element is to indicate

that a received IAX2 command was unknown or unrecognized.  The one

octet data field contains the subclass of the received frame that was

unrecognized.

The IAX2 UNKNOWN information element MUST be sent with IAX2 UNSUPPORT

messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x17     |      0x01     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Rec'd Subclass|

+-+-+-+-+-+-+-+-+

8.6.23.  MSGCOUNT

The purpose of the MSGCOUNT information element is to indicate how

many voicemail messages are waiting in a registered user's mailbox.

The data field is 2 octets long.  If it is set to all 1s, there is at

least one message present.  Any other value specifies the number of

old messages in the high 8 bits and the number of new messages in the

low 8 bits.

The IAX2 MSGCOUNT information element MAY be sent with IAX2 REGACK

messages.

Spencer, et al.          Expires April 26, 2007                [Page 77]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x18     |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Old messages |  New messages |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.24.  AUTOANSWER

The purpose of the AUTOANSWER information element is to request that

a call be auto-answered upon receipt of a NEW message which includes

the AUTOANSWER information element.  Note that this is a request and

may or may not be granted by the remote peer.  There is no data field

with this information element, as its presence alone indicates all

necessary information.

The AUTOANSWER information element MAY be sent with IAX2 NEW

messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x19     |      0x00     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.25.  MUSICONHOLD

The purpose of the MUSICONHOLD information element is to request that

music-on-hold be played while a call is in the QUELCH state.  The

optional data field specifies a music-on-hold class to be used, as

UTF-8-encoded data.  In the absence of a data field, no music-on-hold

class is specified and the IE SHOULD be treated as a generic request

for music-on-hold.

The MUSICONHOLD information element MAY be sent with IAX2 QUELCH

messages.

If no MUSICONHOLD information element is received, music-on-hold is

not requested.

Spencer, et al.          Expires April 26, 2007                [Page 78]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x1a     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:  Optional Music On Hold Class :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.26.  TRANSFERID

The purpose of the TRANSFERID information element is to identify a

transfer across all three peers participating in a transfer event.

It carries a number, four octets long, that SHOULD be unique for the

duration of the transfer process.

The TRANSFERID information element SHOULD be sent with IAX2 TXREQ and

TXCNT messages to aid the peers involved in a transfer in identifying

the proper calls.  It is not required as long as the transferring

peers can positively identify the calls participating in the transfer

without the TRANSFERID.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x1b     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       4 octet transfer        |

|           identifier          |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.27.  RDNIS

The purpose of the RDNIS information element is to indicate the

referring DNIS.  It carries UTF-8-encoded data.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x1c     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:      UTF-8-encoded RDNIS      :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 79]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.6.28.  PROVISIONING

The purpose of the PROVISIONING information element is to carry

provisioning data.  It carries binary data used to provision an IAX2

device.  The coding scheme of the data included in the PROVISIONING

IE is unspecified by the protocol, and is left to the implementor,

however, it is strongly recommended that the provisioning data define

its version, format, etc., to enhance interoperability.

The PROVISIONING information element may only be sent with an IAX2

PROVISION message.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x1d     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:     Raw provisioning data     :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.29.  AESPROVISIONING

The purpose of the AESPROVISIONING information element is to carry

AES-encrypted provisioning data.  It carries encrypted data used to

provision an IAX2 device.  The coding scheme of the data included in

the AESPROVISIONING IE is unspecified by the protocol, as is left to

the implementor.

The AESPROVISIONING information element may only be sent with an IAX2

PROVISION message.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x1e     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:     AES provisioning data     :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 80]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.6.30.  DATETIME

The DATETIME information element indicates the time a message is

sent.  This differs from the header time-stamp because that time-

stamp begins at 0 for each call, while the DATETIME is a call-

independent value representing the actual real-world time.  The data

field of a DATETIME information element is four octets long and

stores the time as follows: The five least significant bits are

seconds, the next six least significant bits are minutes, the next

least significant five bits are hours, the next least significant

five bits are the day of the month, the next least significant four

bits are the month, and the most significant seven bits are the year.

The year is offset from 2000, and the month is a 1-based index (i.e.,

January == 1, February == 2, etc).  The timezone of the clock is

undefined but UTC is recommended.

The DATETIME information element SHOULD be sent with IAX2 NEW and

REGACK messages.  However, it is strictly informational.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x1f     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     year    | month |   day   |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  hours  |  minutes  | seconds |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.31.  DEVICETYPE

The purpose of the DEVICETYPE information element is to indicate the

type of device requesting registration or firmware downloading.  It

carries UTF-8-encoded data describing the type of device of the IAX2

peer.

The DEVICETYPE information element MUST be sent with IAX2 FWDOWNL

messages in order to help identify which firmware image is

appropriate for the peer.  It MAY be sent with IAX2 REGREQ messages,

as well.

If the initial FWDOWNL message of a call does not include the

DEVICETYPE information element, the firmware download fails and a

REJECT MUST be sent.  If at least one FWDOWNL has been sent with a

valid DEVICETYPE information element, subsequent FWDOWNL messages are

not required to include it.

Spencer, et al.          Expires April 26, 2007                [Page 81]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x20     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:   UTF-8-encoded device type   :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.32.  SERVICEIDENT

The SERVICEIDENT information element uniquely identifies a device

requesting provisioning so that appropriate data may be made

available.  The data field carries some guaranteed unique device

identifier, e.g. the six octet hardware MAC address of the device.

The SERVICEIDENT information element MAY be sent with IAX2 REGREQ

messages so that an IAX2 device may be provisioned appropriately.  It

is never required.

If the SERVICEIDENT information element is absent, a 'default'

template may be used to provision the device.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x21     |      0x06     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       Unique identifier       |

:       (e.g. peer's MAC)       :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.33.  FIRMWAREVER

The purpose of the FIRMWAREVER information element is to indicate the

version of firmware an IAX2 device has.  The data field is 2 octets

long and specifies a firmware version the server has available for

download for the received DEVICETYPE.  The meaning of this value

depends on the DEVICETYPE of the device.

The FIRMWAREVER information element may be sent with IAX2 REGACK

messages.

Spencer, et al.          Expires April 26, 2007                [Page 82]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x22     |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|       Firmware version        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.34.  FWBLOCKDESC

The purpose of the FWBLOCKDESC information element is to identify a

block of firmware.  Sent with an FWDOWNL it is a request for the

specified block of firmware.  Sent with an FWDATA it is a declaration

of the block of firmware included in the FWBLOCKDATA IE.  The data

field is 4 octets long and carries binary data.

The FWBLOCKDESC information element is sent with IAX2 FWDOWNL and

FWDATA messages.  It can be used to acknowledge the amount of data

transmitted or received.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x23     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         Firmware Block        |

|         Identification        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.35.  FWBLOCKDATA

The purpose of the FWBLOCKDATA information element is to carry a

block of a firmware image from a server to an IAX2 device.  The data

field carries raw binary data.

The FWBLOCKDATA information element is sent with IAX2 FWDATA

messages.  Once the firmware image is completely transmitted and

acknowledged (via the FWBLOCKDESC information element), a FWDATA

message containing a FWBLOCKDATA information element with no data

MUST be sent to indicate that the image has been completely

transmitted.

Spencer, et al.          Expires April 26, 2007                [Page 83]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x24     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

: Binary Block of Firmware Data :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.36.  PROVVER

The purpose of the PROVVER information element is to indicate the

provisioning version.  The data field is 4 octets long and contains

the version of the provisioning software used.

The PROVVER information element MAY be sent with IAX2 REGREQ

messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x25     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      Provisioning Version     |

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.37.  CALLINGPRES

The purpose of the CALLINGPRES information element is to indicate the

calling presentation of a caller.  The data field is 1 octet long and

contains a value from the table below.

The CALLINGPRES information element MAY be sent with IAX2 NEW

messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x26     |      0x01     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| Calling Pres. |

+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 84]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

The following table lists valid calling presentation values:

+------+--------------------------------------+

| FLAG | PRESENTATION                         |

+------+--------------------------------------+

| 0x00 | Allowed user/number not screened     |

|      |                                      |

| 0x01 | Allowed user/number passed screen    |

|      |                                      |

| 0x02 | Allowed user/number failed screen    |

|      |                                      |

| 0x03 | Allowed network number               |

|      |                                      |

| 0x20 | Prohibited user/number not screened  |

|      |                                      |

| 0x21 | Prohibited user/number passed screen |

|      |                                      |

| 0x22 | Prohibited user/number failed screen |

|      |                                      |

| 0x23 | Prohibited network number            |

|      |                                      |

| 0x43 | Number not available                 |

+------+--------------------------------------+

8.6.38.  CALLINGTON

The purpose of the CALLINGTON information element is to indicate the

calling type of number of a caller, according to ITU-T Recommendation

Q.931 specifications.  The data field is 1 octet long and contains

data from the table below.

The CALLINGTON information element MAY be sent with IAX2 NEW

messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x27     |      0x01     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Calling TON  |

+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 85]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

The following table lists valid calling type of number values from

ITU-T Recommendation Q.931:

+-------+-------------------------+

| VALUE | DESCRIPTION             |

+-------+-------------------------+

| 0x00  | Unknown                 |

|       |                         |

| 0x10  | International Number    |

|       |                         |

| 0x20  | National Number         |

|       |                         |

| 0x30  | Network Specific Number |

|       |                         |

| 0x40  | Subscriber Number       |

|       |                         |

| 0x60  | Abbreviated Number      |

|       |                         |

| 0x70  | Reserved for extension  |

+-------+-------------------------+

8.6.39.  CALLINGTNS

The CALLINGTNS information element indicates the calling transit

network selected for a call.  Values are chosen according to ITU-T

Recommendation Q.931 specifications.  The data field is two octets

long.  The first octet stores the network identification plan in the

least significant four bits according to the first table below, and

the type of network in the next three least significant bits

according to the second table below.  The second octet stores the

actual network identification in UTF-8-encoded data.

The CALLINGTNS information element MAY be sent with IAX2 NEW

messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x28     |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

| | TON | Plan  | UTF-8 Net ID  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The following tables list the valid values for the data field of the

'calling tns' IE.

Spencer, et al.          Expires April 26, 2007                [Page 86]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

Q.931 Network Identification Plan Values:

+------+----------------------------------+

| BITS | DESCRIPTION                      |

+------+----------------------------------+

| 0000 | Unknown                          |

|      |                                  |

| 0001 | Caller Identification Code       |

|      |                                  |

| 0011 | Data Network Identification Code |

+------+----------------------------------+

Q.931 Type of Network Values:

+------+--------------------------------------+

| BITS | DESCRIPTION                          |

+------+--------------------------------------+

| 000  | User Specified                       |

|      |                                      |

| 010  | National Network Identification      |

|      |                                      |

| 011  | International Network Identification |

+------+--------------------------------------+

8.6.40.  SAMPLINGRATE

The purpose of the SAMPLINGRATE information element is to specify to

a remote IAX2 peer the sampling rate in hertz of the audio data being

the peer will use when sending data.  Its data field is 2 octets

long.

If the SAMPLINGRATE information element is not specified, a default

sampling rate of 8 kHz may be assumed.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x29     |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|     Sampling Rate in Hertz    |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.41.  CAUSECODE

The purpose of the CAUSECODE information element is to indicate the

reason a call was REJECTed or HANGUPed.  It derives from ITU-T

Recommendation Q.931.  The data field is one octet long and contains

Spencer, et al.          Expires April 26, 2007                [Page 87]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

an entry from the table below.

The CAUSECODE information element SHOULD be sent with IAX2 HANGUP,

REJECT, REGREJ, and TXREJ messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x2a     |      0x01     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|   Cause Code  |

+-+-+-+-+-+-+-+-+

+--------+----------------------------------------------------------+

| NUMBER | CAUSE                                                    |

+--------+----------------------------------------------------------+

| 1      | Unassigned/unallocated number                            |

|        |                                                          |

| 2      | No route to specified transit network                    |

|        |                                                          |

| 3      | No route to destination                                  |

|        |                                                          |

| 6      | Channel unacceptable                                     |

|        |                                                          |

| 7      | Call awarded and delivered                               |

|        |                                                          |

| 16     | Normal call clearing                                     |

|        |                                                          |

| 17     | User busy                                                |

|        |                                                          |

| 18     | No user response                                         |

|        |                                                          |

| 19     | No answer                                                |

|        |                                                          |

| 21     | Call rejected                                            |

|        |                                                          |

| 22     | Number changed                                           |

|        |                                                          |

| 27     | Destination out of order                                 |

|        |                                                          |

| 28     | Invalid number format/incomplete number                  |

|        |                                                          |

| 29     | Facility rejected                                        |

|        |                                                          |

| 30     | Response to status enquiry                               |

|        |                                                          |

| 31     | Normal, unspecified                                      |

Spencer, et al.          Expires April 26, 2007                [Page 88]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

| 34     | No circuit/channel available                             |

|        |                                                          |

| 38     | Network out of order                                     |

|        |                                                          |

| 41     | Temporary failure                                        |

|        |                                                          |

| 42     | Switch congestion                                        |

|        |                                                          |

| 43     | Access information discarded                             |

|        |                                                          |

| 44     | Requested channel not available                          |

|        |                                                          |

| 45     | Pre-empted (causes.h only)                               |

|        |                                                          |

| 47     | Resource unavailable, unspecified (Q.931 only)           |

|        |                                                          |

| 50     | Facility not subscribed (causes.h only)                  |

|        |                                                          |

| 52     | Outgoing call barred (causes.h only)                     |

|        |                                                          |

| 54     | Incoming call barred (causes.h only)                     |

|        |                                                          |

| 57     | Bearer capability not authorized                         |

|        |                                                          |

| 58     | Bearer capability not available                          |

|        |                                                          |

| 63     | Service or option not available (Q.931 only)             |

|        |                                                          |

| 65     | Bearer capability not implemented                        |

|        |                                                          |

| 66     | Channel type not implemented                             |

|        |                                                          |

| 69     | Facility not implemented                                 |

|        |                                                          |

| 70     | Only restricted digital information bearer capability is |

|        | available (Q.931 only)                                   |

|        |                                                          |

| 79     | Service or option not available (Q.931 only)             |

|        |                                                          |

| 81     | Invalid call reference                                   |

|        |                                                          |

| 82     | Identified channel does not exist (Q.931 only)           |

|        |                                                          |

| 83     | A suspended call exists, but this call identity does not |

|        | (Q.931 only)                                             |

|        |                                                          |

| 84     | Call identity in use (Q.931 only)                        |

|        |                                                          |

Spencer, et al.          Expires April 26, 2007                [Page 89]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

| 85     | No call suspended (Q.931 only)                           |

|        |                                                          |

| 86     | Call has been cleared (Q.931 only)                       |

|        |                                                          |

| 88     | Incompatible destination                                 |

|        |                                                          |

| 91     | Invalid transit network selection (Q.931 only)           |

|        |                                                          |

| 95     | Invalid message, unspecified                             |

|        |                                                          |

| 96     | Mandatory information element missing (Q.931 only)       |

|        |                                                          |

| 97     | Message type nonexistent/not implemented                 |

|        |                                                          |

| 98     | Message not compatible with call state                   |

|        |                                                          |

| 99     | Information element nonexistent                          |

|        |                                                          |

| 100    | Invalid information element contents                     |

|        |                                                          |

| 101    | Message not compatible with call state                   |

|        |                                                          |

| 102    | Recovery on timer expiration                             |

|        |                                                          |

| 103    | Mandatory information element length error (causes.h     |

|        | only)                                                    |

|        |                                                          |

| 111    | Protocol error, unspecified                              |

|        |                                                          |

| 127    | Internetworking, unspecified                             |

+--------+----------------------------------------------------------+

8.6.42.  ENCRYPTION

The purpose of the ENCRYPTION information element is to indicate what

encryption methods are accepted for an IAX2 peer.  The data field is

a 2 octet bit mask specifying which encryption methods from the table

below are accepted.

The ENCRYPTION information element MAY be sent with IAX2 NEW and

AUTHREQ messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x2b     |      0x01     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 90]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

|      Encryption Methods       |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

The following table lists valid native encryption methods:

+--------+-------------+

| METHOD | DESCRIPTION |

+--------+-------------+

| 0x0001 | AES-128     |

+--------+-------------+

8.6.43.  ENCKEY

The purpose of the ENCKEY information element is to share an

encryption key with a remote peer in an already-encrypted exchange.

This makes the process of rotating encryption keys simple to

implement.  Note that an ENCKEY may only be sent between peers that

are already exchanging encrypted data.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x2c     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:          ENCKEY Data          :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.44.  CODEC PREFS

The purpose of the CODEC PREFS information element is to indicate the

CODEC preferences of the calling peer.  The data field consists of a

list of CODECs in the peer's order of preference as UTF-8-encoded

data.

The CODEC PREFS information element MAY be sent with IAX2 NEW

messages.

If the CODEC PREFS information element is absent, CODEC negotiation

takes place via the CAPABILITY and FORMAT information elements.

Spencer, et al.          Expires April 26, 2007                [Page 91]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x2d     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                               |

:       CODEC Prefs Data        :

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.45.  RR JITTER

The purpose of the RR JITTER information element is to indicate the

received jitter on a call, per rfc1889[RFC1889].  The data field is 4

octets long and carries the current measured jitter.

The RR JITTER information element MAY be sent with IAX2 PONG

messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x2e     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         Received Jitter       |

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.46.  RR LOSS

The purpose of the RR LOSS information element is to indicate the

number of lost frames on a call, per rfc1889[RFC1889].  The data

field is 4 octets long and carries the percentage of frames lost in

the first octet, and the count of lost frames in the next 3 octets.

The RR LOSS information element MAY be sent with IAX2 PONG messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x2f     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Loss Percent |               |

+-+-+-+-+-+-+-+-+  Loss Count   |

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 92]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.6.47.  RR PKTS

The purpose of the RR PKTS information element is to indicate the

total number of frames received on a call, per rfc1889[RFC1889].  The

data field is 4 octets long and carries the count of frames received.

The RR PKTS information element MAY be sent with IAX2 PONG messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x30     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    Frames Received Count      |

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.48.  RR DELAY

The purpose of the RR DELAY information element is to indicate the

maximum playout delay for a call, per rfc1889[RFC1889].  The data

field is 2 octets long and specifies the number of milliseconds a

frame may be delayed before it MUST be discarded.

The RR DELAY information element MAY be sent with IAX2 PONG messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x31     |      0x02     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|    Maximum Playout Delay      |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.49.  RR DROPPED

The purpose of the RR DROPPED information element is to indicate the

total number of dropped frames for a call, per rfc1889[RFC1889].  The

data field is 4 octets long and carries the number of frames dropped.

The RR DROPPED information element MAY be sent with IAX2 PONG

messages.

Spencer, et al.          Expires April 26, 2007                [Page 93]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x32     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      Total Frames Dropped     |

|                               |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.50.  RR OOO

The purpose of the RR OOO information element is to indicate the

number of frames received out of order for a call, per

rfc1889[RFC1889].  The data field is 4 octets long and carries the

number of frames received out of order.

The RR OOO information element MAY be sent with IAX2 PONG messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x33     |      0x04     |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|         Frames Received       |

|          Out of Order         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

8.6.51.  OSPTOKEN

The purpose of the OSPTOKEN information element is to carry European

Telecommunications Standards Institute (ETSI) Technical Specification

101 321 V4.11 (also referred to as the Open Settlement Protocol or OSP)

tokens.  The OSP tokens will be used to provide authorization,

authentication and account support for IAX2 by using the OSP protocol.

The first octet of the data field is the OSP token block index starting

from 0.

The OSPTOKEN information element MAY only be sent with IAX2 NEW

messages.

1

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      0x34     |  Data Length  |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|  Block Index  |               |

+-+-+-+-+-+-+-+-+               +

|        OSP Token Block        |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

Spencer, et al.          Expires April 26, 2007                [Page 94]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

8.7.  Media Formats

Media Format Subclass Values

+------------+----------------+-------------------------------------+

| SUBCLASS   | DESCRIPTION    | LENGTH CALCULATION                  |

+------------+----------------+-------------------------------------+

| 0x00000001 | G.723.1        | 4, 20, and 24 byte frames of 240    |

|            |                | samples                             |

|            |                |                                     |

| 0x00000002 | GSM Full Rate  | 33 byte chunks of 160 samples or 65 |

|            |                | byte chunks of 320 samples          |

|            |                |                                     |

| 0x00000004 | G.711 mu-law   | 1 byte per sample                   |

|            |                |                                     |

| 0x00000008 | G.711 a-law    | 1 byte per sample                   |

|            |                |                                     |

| 0x00000010 | G.726          |                                     |

|            |                |                                     |

| 0x00000020 | IMA ADPCM      | 1 byte per 2 samples                |

|            |                |                                     |

| 0x00000040 | 16-bit linear  | 2 bytes per sample                  |

|            | little-endian  |                                     |

|            |                |                                     |

| 0x00000080 | LPC10          | Variable size frame of 172 samples  |

|            |                |                                     |

| 0x00000100 | G.729          | 20 bytes chunks of 172 samples      |

|            |                |                                     |

| 0x00000200 | Speex          | Variable                            |

|            |                |                                     |

| 0x00000400 | ILBC           | 50 bytes per 240 samples            |

|            |                |                                     |

| 0x00000800 | AMR            | Variable                            |

|            |                |                                     |

| 0x00010000 | JPEG           |                                     |

|            |                |                                     |

| 0x00020000 | PNG            |                                     |

|            |                |                                     |

| 0x00040000 | H.261          |                                     |

|            |                |                                     |

| 0x00080000 | H.263          |                                     |

|            |                |                                     |

| 0x00100000 | H.263p         |                                     |

+------------+----------------+-------------------------------------+

Spencer, et al.          Expires April 26, 2007                [Page 95]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

9.  Example Message Flows

This section includes call flow diagrams for some of the various

types of IAX2 calls that can be made.  In each diagram, the '='

character represents a full frame and the '-' character represents a

mini frame.  Notes applicable to a generic call may be presented

alongside each diagram.

9.1.  Ping/Pong

PING->PONG

Peer A                                Peer B

________________________________________

|                                        |

T    |                                        |

i    |  ===PING============================>  |

m    |                                        |

e    |  <============================PONG===  |Has same time-stamp

|                                        | as received PING.

|    |  ===ACK=============================>  |Has same time-stamp

|    |                                        | as received PONG

\ /   |________________________________________| and original PING

9.2.  Lagrq/Lagrp

LAGRQ->LAGRP

Peer A                                Peer B

________________________________________

|                                        |

T    |                                        |

i    |  ===LAGRQ===========================>  |

m    |                                        |

e    |  <===========================LAGRP===  |Same time-stamp as

|                                        | received LAGRQ.

|    |  ===ACK=============================>  |Same time-stamp as

|    |                                        | received LAGRP and

\ /   |________________________________________| original LAGRQ.

Spencer, et al.          Expires April 26, 2007                [Page 96]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

9.3.  Registration

Registration of an IAX2 Peer

Registrant  A                     Registrar B

________________________________________

|                                        |

T    |  ===REGREQ==========================>  |

i    |                                        |

m    |  <=========================REGAUTH===  |

e    |                                        |

|  ===REGREQ==========================>  |

|    |                                        |

|    |  <==========================REGACK===  |

\ | /  |                                        |

\|/   |  ===ACK=============================>  |

|                                        |

|________________________________________|

9.4.  Registration

Registration Release

Registrant A                        Registrar B

________________________________________

|                                        |

T    |  ===REGREL==========================>  |

i    |                                        |

m    |  <=========================REGAUTH===  |

e    |                                        |

|  ===REGREL==========================>  |

|    |                                        |

|    |  <==========================REGACK===  |

\ | /  |                                        |

\|/   |  ===ACK=============================>  |

|                                        |

|________________________________________|

Spencer, et al.          Expires April 26, 2007                [Page 97]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

9.5.  Provisioning

Provisioning an IAX2 Device

Peer A (Provisioning server)          Peer B (IAX2 Device)

________________________________________

|                                        |

T    |                                        |

i    |  ===PROVISION=======================>  |Carries

m    |                                        |  PROVISIONING IE

e    |  <==========================ACCEPT===  |

|                                        |

|    |  ===ACK=============================>  |

|    |                                        |

\ /   |________________________________________|

9.6.  Firmware Download

IAX2 Firmware Download

Peer A (IAX2 Device)              Peer B (FW manager)

________________________________________

|                                        |

T    |                                        |

|  ===FWDOWNL=========================>  |IE spec Device Type

i    |                 .                      | and firmware block

|                 .                      | desired.

m    |                 .                      |

|  <==========================FWDATA===  |IEs spec firmware

e    |                 .                      |  block identifier

|                 .                      |  and data block

|                 .                      |

|    |  ===FWDOWNL=========================>  |FWDOWNL/FWDATA

|                 .                      | messages pass back

|                 .                      | forth until all

|                 .                      | firmware xmitted.

|                 .                      |

|    |  <==========================FWDATA===  |The last FWDATA

|                 .                      | contains 0 length

|                 .                      | FWBLOCKDATA IE

|                 .                      |

\ /   |  ==============================ACK==>  |The last FWDATA

|                                        |  MUST be ACKed.

|________________________________________|

Spencer, et al.          Expires April 26, 2007                [Page 98]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

9.7.  Call Path Optimization

IAX2 Transfer

Peer L         Peer C                Peer R

________________________________________

|                 |                      |

T    |                 |                      |

| <== TXREQ =====[*]== TXREQ =========>  |C requests transfer

i    |                 |                      |

| ========================== TXCNT  ==>  |L sends to R

m    |                 |                      |

| <========================= TXACC  ==== |R replies

e    |                 |                      |R sends Media

|                 |                      | to L

|    |                 |                      |

|    | = TXREADY ====> |                      |L tells C 'ready'

|    |                 |                      | C stops media to L

|    |                 |                      |

|    | <== TXCNT ===========================  |L sends to R

|    |                 |                      |

|    | === TXACC ===========================> |R replies

\ /   |                 |                      |

|                 | <== TXREADY ======   |R tells C 'ready'

|                 |                      | C stops media to R

|                 |                      |

| <== TXREL =====[*]== TXREL =========>  |C Releases

|                                        |

|________________________________________|

9.8.  IAX2 Media Call

Complete end-to-end IAX2 media exchange

Peer A                            Peer B

________________________________________

|                                        |

|  ====NEW============================>  |

T    |  <=========================AUTHREQ===  |If authentication

|                                        |   specified.

i    |  ====AUTHREP========================>  |

m    |  <==========================ACCEPT===  |

e    |  ====ACK============================>  |

|                                        |

|    |  <=============Voice (full frame)===   |

Spencer, et al.          Expires April 26, 2007                [Page 99]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

|    |  ====ACK===========================>   |

|    |                                        |

|    |  <---------Voice miniframe (ring)---   |

|    |  <---------Voice miniframe (ring)---   |

|    |                                        |

\ | /  |  <=========================RINGING===  |

\|/   |  ====ACK============================>  |

|                                        |

|  <---------Voice miniframe (ring)---   |

|  <---------Voice miniframe (ring)---   |

|                                        |

|  <==========================ANSWER===  |

|  ====ACK============================>  |

|                                        |

|  ====Voice (full frame)=============>  |

|  <=============================ACK===  |

|                                        |

|                                        |

|  <-----------Voice miniframes------->  |  exchange occurs

|  <---               .            --->  |

|  <---               .            --->  |

|  <---               .            --->  |

|  <-----------Voice miniframes------->  |

|                                        |

|                                        |

|  ====Voice (full frame)=============>  |  (note 1)

|  <===ACK=============================  |  (note 2)

|                                        |  (every 65536 ms).

|  <=============Voice (full frame)====  |  (note 3)

|  ====ACK============================>  |

|                                        |

|                                        |

|  <-----------Voice miniframes------->  |

|  <---               .            --->  |

|  <---               .            --->  |

|  <---               .            --->  |

|  <-----------Voice miniframes------->  |

|                                        |

|                                        |

|  ====HANGUP=========================>  |  Either can hangup

|  <=============================ACK===  |

|________________________________________|

Note 1: Mini Frames carry the low 16 bits of the peer's

32-bit time-stamp.

Note 2: Full frames re-sync the 32 bit time-stamp when the 16 bit

time-stamp overflows.

Spencer, et al.          Expires April 26, 2007               [Page 100]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

Note 3:Each side has its own 32 bit time-stamp so each side needs

to sync at 16 bit overflow.

9.9.  IAX2 Media Call via an IAX2 Device

An IAX2 peer is not required to maintain a complete dialplan.  In the

event that a user wishes to dial from an IAX2 peer which does not

switch its own calls, the following call flow diagram may represent

the transaction:

Peer A (IAX2 Device)                 Peer B (Dialplan Server)

________________________________________

|                                        |

|  ====NEW============================>  |

T    |  <=========================AUTHREQ===  |  If auth specified

i    |  ====AUTHREP========================>  |

m    |  <==========================ACCEPT===  |

e    |  ====ACK============================>  |

|                                        |

|  ====DPREQ==========================>  |  (Note 1)

|    |  <===========================DPREP===  |

|    |                                        |

|    |  ====DIAL===========================>  |

|    |  <========================PROGRESS===  |

|    |  ====ACK============================>  |

\ | /  |  <==========================ANSWER===  |

\|/   |  ====ACK============================>  |

|                                        |

|  ====Voice (full frame)=============>  |

|  <=============================ACK===  |

|  <=============Voice (full frame)====  |

|  ====ACK============================>  |

|                                        |

|                                        |

|  <-----------Voice miniframes------->  |  Media exchange

|  <---               .            --->  |

|  <---               .            --->  |

|  <---               .            --->  |

|  <-----------Voice miniframes------->  |

|                                        |

|                                        |

|  ====Voice (full frame)=============>  |  (note 2)

|  <===ACK=============================  |  (note 3)

|                                        |  (every 65536 ms).

|  <=============Voice (full frame)====  |  (Note 4)

|  ====ACK============================>  |

Spencer, et al.          Expires April 26, 2007               [Page 101]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

|                                        |

|                                        |

|  <-----------Voice miniframes------->  |

|  <---               .            --->  |

|  <---               .            --->  |

|  <---               .            --->  |

|  <-----------Voice miniframes------->  |

|                                        |

|                                        |

|  ====HANGUP=========================>  |  Either can hangup

|  <=============================ACK===  |

|________________________________________|

Note 1: There will be multiple DPREQ/DPREPs per call unless

dialed number is 1 digit long

Note 2: Mini Frames carry the low 16 bits of the peer's

32 bit time-stamp.

Note 3: Full frames re-sync the 32 bit time-stamp when the 16 bit

time-stamp overflows

Note 4: Each side has its own 32 bit time-stamp so each side needs

to sync at 16 bit overflow.

Spencer, et al.          Expires April 26, 2007               [Page 102]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

10.  Security Considerations

IAX2 is a binary protocol for setting up point to point call legs

which includes both media and signaling.  As Such, it is simpler to

secure than other more general purpose VoIP protocols, however,

security remains a difficult task.

IAX2 registration is one area that requires attention.  While clear

text passwords are supported, they should not be used.  The MD5 and

RSA alternatives offer much higher security.  Although not specified

by IAX2, some implementations limit the number of registrants per

account to one.  And a subsequent registrant with the same

credentials would overwrite the prior and receive the calls destined

for that user.  Theft of service is trivial once a malicious caller

has the ability to authenticate.

Denial of service attacks can take at least two forms in IAX2.  One

is simply overloading the peers with bogus requests.  A carefully

implemented IAX2 peer would identify this situation and raise an

alarm or take other protective action.

Another form of denial of service (DoS) against an existing call is

an engineered attack against an existing call.  Injecting media,

causing excess processing by inserting out of order packets, and

sending commands such as hangup or transfer.  These attacks require

close monitoring of the binary channel to be successful as the

message sequence numbers would need to be synchronized with the

protocol exchange.

Of course, using IPSEC, [RFC2401], would address many of these

potential issues.

Spencer, et al.          Expires April 26, 2007               [Page 103]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

11.  IANA Considerations

This document requests registration of the "iax2" URI Scheme.  See

section Section 5.  IAX2 also requires a well-known UDP port to be

assigned.  The current port in use is 4569.

Spencer, et al.          Expires April 26, 2007               [Page 104]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

12.  Implementation Notes

This specification differs from the current Implementation in the

following ways:

*  Plain text authenication has been removed.

*  All IEs now use UTF-8 instead of ASCII.

*  AESProvisioning Section 8.6.29 is not currently implemented.

Spencer, et al.          Expires April 26, 2007               [Page 105]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

13.  Acknowledgments

This work was supported by Internet Foundation Austria.  The authors

would like to thank Olle Johansson, Steve Kann, Alexander Mayrhofer

and Mohamed Boucadair for their extensive review and technical input.

14.  References

[AES]      U.S. Department of Commerce/N.I.S.T., "FIPS-197,

Announcing the Advanced Encryption Standard",

November 2001.

[E164]     ITU-T, "The International Public Telecommunication Number

Plan",  Recommendation E.164, May 1997.

[RFC1321]  Rivest, R., "The MD5 Message-Digest Algorithm", RFC 1321,

April 1992.

[RFC1851]  Karn, P., Metzger, P., and W. Simpson, "The ESP Triple DES

Transform", RFC 1851, September 1995.

[RFC1889]  Schulzrinne, H., Casner, S., Frederick, R., and V.

Jacobson, "RTP: A Transport Protocol for Real-Time

Applications", RFC 1889, January 1996.

[RFC2119]  Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119, March 1997.

[RFC2401]  Kent, S. and R. Atkinson, "Security Architecture for the

Internet Protocol", RFC 2401, November 1998.

[RFC2437]  Kaliski, B. and J. Staddon, "PKCS #1: RSA Cryptography

Specifications Version 2.0", RFC 2437, October 1998.

[RFC2526]  Johnson, D. and S. Deering, "Reserved IPv6 Subnet Anycast

Addresses", RFC 2526, March 1999.

[RFC2833]  Schulzrinne, H. and S. Petrack, "RTP Payload for DTMF

Digits, Telephony Tones and Telephony Signals", RFC 2833,

May 2000.

[RFC3261]  Rosenberg, J., Schulzrinne, H., Camarillo, G., Johnston,

A., Peterson, J., Sparks, R., Handley, M., and E.

Schooler, "SIP: Session Initiation Protocol", RFC 3261,

June 2002.

[RFC3629]  Yergeau, F., "UTF-8, a transformation format of ISO

10646", STD 63, RFC 3629, November 2003.

Spencer, et al.          Expires April 26, 2007               [Page 106]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

[RFC3761]  Faltstrom, P. and M. Mealling, "The E.164 to Uniform

Resource Identifiers (URI) Dynamic Delegation Discovery

System (DDDS) Application (ENUM)", RFC 3761, April 2004.

[RFC3986]  Berners-Lee, T., Fielding, R., and L. Masinter, "Uniform

Resource Identifier (URI): Generic Syntax", STD 66,

RFC 3986, January 2005.

[RFC4395]  Hansen, T., Hardie, T., and L. Masinter, "Guidelines and

Registration Procedures for New URI Schemes", BCP 115,

RFC 4395, February 2006.

Spencer, et al.          Expires April 26, 2007               [Page 107]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

Authors' Addresses

Mark A. Spencer

Digium, Inc.

150 West Park Loop Suite 100

Huntsville, AL  35806

US

Phone: +1 256 428 6000

Email: markster@digium.com

URI:   http://www.digium.com/

Brian Capouch

Saint Joseph's College

PO Box 909

Rensselaer, IN  47978

US

Phone: +1 219 866 6114

Email: brianc@saintjoe.edu

Ed Guy (editor)

E-MC Software

235 Main Street, STE 253

Madison, NJ  07940

US

Phone: +1 973 437 4519

Email: edguy@emcsw.com

URI:   http://www.emcsw.com/

Frank Miller

Cornfed Systems, Inc.

103 Overhill Road

Baltimore, MD  21210

US

Phone: +1 410 404-8790

Email: fwmiller@cornfed.com

URI:   http://www.digium.com/

Spencer, et al.          Expires April 26, 2007               [Page 108]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

Kenneth C. Shumard

3818 N Lakegrove Way

Boise, ID  83713

US

Phone: +1 208 724 7801

Email: kshumard@gmail.com

Spencer, et al.          Expires April 26, 2007               [Page 109]

Internet-Draft   IAX2: Inter-Asterisk eXchange Version 2    October 2006

Intellectual Property Statement

The IETF takes no position regarding the validity or scope of any

Intellectual Property Rights or other rights that might be claimed to

pertain to the implementation or use of the technology described in

this document or the extent to which any license under such rights

might or might not be available; nor does it represent that it has

made any independent effort to identify any such rights.  Information

on the procedures with respect to rights in RFC documents can be

found in BCP 78 and BCP 79.

Copies of IPR disclosures made to the IETF Secretariat and any

assurances of licenses to be made available, or the result of an

attempt made to obtain a general license or permission for the use of

such proprietary rights by implementers or users of this

specification can be obtained from the IETF on-line IPR repository at

http://www.ietf.org/ipr.

The IETF invites any interested party to bring to its attention any

copyrights, patents or patent applications, or other proprietary

rights that may cover technology that may be required to implement

this standard.  Please address the information to the IETF at

ietf-ipr@ietf.org.

Disclaimer of Validity

This document and the information contained herein are provided on an

"AS IS" basis and THE CONTRIBUTOR, THE ORGANIZATION HE/SHE REPRESENTS

OR IS SPONSORED BY (IF ANY), THE INTERNET SOCIETY AND THE INTERNET

ENGINEERING TASK FORCE DISCLAIM ALL WARRANTIES, EXPRESS OR IMPLIED,

INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE

INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED

WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

Copyright Statement

Copyright (C) The Internet Society (2006).  This document is subject

to the rights, licenses and restrictions contained in BCP 78, and

except as set forth therein, the authors retain all their rights.

Acknowledgment

Funding for the RFC Editor function is currently provided by the

Internet Society.

Spencer, et al.          Expires April 26, 2007               [Page 110]