QSIG CCBS/CCNR with Dahdi-Channel

The QSIG CCBS/CCNR implementation is based on the latest versions of ETSI 300 366 resp. ECMA-186. The implementation is restricted to the service options “without path reservation” und “without service retention”.

The option “connection retention method” is used for the signalling connection.

There are up to 256 recall requests (signalling connections) supported.

There are 4 new API-functions for CCBS/CCNR:

DAHDIQsigCcbsnrInitialize

DAHDIQsigCheckCcbsnr

DAHDIQsigCcbsnrRequest

DAHDIQsigClearNoChannel

DAHDIQsigCcbsnrInitialize()

Initialize Channel-Variables:

CCPEERLINKID          = id

CCBSNRREQSTATE   = "AVAILABLE" and

CCBSNRONPRISPAN = 0.

The CCPEERLINKID is used to store the calling channel internally.

This function must be called initially before DAHDIQsigCheckCcbsnr() or DAHDIQsigCcbsnrRequest() can be used.

DAHDIQsigCheckCcbsnr(<limit>|< callingnum>|< callernum >|

<disallowed_callernum>|<disallowed_callernum>|

<disalowed_callernum>|<disallowed_callernum>)

limit:			This parameter defines how often a recall-request for a specific

subscriber is possible at the same time.

If it is ‘0’, the maximum value (256) is used.

callingnum:		Calling number.

callernum:	 	Called number.

Parameter 4 to 7: 	Parameter 4 to 7 defines optional 'numbers' for which a recall offer

shall not be indicated. (e.g. ‚ anonymous’, ‚ Anonymous’ or ‚

unknown’.)

This function must be called before DAHDIQsigCcbsnrRequest() can be used.

DAHDIQsigCcbsnrRequest(<type>|<callingnum>|<callernum>|<callername>|<context>|<priority>)

type:		‚CCBS’ or ‚CCNR’.

callingnum:	Calling number.

callernum: 	Caller number.

callername: 	Caller name.

context:	Context of the dialplan in which the recall is made when B –Party is free again.

priority:		Priority of the context.

Send a request specified by type.

DAHDIQsigClearNoChannel(<callingnum>|<callernum >)

callingnum:	Calling number.

callernum: 	Caller number.

Clear a CCBS/CCNR request for this pair.

New parameter for the Dial-API

Dial(Dahdi/g1q/${EXTEN},30,g)

‚q’ indicates to send the setup-message with an ‘cc-ringout invoke facility.

Config Files:

Chan_dahdi.conf

New  ISDN Timer CCT2

; CCT2: supplementary service call completion service duration.

;       Lifetime of CCBS/CCNR-Request (default 45 minutes)

;pritimer => cct2, 2700000

Dialplan:

extension.conf

Example:

(SIP-Number A-party calls ISDN-number B-party)

[sip]
exten => t,1,Hangup()
exten => i,1,Hangup()

exten => _X.,1,Set(CALLINGNUM=${EXTEN})
exten => _X.,n,Set(CALLERNUM=${CALLERID(num)})
exten => _X.,n,ExecIf($["${CALLERID(name)}"="Anonymous"&&"${CALLERID(num)}"="anonymous"]?

Macro(clircnir))
exten => _X.,n,ExecIf($["${CALLERID(num)}"="Anonymous"&&"${CALLERID(name)}"="anonymous"]?

Macro(clir)
exten => _X.,n,DAHDIQsigCcbsnrInitialize()
exten => _X.,n,ExecIf($["${CCREQ}"!="q"]?Dial(Dahdi/g1/${EXTEN},60,g)

:Dial(Dahdi/g1q/${EXTEN},30,g))
exten => _X.,n,Goto(X-${DIALSTATUS},1)

exten => X-BUSY,1,Macro(CCBS)
exten => X-BUSY,n,Hangup()

exten => X-NOANSWER,1,Macro(CCNR)
exten => X-NOANSWER,n,Hangup()

exten => X-ANSWER,1,Hangup()

[macro-CCBS]
exten => s,1,ExecIf($["${CCBSNRREQSTATE}"!="AVAILABLE"]?MacroExit)
exten => s,n,DAHDIQsigCheckCcbsnr(5|${CALLINGNUM}|${CALLERNUM}

|anonymous|Anonymous|unknown)

exten => s,n,ExecIf($["${CCBSNRREQSTATE}"="NOTAVAILABLE"]?MacroExit)
exten => s,n,Macro(AW)
exten => s,n,Background(<The subscriber can not answer their calling>&

<break-1-sec>&

<for an automatic recall press the star button>&

<break-10-sec>,m,,${CONTEXT})
exten => *,1,DAHDIQsigCcbsnrRequest(CCBS|${CALLINGNUM}|${CALLERNUM}|${CALLERID(name)}|

ccbsnr-signal|1)

exten => *,n,Macro(TIMER_T1)
exten => *,n,ExecIf($["${CCBSNRREQSTATE}"="ACTIVATED"]?MacroExit)
exten => *,n,DAHDIQsigClearNoChannel(${CALLINGNUM}|${CALLERNUM})
exten => *,n,MacroExit

[macro-CCNR]
exten => s,1,ExecIf($["${CCBSNRREQSTATE}"!="AVAILABLE"]?MacroExit)
exten => s,n,DAHDIQsigCheckCcbsnr(5|${CALLINGNUM}|${CALLERNUM}

|anonymous|Anonymous|unknown) 
exten => s,n,ExecIf($["${CCBSNRREQSTATE}"="NOTAVAILABLE"]?MacroExit)
exten => s,n,Macro(AW)
exten => s,n,Background(<The subscriber did not respond>&

<break-1-sec>&

<for an automatic recall press the star button>&

<break-10-sec>,m,,${CONTEXT})
exten => *,1,DAHDIQsigCcbsnrRequest(CCNR|${CALLINGNUM}|${CALLERNUM}

|${CALLERID(name)}|ccbsnr-signal|1)
exten => *,n,Macro(TIMER_T1)
exten => *,n,ExecIf($["${CCBSNRREQSTATE}"="ACTIVATED"]?MacroExit)
exten => *,n,DAHDIQsigClearNoChannel(${CALLINGNUM}|${CALLERNUM})
exten => *,n,MacroExit

[ccbsnr-signal]
; this is triggered when the network signals 'remote party free'
exten => _X.,1,Set(RETRYTIME=30)    ; ring 30 sec
exten => _X.,n,Set(MAXRETRIES=4)    ; retrys to numberA
exten => _X.,n,Set(WAITTIME=60)       ; Wait 60 sec to recall numberA
exten => _X.,n,System(/usr/local/bin/ccbsnr-qsig.sh ${CALLERID(num)}

${EXTEN} "${CALLERID(name)}" ${RETRYTIME} ${MAXRETRIES}

${WAITTIME})

[ccbsnr-callback]
; this should be activated by the calloutfile
;; call numberB
exten => _X.,1,Set(CALLERID(num)=${CC_CALLING_ID})
exten => _X.,n,Set(CALLERID(name)=${CC_CALLING_NAME})
exten => _X.,n,Set(CCREQ=q)
exten => _X.,n,System(rm/var/spool/asterisk/outgoing/callfile.${EXTEN}.${CALLERID(num)})
exten => _X.,n,GoTo(sip,${EXTEN},1)

[fromCallFile]
;; call numberA
; set Callername to 'Recall'
exten => _X.,1,NoOp(fromCallFile call from '${CALLERID(all)}' to ${EXTEN})
exten => _X.,n,ExecIf($["${EXTEN}"="${CC_CALLING_ID}"]?Set(CALLERID(name)="Recall"))
exten => _X.,n,Dial(SIP/${EXTEN},30)
exten => _X.,n,Goto(X-${DIALSTATUS},1)

exten => X-BUSY,1,DAHDIQsigClearNoChannel(${CC_CALLED_ID}|${CC_CALLING_ID})

exten => X-NOANSWER,1,DAHDIQsigClearNoChannel(${CC_CALLED_ID}|${CC_CALLING_ID})

exten => X-CONGESTION,1,DAHDIQsigClearNoChannel(${CC_CALLED_ID}|${CC_CALLING_ID})

exten => t,1,Hangup()
exten => t,n,DAHDIQsigClearNoChannel(${CC_CALLED_ID}|${CC_CALLING_ID})

exten => i,1,Hangup()
exten => i,n,DAHDIQsigClearNoChannel(${CC_CALLED_ID}|${CC_CALLING_ID})

[macro-TIMER_T1]
exten => s,1,Set(CC_TIMER_T1=10)        ; Wait 10-30 seconds
exten => s,n(loop),GoToIf($[${CC_TIMER_T1}=0]?timeout)
exten => s,n,Set(CC_TIMER_T1=$[${CC_TIMER_T1}-1])
exten => s,n,Wait(1)
exten => s,n,GoToIf($["${CCBSNRREQSTATE}"!="ACTIVATED"]?loop)
exten => s,n,Playback(<recall programmed>)
exten => s,n,MacroExit
exten => s,n(timeout),Playback(<no recall programmed>)
exten => s,n,MacroExit



[macro-clircnir]
exten => s,1,Verbose(1|**** CLIR + CNIR is set, the number and the name is suppressed)
exten => s,n,Set(CALLERPRES()=prohib_not_screened)
exten => s,n,Set(CALLERID(name)=****)
exten => s,n,Set(CALLERID(num)=)

exten => s,n,MacroExit


[macro-clir]
exten => s,1,Verbose(1|**** Clir is set the number is suppressed. The name  is unchanged )
exten => s,n,Set(CALLERPRES()=prohib_not_screened)
exten => s,n,Set(CALLERID(num)=)
exten => s,n,MacroExit

[macro-AW]
exten => s,1,Answer()
exten => s,n,Wait(1)
exten => s,n,MacroExit

Shell-Script:

The shell script, ccbsnr-qsig.sh 'is located in the directory '/usr/local/bin'.

It is used in context ‚ccbsnr-signal’ and generates a call file for the Asterisk.

When the script is running the variables ‘CC_CALLING_ID", "CC_CALLED_ID" and "CC_CALLING_NAME’ are set and can be used in the dialplan.

#!/bin/sh
CALLINGNUMBER=$1
CALLEDNUMBER=$2
CALLINGNAME=$3
RETRYTIME=$4
MAXRETRIES=$5
WAITTIME=$6
(
	rm /var/spool/asterisk/outgoing/callfile.$CALLEDNUMBER.$CALLINGNUMBER
	echo "Channel: local/$CALLINGNUMBER@fromCallFile"
	echo "RetryTime: $RETRYTIME"
	echo "MaxRetries: $MAXRETRIES"
	echo "WaitTime: $WAITTIME"
	echo "Context: ccbsnr-callback"
	echo "Extension: $CALLEDNUMBER"
	echo "CallerID: $CALLINGNAME <$CALLEDNUMBER>"
	echo "Setvar: CC_CALLING_ID=$CALLINGNUMBER";
	echo "Setvar: CC_CALLED_ID=$CALLEDNUMBER";
	echo "Setvar: CC_CALLING_NAME=$CALLINGNAME";
	echo "Priority: 1"
) > /tmp/callfile.$CALLEDNUMBER.$CALLINGNUMBER
mv /tmp/callfile.$CALLEDNUMBER.$CALLINGNUMBER /var/spool/asterisk/outgoing/