SIP Debugging Enabled for IP: 69.159.199.98:5060

Reliably Transmitting (NAT) to 69.159.199.98:5060:

OPTIONS sip:2898407015@69.159.199.98:5060 SIP/2.0

Via: SIP/2.0/UDP 209.217.98.130:5060;branch=z9hG4bK5311c99e;rport

Max-Forwards: 70

From: "asterisk" <sip:asterisk@209.217.98.130>;tag=as556062fb

To: <sip:2898407015@69.159.199.98:5060>

Contact: <sip:asterisk@209.217.98.130>

Call-ID: 0cdb57501e0d06ca67888cec6e566ab3@209.217.98.130

CSeq: 102 OPTIONS

User-Agent: HMNet2.1

Date: Tue, 19 May 2009 05:29:09 GMT

Allow: INVITE, ACK, CANCEL, OPTIONS, BYE, REFER, SUBSCRIBE, NOTIFY

Supported: replaces, timer

Content-Length: 0

---

sip01*CLI>

<--- SIP read from UDP://69.159.199.98:5060 --->

SIP/2.0 200 Ok

Via: SIP/2.0/UDP 209.217.98.130:5060;branch=z9hG4bK5311c99e;received=209.217.98.130;rport=5060

To: <sip:2898407015@69.159.199.98:5060>

From: "asterisk" <sip:asterisk@209.217.98.130>;tag=as556062fb

CSeq: 102 OPTIONS

Call-ID: 0cdb57501e0d06ca67888cec6e566ab3@209.217.98.130

Date: Tue, 19 May 2009 05:28:48 GMT

Contact: <sip:69.159.199.98:5060>

Server: IST-OIS-7.0.2.1

Allow: INVITE,ACK,CANCEL,BYE,OPTIONS,REFER,NOTIFY

Content-Length: 0

sip01*CLI>

<------------->

--- (11 headers 0 lines) ---

Really destroying SIP dialog '0cdb57501e0d06ca67888cec6e566ab3@209.217.98.130' Method: OPTIONS

== Using SIP RTP CoS mark 5

== Using UDPTL CoS mark 5

-- Executing [2898407015@from-vss-gateway:1] Dial("SIP/64.26.157.244-b702f448", "SIP/2898407015,24,r") in new stack

== Using SIP RTP CoS mark 5

== Using UDPTL CoS mark 5

Audio is at 209.217.98.130 port 11840

Adding codec 0x4 (ulaw) to SDP

Adding codec 0x2 (gsm) to SDP

Adding non-codec 0x1 (telephone-event) to SDP

Reliably Transmitting (NAT) to 69.159.199.98:5060:

INVITE sip:2898407015@69.159.199.98:5060 SIP/2.0

Via: SIP/2.0/UDP 209.217.98.130:5060;branch=z9hG4bK17c1d5e2;rport

Max-Forwards: 70

From: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

To: <sip:2898407015@69.159.199.98:5060>

Contact: <sip:6136886212@209.217.98.130>

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

CSeq: 102 INVITE

User-Agent: HMNet2.1

Date: Tue, 19 May 2009 05:29:10 GMT

Allow: INVITE, ACK, CANCEL, OPTIONS, BYE, REFER, SUBSCRIBE, NOTIFY

Supported: replaces, timer

Content-Type: application/sdp

Content-Length: 276

v=0

o=root 177873221 177873221 IN IP4 209.217.98.130

s=HMNet2.1

c=IN IP4 209.217.98.130

t=0 0

m=audio 11840 RTP/AVP 0 3 101

a=rtpmap:0 PCMU/8000

a=rtpmap:3 GSM/8000

a=rtpmap:101 telephone-event/8000

a=fmtp:101 0-16

a=silenceSupp:off - - - -

a=ptime:20

a=sendrecv

---

-- Called 2898407015

sip01*CLI>

<--- SIP read from UDP://69.159.199.98:5060 --->

SIP/2.0 100 Trying

Via: SIP/2.0/UDP 209.217.98.130:5060;branch=z9hG4bK17c1d5e2;received=209.217.98.130;rport=5060

To: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

From: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

CSeq: 102 INVITE

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

Date: Tue, 19 May 2009 05:28:48 GMT

Server: IST-OIS-7.0.2.1

Content-Length: 0

<------------->

--- (9 headers 0 lines) ---

sip01*CLI>

<--- SIP read from UDP://69.159.199.98:5060 --->

SIP/2.0 180 Ringing

Via: SIP/2.0/UDP 209.217.98.130:5060;branch=z9hG4bK17c1d5e2;received=209.217.98.130;rport=5060

To: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

From: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

CSeq: 102 INVITE

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

Date: Tue, 19 May 2009 05:28:48 GMT

Server: IST-OIS-7.0.2.1

Content-Length: 0

<------------->

--- (9 headers 0 lines) ---

-- SIP/2898407015-098d6588 is ringing

sip01*CLI>

<--- SIP read from UDP://69.159.199.98:5060 --->

SIP/2.0 200 Ok

Via: SIP/2.0/UDP 209.217.98.130:5060;branch=z9hG4bK17c1d5e2;received=209.217.98.130;rport=5060

To: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

From: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

CSeq: 102 INVITE

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

Date: Tue, 19 May 2009 05:28:48 GMT

Contact: <sip:2898407015@69.159.199.98:5060>

Supported: replaces, 100rel

Server: IST-OIS-7.0.2.1

Content-Type: application/sdp

Content-Length: 207

v=0

o=- 1242710928 1242710929 IN IP4 69.159.199.98

s=Call

c=IN IP4 69.159.199.98

t=0 0

m=audio 15144 RTP/AVP 0 101

a=ptime:20

a=rtpmap:0 PCMU/8000

a=rtpmap:101 telephone-event/8000

a=fmtp:101 0-15

<------------->

--- (12 headers 10 lines) ---

Found RTP audio format 0

Found RTP audio format 101

Peer audio RTP is at port 69.159.199.98:15144

Found audio description format PCMU for ID 0

Found audio description format telephone-event for ID 101

Capabilities: us - 0x6 (gsm|ulaw), peer - audio=0x4 (ulaw)/video=0x0 (nothing)/text=0x0 (nothing), combined - 0x4 (ulaw)

Non-codec capabilities (dtmf): us - 0x1 (telephone-event), peer - 0x1 (telephone-event), combined - 0x1 (telephone-event)

Peer audio RTP is at port 69.159.199.98:15144

list_route: hop: <sip:2898407015@69.159.199.98:5060>

set_destination: Parsing <sip:2898407015@69.159.199.98:5060> for address/port to send to

set_destination: set destination to 69.159.199.98, port 5060

Transmitting (NAT) to 69.159.199.98:5060:

ACK sip:2898407015@69.159.199.98:5060 SIP/2.0

Via: SIP/2.0/UDP 209.217.98.130:5060;branch=z9hG4bK7d0c073d;rport

Max-Forwards: 70

From: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

To: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

Contact: <sip:6136886212@209.217.98.130>

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

CSeq: 102 ACK

User-Agent: HMNet2.1

Content-Length: 0

---

-- SIP/2898407015-098d6588 answered SIP/64.26.157.244-b702f448

-- Packet2Packet bridging SIP/64.26.157.244-b702f448 and SIP/2898407015-098d6588

sip01*CLI>

<--- SIP read from UDP://69.159.199.98:5060 --->

INVITE sip:6136886212@209.217.98.130 SIP/2.0

Via: SIP/2.0/UDP 69.159.199.98:5060;branch=z9hG4bK-12439f-2fe7

To: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

From: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

CSeq: 43245 INVITE

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

Date: Tue, 19 May 2009 05:29:03 GMT

Max-Forwards: 70

Contact: <sip:2898407015@69.159.199.98:5060>

Accept: application/sdp

Allow: INVITE,ACK,CANCEL,BYE,OPTIONS,REFER,NOTIFY

Supported: replaces, 100rel

User-Agent: IST-OIS-7.0.2.1

Content-Type: application/sdp

Content-Length: 207

v=0

o=- 1242710928 1242710930 IN IP4 69.159.199.98

s=Call

c=IN IP4 69.159.199.98

t=0 0

m=audio 15372 RTP/AVP 0 101

a=ptime:20

a=rtpmap:0 PCMU/8000

a=rtpmap:101 telephone-event/8000

a=fmtp:101 0-15

<------------->

--- (15 headers 10 lines) ---

Sending to 69.159.199.98 : 5060 (NAT)

Found RTP audio format 0

Found RTP audio format 101

Peer audio RTP is at port 69.159.199.98:15372

Found audio description format PCMU for ID 0

Found audio description format telephone-event for ID 101

Capabilities: us - 0x6 (gsm|ulaw), peer - audio=0x4 (ulaw)/video=0x0 (nothing)/text=0x0 (nothing), combined - 0x4 (ulaw)

Non-codec capabilities (dtmf): us - 0x1 (telephone-event), peer - 0x1 (telephone-event), combined - 0x1 (telephone-event)

Peer audio RTP is at port 69.159.199.98:15372

sip01*CLI>

<--- Transmitting (NAT) to 69.159.199.98:5060 --->

SIP/2.0 100 Trying

Via: SIP/2.0/UDP 69.159.199.98:5060;branch=z9hG4bK-12439f-2fe7;received=69.159.199.98

From: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

To: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

CSeq: 43245 INVITE

User-Agent: HMNet2.1

Allow: INVITE, ACK, CANCEL, OPTIONS, BYE, REFER, SUBSCRIBE, NOTIFY

Supported: replaces, timer

Contact: <sip:6136886212@209.217.98.130>

Content-Length: 0

<------------>

Audio is at 209.217.98.130 port 11840

Adding codec 0x4 (ulaw) to SDP

Adding non-codec 0x1 (telephone-event) to SDP

<--- Reliably Transmitting (NAT) to 69.159.199.98:5060 --->

SIP/2.0 200 OK

Via: SIP/2.0/UDP 69.159.199.98:5060;branch=z9hG4bK-12439f-2fe7;received=69.159.199.98

From: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

To: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

CSeq: 43245 INVITE

User-Agent: HMNet2.1

Allow: INVITE, ACK, CANCEL, OPTIONS, BYE, REFER, SUBSCRIBE, NOTIFY

Supported: replaces, timer

Contact: <sip:6136886212@209.217.98.130>

Content-Type: application/sdp

Content-Length: 253

v=0

o=root 177873221 177873222 IN IP4 209.217.98.130

s=HMNet2.1

c=IN IP4 209.217.98.130

t=0 0

m=audio 11840 RTP/AVP 0 101

a=rtpmap:0 PCMU/8000

a=rtpmap:101 telephone-event/8000

a=fmtp:101 0-16

a=silenceSupp:off - - - -

a=ptime:20

a=sendrecv

<------------>

sip01*CLI>

<--- SIP read from UDP://69.159.199.98:5060 --->

ACK sip:6136886212@209.217.98.130 SIP/2.0

Via: SIP/2.0/UDP 69.159.199.98:5060;branch=z9hG4bK-12439f-2fe9

To: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

From: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

CSeq: 43245 ACK

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

Date: Tue, 19 May 2009 05:29:03 GMT

Max-Forwards: 70

User-Agent: IST-OIS-7.0.2.1

Content-Length: 0

<------------->

--- (10 headers 0 lines) ---

sip01*CLI>

<--- SIP read from UDP://69.159.199.98:5060 --->

INVITE sip:6136886212@209.217.98.130 SIP/2.0

Via: SIP/2.0/UDP 69.159.199.98:5060;branch=z9hG4bK-12439f-3002

To: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

From: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

CSeq: 43246 INVITE

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

Date: Tue, 19 May 2009 05:29:03 GMT

Max-Forwards: 70

Contact: <sip:2898407015@69.159.199.98:5060>

Accept: application/sdp

Allow: INVITE,ACK,CANCEL,BYE,OPTIONS,REFER,NOTIFY

Supported: replaces, 100rel

User-Agent: IST-OIS-7.0.2.1

Content-Type: application/sdp

Content-Length: 207

v=0

o=- 1242710928 1242710931 IN IP4 69.159.199.98

s=Call

c=IN IP4 69.159.199.98

t=0 0

m=audio 15454 RTP/AVP 0 101

a=ptime:20

a=rtpmap:0 PCMU/8000

a=rtpmap:101 telephone-event/8000

a=fmtp:101 0-15

<------------->

--- (15 headers 10 lines) ---

Sending to 69.159.199.98 : 5060 (NAT)

Found RTP audio format 0

Found RTP audio format 101

Peer audio RTP is at port 69.159.199.98:15454

Found audio description format PCMU for ID 0

Found audio description format telephone-event for ID 101

Capabilities: us - 0x6 (gsm|ulaw), peer - audio=0x4 (ulaw)/video=0x0 (nothing)/text=0x0 (nothing), combined - 0x4 (ulaw)

Non-codec capabilities (dtmf): us - 0x1 (telephone-event), peer - 0x1 (telephone-event), combined - 0x1 (telephone-event)

Peer audio RTP is at port 69.159.199.98:15454

<--- Transmitting (NAT) to 69.159.199.98:5060 --->

SIP/2.0 100 Trying

Via: SIP/2.0/UDP 69.159.199.98:5060;branch=z9hG4bK-12439f-3002;received=69.159.199.98

From: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

To: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

CSeq: 43246 INVITE

User-Agent: HMNet2.1

Allow: INVITE, ACK, CANCEL, OPTIONS, BYE, REFER, SUBSCRIBE, NOTIFY

Supported: replaces, timer

Contact: <sip:6136886212@209.217.98.130>

Content-Length: 0

<------------>

Audio is at 209.217.98.130 port 11840

Adding codec 0x4 (ulaw) to SDP

Adding non-codec 0x1 (telephone-event) to SDP

<--- Reliably Transmitting (NAT) to 69.159.199.98:5060 --->

SIP/2.0 200 OK

Via: SIP/2.0/UDP 69.159.199.98:5060;branch=z9hG4bK-12439f-3002;received=69.159.199.98

From: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

To: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

CSeq: 43246 INVITE

User-Agent: HMNet2.1

Allow: INVITE, ACK, CANCEL, OPTIONS, BYE, REFER, SUBSCRIBE, NOTIFY

Supported: replaces, timer

Contact: <sip:6136886212@209.217.98.130>

Content-Type: application/sdp

Content-Length: 253

v=0

o=root 177873221 177873223 IN IP4 209.217.98.130

s=HMNet2.1

c=IN IP4 209.217.98.130

t=0 0

m=audio 11840 RTP/AVP 0 101

a=rtpmap:0 PCMU/8000

a=rtpmap:101 telephone-event/8000

a=fmtp:101 0-16

a=silenceSupp:off - - - -

a=ptime:20

a=sendrecv

<------------>

sip01*CLI>

<--- SIP read from UDP://69.159.199.98:5060 --->

ACK sip:6136886212@209.217.98.130 SIP/2.0

Via: SIP/2.0/UDP 69.159.199.98:5060;branch=z9hG4bK-12439f-3007

To: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

From: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

CSeq: 43246 ACK

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

Date: Tue, 19 May 2009 05:29:03 GMT

Max-Forwards: 70

User-Agent: IST-OIS-7.0.2.1

Content-Length: 0

<------------->

--- (10 headers 0 lines) ---

-- Registered SIP '6477223566' at 67.153.38.230 port 5060

Scheduling destruction of SIP dialog '27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130' in 6400 ms (Method: ACK)

set_destination: Parsing <sip:2898407015@69.159.199.98:5060> for address/port to send to

set_destination: set destination to 69.159.199.98, port 5060

Reliably Transmitting (NAT) to 69.159.199.98:5060:

BYE sip:2898407015@69.159.199.98:5060 SIP/2.0

Via: SIP/2.0/UDP 209.217.98.130:5060;branch=z9hG4bK1ce987ef;rport

Max-Forwards: 70

From: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

To: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

CSeq: 103 BYE

User-Agent: HMNet2.1

X-Asterisk-HangupCause: Normal Clearing

X-Asterisk-HangupCauseCode: 16

Content-Length: 0

---

== Spawn extension (from-vss-gateway, 2898407015, 104) exited non-zero on 'SIP/64.26.157.244-b702f448'

sip01*CLI>

<--- SIP read from UDP://69.159.199.98:5060 --->

SIP/2.0 200 Ok

Via: SIP/2.0/UDP 209.217.98.130:5060;branch=z9hG4bK1ce987ef;received=209.217.98.130;rport=5060

To: <sip:2898407015@69.159.199.98:5060>;tag=205ef-4390-2fc2

From: "Unknown Name" <sip:6136886212@209.217.98.130>;tag=as5192ea61

CSeq: 103 BYE

Call-ID: 27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130

Date: Tue, 19 May 2009 05:29:12 GMT

Server: IST-OIS-7.0.2.1

Content-Length: 0

<------------->

--- (9 headers 0 lines) ---

SIP Response message for INCOMING dialog BYE arrived

Really destroying SIP dialog '27c0e4cb48e76a3c3f6c0d0662f4f96c@209.217.98.130' Method: ACK